Things to check if the ADAT XT won’t power up:

# Check the bridge rectifier for shorts

Test Q1 (the main switching transistor) for shorts.

ADAT XT PSU switching transistor   NK0905LF

Replacement            RS 438-584      RFP4N100  (Hitachi) £5.35

Check D1 is intact

Check D7 is intact (fast recovery diode - LT SR3002)

Available from CPC Ltd.   Part No. SCBY399

Check R2 is 330Ω

Check R3 (75KΩ) for open circuit

Check R9 is 22Ω

Check C7 is 100µF 25V

# If PWM chip (U1) is suspect replace with device RS 265-263

UC3842AN £0.89 each

If PSU does not appear to be resetting, disconnect the thyristor, Q4, (BT151 RS 284-898  £0.64)

Also, the optoisolator can give problems. Replace with RS 261-0283

TLP734 £0.46

If it still doesn’t work, your fucked, and will need to send it back to Graham Bryant at Aspen Media to fix, buy a completely new supply from him, or throw the complete ADAT (piece of junk) in the pond, and go down the pub!