## CANON

XJ100x, XJ86xII, XJ75x, XJ72x

SW2:No.6 ON→LDK Lens + SONY Camera (BTA Camera)

SW2:No.6 OFF→LDK Lens + LDK Camera

### LDK TO BVP

At the lens 36 pin connector, remove 2 screws

# Pin 22 remove blue wire & shrink tube end

Pin 16 remove grey wire & move to pin 22 position

Pin 30 remove yellow wire & move to pin 16 position

Replace connector and screws