Graphic machine network setup procedure

This guide is specific to Windows XP.

- Open My Computer.

- In tools menu open File Options.

- In the View tab scroll to the bottom and deselect Use Simple File Sharing if it is selected.

- Right click the drive you want to share on the network and choose Sharing and Security.

- Select Share this folder.

- Click New Share near the bottom and fill in a name for the drive to appear as on the network.

- Click Permissions and check all the boxes under Allow.

- Press OK and OK again on the New Share window.

- In the Sharing tab you should see “Windows Firewall is configured to allow this folder to be shared with other computers on the network”. If you do not see that then you can disable the Windows Firewall in the control panel or configure the firewall to allow shared network drives.

- Select the Security tab and in the window just below the tabs select Everyone. *** This step is what frequently screws this up.

- Check Full Control in the window below then press OK.

If the network drive does not show up immediately in My Network Places follow these steps to ensure the network is setup properly.

- Make sure any antivirus programs are disabled.

- Turn off Windows Firewall or any 3rd party firewall software.

- Check that the 2 computers can ping each other in command prompt.

- Make sure both computers are in the same workgroup. To check this right click my computer then properties and it will be under the computer name tab. Click Change to change the workgroup. The computer must restart after.

- Sometimes the computers will not be able to talk properly if they are on different switches. For example if you’re trying to network the Duet and a laptop make sure the laptop is plugged into the same switch that the Graphics Hub is plugged into. Typically it is in the Cisco switch.

- If the systems are not pinging each other try setting a static IP on both machines of 192.168.25.161 & 192.168.25.162. Subnet is 255.255.255.1 and DNS is 192.168.25