Subject:  BU Cameras:  Software Release Schedule

BU Cameras announces new versions of software introducing new functionality and features for LDK 4000, LDK 5000 and LDK 6000 HD camera systems.

This release includes the following new software releases: (click on the hyperlinks for more details)

HDTV range

LDK 6000 Camera head	Version 25

LDK 5000 Camera head	Version 08

LDK 4000 Camera head	Version 02

LDK 5460 HD Triax adapter for LDK 6000 and LDK 4000	Version 10

LDK 5440 Triaxadapter for LDK 5000	Version 04

LDK 4502 HD Basestation software for LDK 6000 and LDK 4000	Version 09

LDK 4505 Basestation software for LDK 5000 	Version 08

Summary of new features

Main reasons for these updates are:

- White clip level in SD outputs corrected.

- Intercom rooting VTR switch on lens and front camera selectable.

- Crash2Air functionality to be set.

Special feature for a customer implemented which is named “Crash2Air”

Therefore the software in camera head, Triax adapter AND base station need to be updated.

LDK 6000 Camera head software Version 25

The new software will be available in one version.

PCB version	PCB 12nc	SW 12nc	Filename

#### Databoard 	3922 406 86791	3922 407 33911	BT3391BK.-25

Installation to be done using script file: 392240733911_165_001_S25.TSC

LDK 5000 Camera head software Version 08

The new software will be available in one version.

PCB version	PCB 12nc	SW 12nc	Filename

Databoard 	3922 406 86791	3922 407 37801	BT3780BK.-08

Installation to be done using script file: 392240737801_165_001_S08.TSC

LDK 4000 Camera head software Version 02

The new software will be available in one version.

PCB version	PCB 12nc	SW 12nc	Filename

Databoard 	3922 406 86791	3922 407 44601	BT4460BK.-02

Installation to be done using script file: 392240744601_165_001_S02.TSC

New Functionality

- VTR switch on lens and front side camera selectable between intercom to Production or Engineering.
This can be selected in the “Install” menu “Intercom switch” to be set to ”Prod” or “ENG”

LDK 4502/01 Base Station software for LDK 6000 and LDK 4000 Version 09

The new software will be available in one version.

PCB version	PCB 12nc	SW 12nc	Filename

Databoard	3922 406 50011	3922 407 38041	BT3804BK.-09

Installation to be done using script file: 392240738041_165_001_S09.tsc

LDK 4505/01 Base Station software for LDK 5000 Version 08

The new software will be available in one version.

PCB version	PCB 12nc	SW 12nc	Filename

Databoard	3922 406 50011	3922 407 38051	BT3805BK.-08

Installation to be done using script file: 392240738051_165_001_S08.tsc

Problem solved

- Max white clip level in SD outputs can be set to 110%.

New functionality

- “Crash2Air”:
In the base station system menu, at service level, the item “Signaling output” is added.
Entering this menu will show the function “Crash2Air” that can be set to ON or OFF.
When Crash2Air is switched OFF the system is operating in normal condition.
When set to ON, the PROD button as well as the ENG button at the zoom bar of the lens are disabled.
Pushing the PROD switch at the zoom bar (LLA configuration) or pressing the CALL button at the back of the triax adapter, will activate pin 2 (Call out ext, relay contact <10 ohm) and pin 10 (Call out ext return) at the base station signaling connector as long as one of the switches is pushed in. 
Also the CALL indicator on the OCP will lit.
In stand alone configuration the RET switch on the zoom handle is assigned for that.

When Crash2Air is switched ON, and the LOC/EXT switch at the back of the triax adapter is set to EXT, the viewfinder will show the camera signal and a black and white text signal that is inserted to the EXT1 input of the base station. 
EXT1 therefore has to be a black and white text signal only !!!
With the LOC/EXT switch at the back of the triax adapter set to LOC, the viewfinder will show the camera signal only.
Also on LLA the EXT switches are disabled, as well as the EXT1 button on the zoom handle.
Pushing the EXT2 switch on the zoom handle will show the EXT2 signal.
The switch EXT1/2 on traix adapter in position EXT2 will show EXT2 
Note:
1. This is only available with the digital sync monitoring board 3922 406 51121 installed in combination with the power supply 3922 406 50221
2. On the sync monitoring board the firmware BT4118BK.110 need to be installed.

LDK 5460 HD Triax adapter software For LDK 6000 and LDK 4000 Version 10

The new software will be available in one version.

PCB version	PCB 12nc	SW 12nc	Filename

Databoard	3922 406 84341	3922 407 34351	BT3435BK.-10

Installation to be done using script file: 392240734351_165_001_S10.tsc

LDK 5440 Triax adapter software for LDK 5000 Version 04

The new software will be available in one version.

PCB version	PCB 12nc	SW 12nc	Filename

Databoard	3922 406 84341	3922 407 37811	BT3781BK.-04

Installation to be done using script file: 392240737811_165_001_S04.tsc

New Functionality

For complete functionality of the Crash2Air system also the triax adapter software must be updated.