Leader Instruments Corp.

LV5700A Software Upgrade Procedure

- Unzip the file (LV5700A Verx.x.zip). x.x is software version.

- Format the compact flash card as file system FAT. (Format NTFS or FAT32 may not work)

- Copy the “SYS” and “LDR” folders to the compact flash card.

- Turn OFF the LV5700A unit.

- Insert the compact flash card for upgrade into the compact flash card slot on the rear panel of the LV5700A unit.

- Turn on the LV5700A unit.

- The menu for upgrade is displayed.

- Push the [SYSTEM UPDATE] key.

- Upgrade of software is completed after several minutes.

- Turn off the LV5700A unit, and remove the CF card from the CF card slot.

- END