Released Versions of 3500Plus and Win3500Plus software:

|  | 3500Plus | V1.0 | V1.1 | V1.2 | V1.3 | V2.0 | V2.1 | #### V3.0 | V3.1 | V3.2 | V3.3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Win3500 ## Plus | # ECO |  | 3299 | 3411 | 3498 | 3636 | 3713 | Dev | CE00 331 | CE00 331 | CE00 867 |
| V1.0 | 3264 | √ | √ |  |  |  |  |  |  |  |  |
| V1.2 | 3478 |  |  | √ | √ |  |  |  |  |  |  |
| V2.0 | 3606 |  |  |  |  | √ | √ |  |  |  |  |
| V3.0 | Dev |  |  |  |  |  |  | √ |  |  |  |
| V3.1 | CE00331 |  |  |  |  |  |  |  | √ | √ |  |
| V3.3 | CE00867 |  |  |  |  |  |  |  | √ | √ | √ |

√ - Indicates those versions of 3500Plus and Win3500Plus that play together.  There may be some other versions that work together if certain items are not configured.   (For example, V2.0 of the 3500Plus works with V1.2 of the Win3500Plus as long as there are no XTN or TGR components involved.)  However, all bets are off if the offending configuration elements are defined.

Summary Of 3500Plus Changes:

V1.0 – Introduction of the product

V1.1 – Bug Fix of Loader program

V1.2 – Added 4 new panels (RCP-CSD, RCP-64X, RCP-128X, RCP-PVPG) as well as increased the number of categories to 254

V1.3 – Panel cleanup of the CSD, PVPG, and MP32 panels.

V2.0 – Added two new component types (XTN & TGR) and fixed a panel problem that could cause panels to reset during high activity.

V2.1 – Cleanup of Trucklink Problem

V3.0 – Massive change in naming schemes for sources, destinations, and reentries.  This was released only as a deviation (D00001) to CDC in Atlanta and Yahoo! Broadcast in Dallas.   Introduction of the Large Matrix Edition (LME).

V3.1 – Added remote clients for integration with e-Route

V3.2 – Improved the performance of the dual configuration transfer. Reduced interrupt latency for the serial port operation at 38400 baud.

V3.3 - Adds the CSD2 panel to the 3500Plus.  The XP Patch software is now included in the standard Win3500 and Win3500Plus distribution.

### Win3500Plus Configuration Upgrades

The preferred method of upgrading configurations saved to disk is to use the Win3500Plus’s File Import/Export mechanism.  This requires the user to load the configuration in the old version of Win3500Plus, export the configuration to a text file, import the text file in the new version of the Win3500Plus, and then save the configuration to disk.

When importing to Win3500Plus versions V3.0 or V3.1 from pre-V3.0 text files, a message appears to the user indicating that they still have to clean up their configuration.  This cleanup only applies to users who used category/index operation on their control panels.  Since the change to V3.0 so radically changed this operation, the user needs to reevaluate how they are going to structure their panels for the new category operation.

Field Upgrade Kits = 81906522680 or 81906523520 for large matrix editions.