Released Versions of 3500 and Win3500 software:

|  | 3500 | V2.0 | V3.0 | V3.1 | V3.2 | V3.3 | V3.5 | #### V4.0 | V4.1 | V4.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Win3500 | # ECO | 2892 | 3070 | 3125 | 3134 | 3164 | 3496 | CE00596 | CE00807 | CE00867 |
| V1.0 | ? |  |  |  |  |  |  |  |  |  |
| V2.0 | 2892 |  |  |  |  |  |  |  |  |  |
| V3.0 | 3070 |  | √ | √ | √ |  |  |  |  |  |
| V3.3 | 3253 |  |  |  |  | √ |  |  |  |  |
| V3.4 | 3410 |  |  |  |  | √ |  |  |  |  |
| V3.5 | 3496 |  |  |  |  |  | √ |  |  |  |
| ##### V4.0 | CE00596 |  |  |  |  |  |  | √ |  |  |
| V4.1 | CE00807 |  |  |  |  |  |  | √ | √ |  |
| V4.2 | CE00867 |  |  |  |  |  |  | √ | √ | √ |

√ - Indicates those versions of 3500 and Win3500 that play together.

Summary Of 3500 Changes:

V2.0 – Introduction of tielines

V3.0 – Introduction of dynamic panel configuration, overlaid components, and truck link. Increased the number of panels to 508.

V3.1 – Corrects a problem of logging panels onto the system when a small number of panels are configured.

V3.2 – Fixes a bug in which salvo information was not transferred across the dual communications link. This bug was present in V3.0 and V3.1

V3.3 – Fixes a problem with the controller locking up when the switcher is turned off or when block checking is turned off. This only impacts the diagnostic mode of operation.

V3.5 – This retrofits the 3500 to include the RCP-64X and RCP-128X panels. It also adds the RCP-MP32 change needed for Burst Video’s Joystick Override option.

Versions 3.5 or earlier used through hole PCB.  All versions after 4.0 use a ‘Surface Mount’ and is in fact the same PCB as the 3500 Plus.

V4.0  Changed the 3500 PCB to be the same as the 3500 Plus PCB. Changed the item descriptions  to add he term OLD. Note: No manual update in this release. We are using 3500 Plus manuals for this product. Changed the color of the shield from gray to black.

V4.1 – Introduction to include the Motorola specified RCP-MOT that has a special lock key functionality. This version also contains code to cause the panels going online/offline to be processed more quickly. This prevents a panel from being refreshed after a power cycle.

V4.2 - Adds the CSD2 panel to the 3500Plus. The XP Patch software is now included in the                                  standard Win3500 and Win3500Plus distribution.

Field Upgrade Kits = 81906523920