Some basic protocol command sequences when using PESA Standard Protocol No. 1 (P1)

Example 1: To make a basic input to output switch on a 2 level system, you could perform the following:

Switching Output # 1 to Input #5 for Level 1(typically video) only:

Change Switcher = H

Output/Destination #1 = 001

Input/Source = 005

Out of Range Source =  255

Check Sum = 0:

Then you must enter a Carriage Return = Enter

Then you must enter a Line Feed, which = Control J if using HyperTerminal

Example 2: Switching a 2 level system to switch Output # 1 to input 5 for both levels (typically in the 3500 controller, Video is associated to Level 1 and Audio is associated to Level 2)

Change Switcher = H

Output/Destination #1 = 001

Input/Source for Level 1 = 005

Input/Source for Level 2 = 005

Check Sum = 03

Then you must enter a Carriage Return = Enter

Then you must enter a Line Feed, which = Control J if using HyperTerminal

Example 3: Switching a 2 level system to switch Output # 1 to input 5 for level 1 and input 10 for level 2 (typically in the 3500 controller, Video is associated to Level 1 and Audio is associated to Level 2)

Change Switcher = H

Output/Destination #1 = 001

Input/Source for Level 1 = 005

Input/Source for Level 2 = 010

Check Sum = ??

Then you must enter a Carriage Return = Enter

Then you must enter a Line Feed, which = Control J if using HyperTerminal