# Premiere Basic Command Set

Menu : cmd [parameters] where cmd is one of the following

CM      Clear macros

CS      Read configuration switches

DN      Destination names

EM      Emulation mode

ET      Ethernet parameters

FD      Reset to factory default

FF      Force flash write of macro data

FW      Force flash write

IG      Input audio gain

LN      Level names

MA      Macro [LEV] [NUM]

ML      Master level configuration

MN      Macro Name

MT      Mute

OG      Output audio gain

PA      Panel information

PD      Predefined configuration

PL      Physical level configuration

PP      Panel Password

PR      PRC base level

RB      Reboot

RN      Source names

SD      Sync delay

SM      Sync mask configuration

SN      Serial number

SP      SNMP parameters

ST      Slot information

SW      Switch

VE      Version of controller software

HE plus any command shows help for that command

# CM      Clear macros

/>cm

/>he cm

Clears the macros.

# CS      Read configuration switches

/>cs

Config switch is set to RUN.

Bottom switch is ignored.

/>he cs

Displays the status of the hardware configuration switches.

Currently, the lower switch is ignored.  This parameter is read only.

# DN      Destination names

/>dn

Level 1 Dst 1 - DST 1.1

Level 1 Dst 2 - DST 1.2

Level 1 Dst 3 - DST 1.3

Level 1 Dst 4 - DST 1.4

Level 1 Dst 5 - DST 1.5

Level 1 Dst 6 - DST 1.6

Level 1 Dst 7 - DST 1.7

Level 1 Dst 8 - DST 1.8

Level 2 Dst 1 - DST 2.1

Level 2 Dst 2 - DST 2.2

Level 2 Dst 3 - DST 2.3

Level 2 Dst 4 - DST 2.4

Level 2 Dst 5 - DST 2.5

Level 2 Dst 6 - DST 2.6

Level 2 Dst 7 - DST 2.7

Level 2 Dst 8 - DST 2.8

Level 3 Dst 1 - DST 3.1

Level 3 Dst 2 - DST 3.2

Level 3 Dst 3 - DST 3.3

Level 3 Dst 4 - DST 3.4

Level 3 Dst 5 - DST 3.5

Level 3 Dst 6 - DST 3.6

Level 3 Dst 7 - DST 3.7

Level 3 Dst 8 - DST 3.8

/>he dn

Syntax: DN < LEV [int]  DST [int] [str] >

Where :  LEV [int] is the level to get the name change.

:  DST [int] is the output to get the name change.

:     where int is an integer between 1 and 6 levels and.

:     between 1 and number of destinations.

:     where str is the output name string.

/>dn lev 1 dst 1 test

/>dn

Level 1 Dst 1 - test

Level 1 Dst 2 - DST 1.2

Level 1 Dst 3 - DST 1.3

Level 1 Dst 4 - DST 1.4

Level 1 Dst 5 - DST 1.5

Level 1 Dst 6 - DST 1.6

Level 1 Dst 7 - DST 1.7

Level 1 Dst 8 - DST 1.8

Level 2 Dst 1 - DST 2.1

Level 2 Dst 2 - DST 2.2

Level 2 Dst 3 - DST 2.3

Level 2 Dst 4 - DST 2.4

Level 2 Dst 5 - DST 2.5

Level 2 Dst 6 - DST 2.6

Level 2 Dst 7 - DST 2.7

Level 2 Dst 8 - DST 2.8

Level 3 Dst 1 - DST 3.1

Level 3 Dst 2 - DST 3.2

Level 3 Dst 3 - DST 3.3

Level 3 Dst 4 - DST 3.4

Level 3 Dst 5 - DST 3.5

Level 3 Dst 6 - DST 3.6

Level 3 Dst 7 - DST 3.7

Level 3 Dst 8 - DST 3.8

# EM      Emulation mode

/>em

Emulation Mode: PESA

/>he em

Syntax: EM < PESA | MODE1 | MODE2 | CPULINK >

Where :  PESA uses the PESA emulation mode.

:  MODE1 uses emulation mode 1.

:  MODE2 uses emulation mode 2.

:  CPULINK uses the PESA CPU Link protocol.

/>em mode1

/>em

Emulation Mode: Mode 1

# ET      Ethernet parameters

/>et

MAC Address:         02:02:02:02:45:87

Ethernet Address:    192.168.1.17

Subnet Mask Address: 255.255.252.0

Gateway Address:     192.168.0.1

NNTP Address:        192.5.41.209

DHCP Enable:         ON

HTTP Enable:         OFF

Telnet Enable:       ON

Telnet Port:         23

Telnet Password:     pesa

/>he et

Syntax: ET < DHCP [ON|OFF]  | IP [addr]     | GW [addr]     |

:      SN [addr]      | TN [ON|OFF]   | TP [tport]    |

:      TPASS [str]    | NNTP [addr]   | HTTP [ON|OFF] >

Where :  DHCP ON enables client automatic IP address negotiation.

:  IP [addr] allows you to set the IP address of the PMFC.

:  GW [addr] allows you to set the gateway address.

:  SN [addr] allows you to set the subnet mask.

:  TN ON enables the on-board telnet server.

:  TP [port] allows you to set the telnet port.

:  TPASS [str] sets the telnet password.

:  NNTP [addr] allows you to set the NNTP address.

:  HTTP ON enables HTTP.

:     where addr is in dotted notation (ex. 192.168.1.1}.

:     where tport is in the range [1...32767].

/>et http on

/>et

MAC Address:         02:02:02:02:45:87

Ethernet Address:    192.168.1.17

Subnet Mask Address: 255.255.252.0

Gateway Address:     192.168.0.1

NNTP Address:        192.5.41.209

DHCP Enable:         ON

HTTP Enable:         ON

Telnet Enable:       ON

Telnet Port:         23

Telnet Password:     pesa

# FD      Reset to factory default

/>fd

/>he fd

Resets the unit to factory default.

# FF      Force flash write of macro data

/>ff

/>he ff

Forces preset data and level, source and destination names to be written

to flash.

# FW      Force flash write

/>fw

/>he fw

Forces data to be written to flash.

# IG      Input audio gain

/>ig

Input 1: -26.0

Input 2: 0.0

Input 3: 0.0

Input 4: 0.0

Input 5: 0.0

Input 6: 0.0

Input 7: 0.0

Input 8: 0.0

Input 9: 0.0

Input 10: 0.0

Input 11: 0.0

Input 12: 0.0

Input 13: 0.0

Input 14: 0.0

Input 15: 3.5

Input 16: 0.0

/>he ig

Syntax: IG < [NUM] + | - >

Where :  [NUM] is the input to change.

:  + | - means to increment or decrement.

Example: IG 1 +.

/>ig 3 +

/>ig

Input 1: -26.0

Input 2: 0.0

Input 3: 0.5

Input 4: 0.0

Input 5: 0.0

Input 6: 0.0

Input 7: 0.0

Input 8: 0.0

Input 9: 0.0

Input 10: 0.0

Input 11: 0.0

Input 12: 0.0

Input 13: 0.0

Input 14: 0.0

Input 15: 3.5

Input 16: 0.0

# LN      Level names

/>ln

Level 1 - All

Level 2 - Video

Level 3 - Audio

Level 4 - Level 4

Level 5 - Level 5

Level 6 - Level 6

/>he ln

Syntax: LN < LEV [int]  [str] >

Where :  LEV [int] is the level to get the name change.

:     where int is an integer between 1 and 6.

:     where str is the level name string.

/>ln lev 4 test

/>ln

Level 1 - All

Level 2 - Video

Level 3 - Audio

Level 4 - test

Level 5 - Level 5

Level 6 - Level 6

# MA      Macro [LEV] [NUM]

/>ma 1 1

Macro on level 1 number 1 (Macro 1) is INACTIVE

Out   1 - In   0

Out   2 - In   0

Out   3 - In   0

Out   4 - In   0

Out   5 - In   0

Out   6 - In   0

Out   7 - In   0

Out   8 - In   0

Out   9 - In   0

Out  10 - In   0

Out  11 - In   0

Out  12 - In   0

Out  13 - In   0

Out  14 - In   0

Out  15 - In   0

Out  16 - In   0

/>he ma

Syntax: MA [lev] [num] < [Action] | OUT [int]  IN [int] >

Where :  where [lev] is the level for the macro.

:  where [num] is the number of the macro.

:  [Action] is the action to take.

:           ACT means make the macro active

:           INA means make the macro inactive

:           TAKE means take the current macro

:  OUT [int] is the output for the macro.

:  IN [int] is the input for the macro.

:     where int is an integer between 1 and 16 for

:     inputs and 1 and 16 for outputs.

/>ma 1 1 out 2 in 2

/>ma 1 1 act

/>ma 1 1

Macro on level 1 number 1 (Macro 1) is ACTIVE

Out   1 - In   0

Out   2 - In   2

Out   3 - In   0

Out   4 - In   0

Out   5 - In   0

Out   6 - In   0

Out   7 - In   0

Out   8 - In   0

Out   9 - In   0

Out  10 - In   0

Out  11 - In   0

Out  12 - In   0

Out  13 - In   0

Out  14 - In   0

Out  15 - In   0

Out  16 - In   0

# ML      Master level configuration

/>ml

Level 1: 1  2

Level 2: 1

Level 3: 2

Level 4:

Level 5:

Level 6:

/>he ml

Displays and changes the control levels.

Syntax: ML < LEV [int]  ADD/DEL [int] >

Where :  LEV [int] is the level to change.

:  ADD/DEL [int] is the slot to add or delete.

:     where int is an integer between 1 and number of levels.

Example: ML LEV 1 ADD 1 2.

Example: ML LEV 2 DEL 4 5.

NOTE: Only add or delete to one level at a time.

/>ml lev 3 add 1

/>ml

Level 1: 1  2

Level 2: 1

Level 3: 1  2

Level 4:

Level 5:

Level 6:

# MN      Macro Name

/>mn 1 1

Macro on level 1 number 1 (Macro 1)

/>he mn

Syntax: MN < LEV [int]  NUM [int] name >

Where :  LEV [int] is the level for the macro.

:  NUM [int] is the number of the macro.

/>mn 1 1 test

/>mn 1 1

Macro on level 1 number 1 (test)

# MT      Mute

/>mt

Mute: OFF

/>he mt

Syntax: MT < ON | OFF >

Where :  Turns mute on and off.

/>mt on

/>mt

Mute: ON

# OG      Output audio gain

/>og

Output 1: -7.0

Output 2: 7.5

Output 3: 0.0

Output 4: 0.0

Output 5: 0.0

Output 6: 5.0

Output 7: 0.0

Output 8: 0.5

Output 9: 0.0

Output 10: 0.0

Output 11: 0.0

Output 12: 0.0

Output 13: 0.0

Output 14: 0.0

Output 15: 0.0

Output 16: 0.0

/>he og

Syntax: OG < [NUM] + | - >

Where :  [NUM] is the output to change.

:  + | - means to increment or decrement.

Example: OG 1 +.

/>og 1 +

/>og

Output 1: 0.5

Output 2: 0.0

Output 3: 0.0

Output 4: 0.0

Output 5: 0.0

Output 6: 0.0

Output 7: 0.0

Output 8: 0.0

Output 9: 0.0

Output 10: 0.0

Output 11: 0.0

Output 12: 0.0

Output 13: 0.0

Output 14: 0.0

Output 15: 0.0

Output 16: 0.0

# PA      Panel information

/>pa

Number Of Panels: 1

Button 1: Level(s): 1  2

Button 2: Level(s): 1

Button 3: Level(s): 2

Button 4: No levels assigned to this button.

Button 5: No levels assigned to this button.

Button 6: No levels assigned to this button.

/>he pa

Displays the panel buttons mappings.

# PD      Predefined configuration

/>pd

Predefined Configuration: Composite Video with Stereo Audio

/>he pd

Displays the predefined configuration.  This parameter is read only.

# PL      Physical level configuration

/>pl

Level 1: 1

Level 2: 2

Level 3:

Level 4:

Level 5:

Level 6:

/>he pl

Displays and changes the slots a level controls.

Syntax: PL < LEV [int]  ADD/DEL [int] >

Where :  LEV [int] is the level to change.

:  ADD/DEL [int] is the slot to add or delete.

:     where int is an integer between 1 and number of levels and

:     between 1 and number of slots, respectively

Example: PL LEV 1 ADD 1 2.

Example: PL LEV 2 DEL 4 5.

NOTE: Only add or delete to one level at a time.

/>pl lev 1 add 2

/>pl

Level 1: 1  2

Level 2: 2

Level 3:

Level 4:

Level 5:

Level 6:

# PP      Panel Password

/>pp

Panel Password : 12345

/>he pp

Syntax: PP < password >

Where :  where password consists of 5 digits in the range 1..6

:        which correspond to the panel level buttons.

/>pp 23456

/>pp

Panel Password : 23456

# PR      PRC base level

/>pr

PRC Base Level: 0

>he pr

Syntax: PR <int>

Where :  where int is the PRC base level in the range 1..62

/>pr 3

/>pr

PRC Base Level: 3

# RB      Reboot

/>rb

/>he rb

Reboots the unit.

# RN      Source names

/>rn

Level 1 Src 1 - SRC 1.1

Level 1 Src 2 - SRC 1.2

Level 1 Src 3 - SRC 1.3

Level 1 Src 4 - SRC 1.4

Level 1 Src 5 - SRC 1.5

Level 1 Src 6 - SRC 1.6

Level 1 Src 7 - SRC 1.7

Level 1 Src 8 - SRC 1.8

Level 1 Src 9 - SRC 1.9

Level 1 Src 10 - SRC 1.10

Level 1 Src 11 - SRC 1.11

Level 1 Src 12 - SRC 1.12

Level 1 Src 13 - SRC 1.13

Level 1 Src 14 - SRC 1.14

Level 1 Src 15 - SRC 1.15

Level 1 Src 16 - SRC 1.16

Level 2 Src 1 – SRC 2.1

Level 2 Src 2 - SRC 2.2

Level 2 Src 3 - SRC 2.3

Level 2 Src 4 - SRC 2.4

Level 2 Src 5 - SRC 2.5

Level 2 Src 6 - SRC 2.6

Level 2 Src 7 - SRC 2.7

Level 2 Src 8 - SRC 2.8

Level 2 Src 9 - SRC 2.9

Level 2 Scr 10 – SRC 2.10

Level 2 Src 11 - SRC 2.11

Level 2 Src 12 - SRC 2.12

Level 2 Src 13 - SRC 2.13

Level 2 Src 14 - SRC 2.14

Level 2 Src 15 - SRC 2.15

Level 2 Src 16 - SRC 2.16

Level 3 Src 1 - SRC 3.1

Level 3 Src 2 - SRC 3.2

Level 3 Src 3 - SRC 3.3

Level 3 Src 4 - SRC 3.4

Level 3 Src 5 - SRC 3.5

Level 3 Src 6 - SRC 3.6

Level 3 Src 7 - SRC 3.7

Level 3 Src 8 - SRC 3.8

Level 3 Src 9 - SRC 3.9

Level 3 Src 10 - SRC 3.10

Level 3 Src 11 - SRC 3.11

Level 3 Src 12 - SRC 3.12

Level 3 Src 13 - SRC 3.13

Level 3 Src 14 - SRC 3.14

Level 3 Src 15 - SRC 3.15

Level 3 Src 16 - SRC 3.16

/>he rn

Syntax: RN < LEV [int]  SRC [int] [str] >

Where :  LEV [int] is the level to get the name change.

:  SRC [int] is the input to get the name change.

:     where int is an integer between 1 and 6 levels and.

:     between 1 and number of sources.

:     where str is the input name string.

/>rn lev 1 src 1 test

/>rn

Level 1 Src 1 - test

Level 1 Src 2 - SRC 1.2

Level 1 Src 3 - SRC 1.3

Level 1 Src 4 - SRC 1.4

Level 1 Src 5 - SRC 1.5

Level 1 Src 6 - SRC 1.6

Level 1 Src 7 - SRC 1.7

Level 1 Src 8 - SRC 1.8

Level 1 Src 9 - SRC 1.9

Level 1 Src 10 - SRC 1.10

Level 1 Src 11 - SRC 1.11

Level 1 Src 12 - SRC 1.12

Level 1 Src 13 - SRC 1.13

Level 1 Src 14 - SRC 1.14

Level 1 Src 15 - SRC 1.15

Level 1 Src 16 - SRC 1.16

Level 2 Src 1 - SRC 2.1

Level 2 Src 2 - SRC 2.2

Level 2 Src 3 - SRC 2.3

Level 2 Src 4 - SRC 2.4

Level 2 Src 5 - SRC 2.5

Level 2 Src 6 - SRC 2.6

Level 2 Src 7 - SRC 2.7

Level 2 Src 8 - SRC 2.8

Level 2 Src 9 - SRC 2.9

Level 2 Scr 10 – SRC 2.10

Level 2 Src 11 - SRC 2.11

Level 2 Src 12 - SRC 2.12

Level 2 Src 13 - SRC 2.13

Level 2 Src 14 - SRC 2.14

Level 2 Src 15 - SRC 2.15

Level 2 Src 16 - SRC 2.16

Level 3 Src 1 - SRC 3.1

Level 3 Src 2 - SRC 3.2

Level 3 Src 3 - SRC 3.3

Level 3 Src 4 - SRC 3.4

Level 3 Src 5 - SRC 3.5

Level 3 Src 6 - SRC 3.6

Level 3 Src 7 - SRC 3.7

Level 3 Src 8 - SRC 3.8

Level 3 Src 9 - SRC 3.9

Level 3 Src 10 - SRC 3.10

Level 3 Src 11 - SRC 3.11

Level 3 Src 12 - SRC 3.12

Level 3 Src 13 - SRC 3.13

Level 3 Src 14 - SRC 3.14

Level 3 Src 15 - SRC 3.15

Level 3 Src 16 - SRC 3.16

# SD      Sync delay

/>sd

Sync Delay    : 16

/>he sd

Syntax: SD < delay >

Where :     where delay is the count of 0.125 second delay

:     intervals and is in the range 0..127

/>sd 18

/>sd

Sync Delay    : 18

# SM      Sync mask configuration

/>sm

Sync Mask    : 0000 0000 0000 0000 0000 0000

/>he sm

Syntax: SM < [lev] [mask] >

Where :     where lev  is the level.

:     where mask is a hexadecimal mask for the level.

/>sm 1 3

/>sm

Sync Mask    : 0003 0000 0000 0000 0000 0000

# SN      Serial number

/>sn

Serial Number: 000000000000123

/>he sn

Displays the unit serial number.  This parameter is read only.

# SP      SNMP parameters

/>sp

SNMP Version:     2

Set Community:    private

Get Community:    public

System Location:  Location

System Contact:   info@pesa.com

Md5 Password:     pesa1234

DES Password:     pesa1234

Trap Enable:      OFF

Auth Trap Enable: OFF

Trapsink:         0.0.0.0

Trap Community:   public

/>he sp

Syntax: SP < GCOMM [str]    | SCOMM [str]   | TCOMM [str]   |

:      SLOCA [str]    | SCONT [str]   | TRAP [ON|OFF] |

:      AUTH [ON|OFF]  | TSINK [addr]  | VE [ver]   >

Where :  GCOMM [str] allows you to set the SNMP get community string.

:  SCOMM [str] allows you to set the SNMP set community string.

:  TCOMM [str] allows you to set the SNMP trap community string.

:  SLOCA [str] allows you to set the SNMP system location.

:  SCONT [str] allows you to set the SNMP system contact.

:  TRAP ON enables SNMP traps.

:  AUTH ON enables authentication traps.

:  TSINK [addr] allows you to set the trap target IP address.

:  VE [ver] allows you to set the SNMP version.

:     where ver is '1' or '2'.

:     where addr is in dotted notation (ex. 192.168.1.1}.

/>sp sloca office

/>sp

SNMP Version:     2

Set Community:    private

Get Community:    public

System Location:  office

System Contact:   info@pesa.com

Md5 Password:     pesa1234

DES Password:     pesa1234

Trap Enable:      OFF

Auth Trap Enable: OFF

Trapsink:         0.0.0.0

Trap Community:   public

# ST      Slot information

/>st

Slot Number:       1

Card Type:         Analog Video Primary

Status:            OK

Software Revision: 1

Number of Inputs:  16

Number of Outputs: 8

Serial Number:     SERIAL NUMBER

Slot Number:       2

Card Type:         Analog Audio Primary

Status:            OK

Software Revision: 1

Number of Inputs:  16

Number of Outputs: 8

Serial Number:     SERIAL NUMBER

/>he st

Displays slot information.  These parameters are read only.

# SW      Switch

/>sw

Level: 1        Output: 1       Input: 0

Level: 1        Output: 2       Input: 6

Level: 1        Output: 3       Input: 0

Level: 1        Output: 4       Input: 0

Level: 1        Output: 5       Input: 0

Level: 1        Output: 6       Input: 0

Level: 1        Output: 7       Input: 0

Level: 1        Output: 8       Input: 0

Level: 2        Output: 1       Input: 0

Level: 2        Output: 2       Input: 6

Level: 2        Output: 3       Input: 0

Level: 2        Output: 4       Input: 0

Level: 2        Output: 5       Input: 0

Level: 2        Output: 6       Input: 0

Level: 2        Output: 7       Input: 0

Level: 2        Output: 8       Input: 0

Level: 3        Output: 1       Input: 0

Level: 3        Output: 2       Input: 6

Level: 3        Output: 3       Input: 0

Level: 3        Output: 4       Input: 0

Level: 3        Output: 5       Input: 0

Level: 3        Output: 6       Input: 0

Level: 3        Output: 7       Input: 0

Level: 3        Output: 8       Input: 0

/>he sw

Displays and takes a switch.

Syntax: SW < LEV [int]  OUT [int]  IN [int] >

Where :  LEV [int] is the level to take the switch.

:  OUT [int] is the output for the switch.

:  IN [int] is the input for the switch.

:     where int is an integer between 1 and 16 for

:     inputs and 1 and 8 for outputs.

EXAMPLE: SW LEV 1 OUT 3 IN 2.

NOTE: Only take one switch per level at a time.

/>sw lev 1 out 2 in 4

/>sw

Level: 1        Output: 1       Input: 6

Level: 1        Output: 2       Input: 4

Level: 1        Output: 3       Input: 0

Level: 1        Output: 4       Input: 0

Level: 1        Output: 5       Input: 0

Level: 1        Output: 6       Input: 0

Level: 1        Output: 7       Input: 0

Level: 1        Output: 8       Input: 0

Level: 2        Output: 1       Input: 6

Level: 2        Output: 2       Input: 4

Level: 2        Output: 3       Input: 0

Level: 2        Output: 4       Input: 0

Level: 2        Output: 5       Input: 0

Level: 2        Output: 6       Input: 0

Level: 2        Output: 7       Input: 0

Level: 2        Output: 8       Input: 0

Level: 3        Output: 1       Input: 6

Level: 3        Output: 2       Input: 4

Level: 3        Output: 3       Input: 0

Level: 3        Output: 4       Input: 0

Level: 3        Output: 5       Input: 0

Level: 3        Output: 6       Input: 0

Level: 3        Output: 7       Input: 0

Level: 3        Output: 8       Input: 0

# VE      Version of controller software

/>ve

Version: 1.2

/>he ve

Displays the version of software in the controller.  This parameter is r

ead only.