ADAM PS.DOC

+5v

+V1

CW to increase

R128

+2.1v

+V4

CCW to increase

R31

-15v

-V3

CCW to increase

R79

+15v

+V2

CW to increase

R78

TOP VIEW

FRONT

ADAM  Power Supply

Voltage Adjustment Locations

Plug in both power supplies into frame.

Turn on only one at a time and adjust 2.1 volt rail.

With both supplies on, the 2.1 volt rail should read around 2.4 volts.

FUSES

110 volt frame 10A   ABC 250V   Telex part # 50547115

220 volt frame 6A     ABC 250V   Telex part # 50547112

Power Supply  10A  ABC 250V   Telex part # 50547115