To have an MCII-e card ping another MCII-e card (or any other device)

1. Run AZedit, and go on-line

2. Type Ctrl-Alt-M to switch to the Debug Terminal view

- If this doesn’t work, you have to enable it in the registry – see below

3. Type

Debug

followed by the Enter key, to enable the debugger. This will not be echoed; however, once you hit Enter, it should respond with

Debugger enabled

If not, try again. If it echoes

Deb

as you type, the debugger is already enabled. Just hit Backspace to erase those characters. (It is accepting them as hex digits for typing in an address.)

4. Type

(get

followed by a Tab. This should expand to

(get_dbg_msgs)

Hit Enter. This should give you a screenful or so of debug messages. Keep doing this until the only response is

==> 0

If you’re not sure, do it again.

5. Type

(Ping,”10.2.206.1”)

(or whatever the IP address is.) This may result in a response such as

Ping: 10.2.206.1: time=0.002 sec

==> 0x0000004d

which is self-explanatory. (Ignore the ==> and the number following it.)

If you don’t get a response, then wait a second and type

(get_dbg_msgs)

as above. Keep doing this, for up to 5 seconds; eventually (get_dbg_msgs) will include either a Ping response as above or a response such as

Ping: 10.2.206.1: No reply

6. Disable the debugger: Type

=az_debug_enabled

followed by the Enter key. This should respond with a line such as

4177394: 01 00 00 00 00 00 00 00  00 00 00 00 00 00 00 00

Type

.=0

followed by Enter (when you type the “.”, the debugger will echo the 7-character address printed out in the previous response).

If you need access to the debugger again, restart this procedure (with step 1, 2, or 3 as appropriate.)

To enable access to the Debug Terminal (step 2, above)

1. Exit AZedit

2. From the Windows Start menu, select Run… and enter the command

regedit

3. Browse on the left to the key

HKEY_CURRENT_USER\Software\Telex\AZeditV3\Intercoms\<icom>\Users\<user>\advanced

where <icom> and <user> represent the intercom and user name with which AZedit connects (either or both of these may be “Default”).

4. Double-click the registry key

AllowDebugTerminal

on the right side, change the value from N to Y, and click OK

5. Exit regedit

6. Restart AZedit.