Menus, SERVICE, FOOT SW (with GPI module ONLY)

NOTE: The foot switch must be connect at J6 (Opto-4 & Opto-3) on the back of the KP-12. It also must be cabled across pins

5 & 9 of the Opto-4 & 3 DB-9 connector.

When the Foot switch is enabled, activated talk keys enter standby mode and the talk LED turns orange. Once you press the

foot switch, the crosspoint is closed and the Talk LED turns green for normal operation.

1. To change the key type, select FOOT SW.

2. Then, turn the SELECT control to choose either ON or OFF, and tap SELECT control.

3. Double-tap the SELECT control to return to the previous menu level. Or, press and hold the SELECT control for two

seconds to exit the menu mode.

Remember to save your changes if you want them retained for future use. See “Menus, SERVICE, SAVE CFG” on page 44..

GROUP 1-4 Assigns several keys for simultaneous activation when a group master key is activated

FOOT SW Enables or Disables the use of the foot switch. (Only available if the GPI board is installed)