Key Fcn.DOCKeypanel Programming Guide6-27-01

DISPLAY  REQUESTS

0-8-0	Test Talk keys and displays (press CLR to exit)

0-8-1	Display frame port number (does not read panel dip switches)

0-8-2	Display level 2 Talk key assignment

0-8-3	Display Listen key assignments

0-8-7	Turn on tone generator (press CLR to turn it off)

0-8-8	Display setup page assignments

0-8-	Display request scroll up / down. press PGM key after making

selection to view  information.  (gets into scroll mode)

Select NAME = name of person talking to this keypanel  button.

Select GAIN = adjusts xpt levels on this panel button.

# SETUP PAGE ASSIGNMENT

8-n-PGM-Talk / Listen	Assign setup page “n” to a keypanel or expansion

panel

# SCROLLING

0- or 0-	Intercom scroll

0-5-	Type scroll (P-P, PL, etc.) (press PGM to view Type selected)

5-	Prefix scroll (press PGM to view Type selected)

	Single-step scroll

0-8 or 0-9	Use this to get to the matrix PGM section, then scroll to the

remote farme you want, then press PGM to scroll to the port

you want.

# TALK AND LISTEN KEY ASSIGNMENT USING NUMERIC ENTRY

Notes 	icm# = intercom system number, enter only for remote key assignment.  n = number of panel, party line etc.; leading zeros may be required for remote assignment.

1-icm#-n-PGM-Talk / Listen	Point-to-point key assignment

2-icm#-n-PGM-Talk / Listen	Party line key assignment

3-1-PGM-Listen	Auto listen (Listen keys only)

3-2-PGM-Listen	Auto follow (Listen keys only)

3-3-PGM-Listen	Auto mute (Listen keys only)

3-4-PGM-Listen	Auto reciprocal (Listen keys only)

3-5-PGM-Talk	All call (Talk keys only)

0-1-icm#-n-PGM-Talk / Listen	Special list key assignment

0-2-icm#-n-PGM-Talk / Listen	IFB key assignment

0-3-icm#-n-PGM-Talk / Listen	Camera ISO key assignment

0-4-icm#-n-PGM-Talk / Listen	Relay key assignment

### COPYING

7-Talk / Listen	Copy Incoming Messages display to a key

0-Talk / Listen - Talk / Listen	Copy one key’s assignment to another key

NUM

1

SLIST

PL

2

IFB

PHONE

4

RELAY

COPY CW

7

EX COPY

CLR

PREFIX

5

TYPE

E-PNL

8

DISPLAY

MULT

0

FUNC

AUTO

3

ISO



6





9



PGM

Note: Press the CLR key if you make a mistake while programming.  Then start over.

KEYPAD

LAYOUT

Keypad Programming Sequences -- Update

1. [MULT] does not work for programming talk level 2.  Instead, prefix the entire sequence with [FUNC] [MULT] (0-0).  E.g. To program Party Line 23 on talk level 2 of a key, enter 0-0-2-2-3-PGM-<key> (Func-Mult, PL-2-3).

2. Leading zeroes can be dropped from function numbers.  For example, Point-to-Point 7 can be programmed as:

0-7-PGM-<key> (NUM-7-PGM-<key>), 0-0-7-PGM-<key>, or

0-0-0-7-PGM-<key>.

3. A remote assignment can be made by prefixing the function number by the matrix number (1 or 2 digits).  In this case, leading 0's cannot be dropped.  For example, to program a key with Point-to-Point 14 for matrix 3, use NUM-3-0-1-4-PGM-<key>.  Note that the local matrix is always referred to as matrix 0.

4. Dial out on a TIF, press the listen key, enter PHONE (4), then PGM, then latch the desired talk key up.  The display will clear.  Enter digits on the keypad; they will scroll through the display of the key being dialed, and sent to the TIF.  Briefly turn the talk key off to exit dial mode; the display will revert to normal.  If the panel is a KP32 and a key is named a TIF in ADAMedit, you must use the new ky equence found in the P32 manual.

5. Hang up a TIF, enter PHONE-CLR-<key> or PHONE-0-PGM-<key>.  This is one of the very few times that the Clear key does something other than clear any entered programming sequences.  However, two Clear keys in succession (PHONE-CLR-CLR) will really clear the sequence (if the keypad is not in TIF dial mode).  If the panel is a KP32 and a key is named a TIF, you must use the new KP32 keypanel sequence found in the KP32 manual.

6. To scroll through different matrices, enter FUNC-UP or FUNC-DOWN.  When the desired matrix name is found, either hit PGM (to get the scroll list for that matrix) or enter FUNC-TYPE-UP/DOWN to get into type scrolling mode for the remote matrix.  For example, enter FUNC-DOWN, followed by DOWN a few times to get the desired matrix; then enter FUNC-TYPE-DOWN, followed by DOWN, until "PL" is displayed; then hit PGM to get the party line scroll list for the desired matrix (if available).

7. Matrix names are embedded in the local scroll list.  When such a name is displayed, the first character alternates between the real alpha and a '+'.  Hitting PGM when a matrix name is displayed requests the point-to-point list for that matrix.

8. Most diagnostic functions, and a few other functions, can be accessed by entering FUNC-DISPLAY-UP/DOWN (0-8-6/9).  Scrolling through this list shows the available functions:

ID	(panel absolute ID)

LEV2	(talk level 2 assignments)

LSTN	(listen assignments)

NAME	(requests scroll list of all inputs talking to this panel)

TYPE	(shows function type, e.g. IFB, of all talk assignments)

MTX	(shows matrix name of all talk assignments)

GAIN	(press a listen key to enter volume control mode for that ssignment)

MVOL	(adjust the master volume for that panel)

V8.0	(shows panel software version is Version 8.0

TEST	(enter test mode)

TONE	(turn on tone generator)

EPNL	(show software setups)