Omneo IP ports.

| Port | Protocol | Multicast IP address | Purpose |
| --- | --- | --- | --- |
| 9470 | TCP | N.A. | OMNEO Devices offer the insecure Bosch OCP service on this port, i.e. they listen for incoming insecure TCP connections on this port. The service registration contains this port. |
| 9471 | TCP | N.A. | OMNEO Devices offer the secure Bosch OCP service on this port, i.e. they listen for incoming secure TLS connections on this port. No separate servcie registratin is performed on this port, the port itself is fixed. |
| 9472 | UDP | Configurable | OMNEO devices send periodic events to this UDP port. Controllers listen for incoming periodic events on this port. When a controller subscribes to a periodic event it can optionally pass a multicast address to which the event must be sent. |
| 9473 | UDP | 239.42.10.1 | Device reset commands are always sent to UDP port 9473. OMNEO Devices only start listening for incoming reset events after a controller has set a reset key. If a controller sends a 'reset all' command it sends it to the fixed multicast IP address 239.42.10.1. Therefore a device joins this multicast group when the reset key is set. |
| 4321 | UDP | 239.255.x.x | Dante uses port 4321 for multicast audio streams. OMNEO devices can only receive multicast audio streams that use an IP address that starts with 239.255. Note: for unicast audio streams Dante uses a UDP port range that can vary per device. When using label-based routing the proper port is automatically selected, when setting up audio manually the OcpAudioRoutingManager automatically uses a free port. |