RED LED task assignments PCiie

On PeriphII-e v1.22.0:

- red #0 = idle task

- red #1 = sort task (e.g. sorting remote alpha lists)

- red #2 = flash update task – ensure the config flash has the latest configuration

- red #3 = alpha change task. When an alpha (local or remote) changes, search through all key assignments for all talking panels; for each occurrence that it finds, set a flag for the send-assigns task to resend the assignment

- red #4 = broadcast task. When an alpha (local or remote) changes, forward it to each I/O card; the I/O card forwards it to keypanels, so they can keep their scroll lists up-to-date

- red #5 = send-assigns task (see previous e-mail)

- Send-assignments task.

- Previous email below.

- Basically, this is the task that sends any key assignments (and also setup page assignments) to the panel. It has a lot of work to do in the following instances:

- (a) On system start up (sending assignments to panels as they power up)

- (b) If a Periph starts up, and goes active immediately because there is no other (active) Periph in the other slot (it has to send all assignments to all talking panels, since it doesn’t know whether they ever got their current key assignments)

- (c) If trunking data is first connected. For each updated remote assignment, it has to resend that assignment to whatever panels have it assigned to a key.