Overview of Rework

The family of RVON products has a problem with a polarized tantalum capacitor which connects to a transceiver IC.  The capacitor is installed incorrectly which can cause connectivity problems in these products.

A polarized capacitor is not required for the application and rework consists of removing the tantalum capacitor and replacing it with a non-polarized ceramic capacitor.  Because the tantalum capacitor can be identified by a polarizing stripe on one end it is easy to identify PCB assemblies that have already been corrected.

RVON PCB Assemblies Affected

F01U191406 (90307877000) RVON-2 PCB KP32                          Revision after rework – H

F01U191407 (90307877001) RVON-2 PCB DKP32                       Revision after rework – H

F01U191408 (90307877002) RVON-2 PCB KP32 CLD W/RC      Revision after rework - H

F01U191375 (90307835000) CRONUS RVON                               Revision after rework – E

F01U191387 (90307848000) RVON-16                                           Revision after rework – D

F01U191333 (90307757001) RVON-1 for KP                                 Revision after rework – M

F01U191334 (90307757100) RVON-1 for RVON IO                      Revision after rework - L

Capacitor Needed for Rework (1 per PCB assembly)

F01U163128 (102881882) CAP CER 1.0 uF, 50V, 10%, X7R 1206

Procedure

- Remove tantalum capacitor.  See locations for RVON PCBs on next page.

- RVON-2                   -> C66

- CRONUS RVON     -> C8

- RVON-16                 -> C21

- RVON-1                   -> C8

- Install Ceramic capacitor.

- Update revision on PCB to reworked revision.

Done