- From: Damien Egan [mailto:damiene@solidstatelogic.com] 
Sent: Thursday, June 14, 2012 8:59 AM
To: Chuck Roberts
Subject: RE: Madi Coax to Fiber convertor

Hi Chuck,

It is multimode, 1310nm.

Hope that helps.

Damien Egan

EMEA Distribution Manager  - Solid State Logic
Tel:    +44 (0)1865 842300
Mobile: +44 (0)7920 474568
Fax:    +44 (0)1865 852239 - direct fax
www.solid-state-logic.com

There are now SPFs with 1310nM for both single & multimode. The Fiber itself makes the difference:

Fiber Types And Typical Specifications

| Core/Cladding | Attenuation | Bandwidth | Applications/Notes |
| --- | --- | --- | --- |
| Multimode Graded-Index |  |  |  |
|  | @850/1300 nm | @850/1300 nm |  |
| 50/125 microns | 3/1 dB/km | 500/500 MHz-km | Laser-rated for GbE LANs |
| 50/125 microns | 3/1 dB/km | 2000/500 MHz-km | Optimized for 850 nm VCSELs |
| 62.5/125 microns | 3/1 dB/km | 160/500 MHz-km | Most common LAN fiber |
| 100/140 microns | 3/1 dB/km | 150/300 MHz-km | Obsolete |
| Singlemode |  |  |  |
|  | @1310/1550 nm |  |  |
| 8-9/125 microns | 0.4/0.25 dB/km | HIGH! ~100 Terahertz | Telco/CATV/long high speed LANs |
| Multimode Step-Index |  |  |  |
|  | @850 nm | @850 nm |  |
| 200/240 microns | 4-6 dB/km | 50 MHz-km | Slow LANs & links |
| POF (plastic optical fiber) |  |  |  |
|  | @ 650 nm | @ 650 nm |  |
| 1 mm | ~ 1 dB/m | ~5 MHz-km | Short Links & Cars |

CAUTION: You cannot mix and match fibers! Trying to connect Singlemode to Multimode fiber can cause 20 dB loss - that's 99% of the power. Even connections between 62.5/125 and 50/125 can cause loss of 3 dB or more - over half the power.