Trunk Master Installation, NEW Partition.

The existing installation checks for a valid partition table. If not present, it creates a default partition table. I’m not sure why there would be a difference depending on whether the partition table is present or not.

If you run into the problem (computer displays “LI” during boot-up, then hangs), you can wipe out the partition table as follows:

- Boot off the installation CD

- Select option 2 (install the TM-2000 software). However, at this stage, we won’t actually install anything.

- Basic configuration information – pick any of the 3 options (doesn’t matter, we won’t use it)

- When it asks if you want to change any of the values, hit Enter (= don’t change them)

- Enable VDP support => select “n”

- Partition options => select option 1 (abort the installation)

- You will now have a “demo_tm login” prompt. Login with the user name

root

(no password is required). You will end up with the prompt “bash-2.04# “.

- Enter the command

dd if=/dev/zero of=/dev/discs/disc0/disc bs=1024 count=1024

- Once the previous command completes, hit the reset button on the computer (or cycle the power off and then on again).

- Boot off the installation CD

- Install the TM-2000 software as usual.