INSTRUCTIONS

CONVERT SONY 1500/1000 CAMERA TO RECEIVE RETURN IN STANDALONE

- Remove left side cover(OUTSIDE PAD ASSY, 2 screws) and heatsink, which reveals SDI-84 board.

- Of (2) coax harnesses connected to SDI-84, disconnect the cable connected to the Prompter out 2 connector(CN851) or labeled SDI 1, CN201, or Link A and connect this removed cable to the EXT VID IN CN601.

- Set Dip switch S201-2(Prompter 2) on the SDI 84 board to OFF

- Set Dip switch S201-3(VBS RET) on the SDI 84 board to ON.

- In the RAM FUNCTION page of the SERVICE menu, set the STANDALONE of RAM FUNC to RET SYNCHRO

- On back of camera, return selector knobs must be in 2-3 or 4 positions.

- The BNC under the SDI 1 BNC is now the return input for analog video.

- The EXT RETURN page is added to the MAINTENANCE menu and the aspect ratio can be set.

- Switch next to test out BNC must be set to RET IN.

After this mod is complete :

- SDI 1 cannot be used.

- In 23.98psf, 24psf or 25psf this mod does not function.

- The VBS return is not output even if SDI 2 is set to RETURN.