BKP-7311 ALIGNMENT

NOTE:  Clear all CCU adjust data by turning on bit 8 of S1002 on the AT board and powering up for a few seconds.  Turn off bit 8 after this process.

Connect a properly aligned camera to the CCU-700A

Auto black the camera.

Set the master black to -99

Adjust the analog black balance on the front of the AD board (R-Y, B-Y) for minimum subcarrier on the vectorscope.

Adjust the Analog Y to 7.5IRE

Looking at the 601 waveform monitor in vector mode x10 gain, adjust the digital black balance R-Y, B-Y on the front panel  for minimum subcarrier (center the dot).

Turn on color bars.

Set digital Y black balance on front panel to maximum CCW so you can see the start of the line.

Adjust the video phase control(RV2771) so the tail end of the 77 IRE bar is 7.7usec from the start of the line.

Readjust the digital Y black balance control for no setup.

Adjust the Y gain (RV2031) for 700mv on the 601 waveform monitor

Adjust the B-Y and R-Y gains (RV2231 and RV2131) to put the color vectors in the boxes on the 601 (75% bar vector mode)

NOTES:

You will  need the following items to adjust the AD board.

1 ea	BKP-7300	CCU extender board

2ea	1-774-157-11	coaxial adapter

1ea	1-751-813-11	cable