- Turn on power to the MSU-750 while holding PARA, PANEL ACTIVE and camera select button 1 pressed.  The numeric keys appear on the display.

- Enter “0359” for the security code and press OK. The Engineer Protection display now appears.

- To delete the security code, press “Code Delete”.  To temporarily disable the security code, press “Code Enable” to change it from inverse video to unhighlighted display.  (Once you press it again to return it to inverse video, the security code is enabled).

- As the message “Code Delete OK?” is displayed if you press “Code Delete” in step 3, press OK to return to the Engineer Protection display.

- Press “Exit”.

750 MSU:  99036750

700 MSU:  96700359