Menu Thru RMB750

Accessing the camera menu through the RMB750 display screen. Sometimes the CAM VF is off. You have to go in and turn it back on.

- Turn on the Security Mode by hitting the maintenance button.

- Then got to RM Config

- Hit security mode

- Turn on ENGINEER MENU

- Back out of that page and go to SW SETTINGS

- Change menu control to CAM VF