Notes for controlling MVS-6500 series processor from CCP-8000/6000/9000

November 1, 2012

When MVS-6500 series processor is controlled from CCP-8000/6000/9000 control panel, there are many limitations. Please note the following limitations.

<System Notes>

- CCP(SCU) should be set as Unit ID = 3.

- Only one CCP can be connected.

- On Menu7312.2 of CCP, please turn on [FM、Data Port Enbl]

- Please set DIP Switch S3802 Bit 2 On of MVS-6500 processor CA-85 board

- When DCU is connected, in Menu7312.1 of CCP, please turn on [Tally Master]

- To connect with MVS-6500, Ethernet switch is necessary.

MVS-LAN, CTL-LAN, Data-LAN, Peripheral-LAN every LAN can be connected to the same Ethernet switch (V LAN)

- Please use the latest version software for CCP

<Uncontrollable Functions>

- MVS-6500 processor SERIAL TALLY(D-sub 9-pin) port and REMOTE1 to 4(D-sub 9-pin) port

- MVS-6500 processor Parallel Tally output (when you need Parallel Tally, MKS-8700/2700 is required)

- AUX Mix

- CCR (Primary Inputs、AUX outputs) (MVS-6500 does not support Full function CCR of MVS-7000X)

- MKS-6570 [Drop Shadow] (Menu4115)

- MKS-6570 [Anti Moire Filter] (Menu4163)

- Color Background 2 (Color Background 1 can be used)

- Format Converter Output(Format converter input can be used)

- MVS-6520 2 channels of Multi Viewer

- NFS

<Function Limitations>

Output Assign (Menu7333.1）

- Output 25-32 are not assignable(work as default but no indication on the menu) (Output 1-24 are assignable)

- MVS-6530 Multi Viewer

CH1(MV1）

- Only Output 25 can be used for MV1

- Inputs for MV windows cannot be assigned

CH2(MV2）

- Only Output 26 can be used for MV2

- Inputs and Outputs for MV windows cannot be assigned

- Resizer

- When you open the following menu, indication for [Rotation Enable] On/Off and [X], [Y] cannot be refreshed.

M/E-x > Keyx > Processed Key > Border/Crop

M/E-x > Keyx > Processed Key > Resizer Process

M/E-x > Keyx > Processed Key > Enhanced Effect

M/E-x > Keyx > Processed Key > Rotation

To refresh the indication, you need to open the menu M/E-x > Keyx > Processed Key again.

- Frame Memory

Only partition 1 to 5 can be used out of 15 partitions of External HDD.

[Append] cannot be used for Backup and Restore for External HDD (Menu 2562)

Format for External HDD is possible and 15 partitions appear but only partition 1 to 5 can be used.

- REMOTE S1‐S2（D-sub 9-pin)

The remote control works as default setup.

- Menu 3211 [RemoteS1], [RemoteS2] settings are invalid.

- Menu 7337.1 [EditorA], [Editor B], [AUX] settings are invalid.

- Menu 7337.5 Control Mode ([Enable]/[Disable]/[Manual]) settings are invalid.

- SWER GPI

- SWER GPI Out Assign is 1-8 fixed.

- Router/Tally

- Menu 7361 Matrix Size : only [136x138] and [128x128] works correctly.

- Following Source and Destination cannot be controlled.

Source      : FC1 to 8

Destination : P/P Key5 to 8 Fill、P/P Key 5 to 8 Source

- Diag

- Menu9991.2、9991 Diag ; Error and Fan display for SWR1 and DME1 are not correct.

<Some notes>

- Menu 7316 SWR1, DME１ Version  are displayed with #

- AUX Bus 1-48 are displayed but Aux 25-48 are invalid.

- Output 1-48 are displayed but 25‐48 are invalid.

- M/E-3、M/E-4 and Key5-Key8  are displayed but these cannot be used except MVS-6530 PP DSK5-8.

- Edge Menu for Key3,4, 7, 8 are displayed but cannot be used.

- In Menu 2564/2565, buttons for FM Ext Clip are displayed but cannot be used.

- Menu of DME Brick is displayed but cannot be used.

- Menu of DME 3ch and 4ch are displayed but cannot be used.

- In Menu 7337.6/7343/7344 , setting for DME1 can be done but invalid

- Menu for Video Process is displayed but cannot be used.

- In System Format, 1080PsF/25 and 29.976 are displayed but cannot be used.

- In 1080PsF/23.976 and 24, BB is displayed but cannot be used. (Tri-Sync can be used)

- Operation for Util2 Bus, DME Util 1/2 Bus can be done but actually don’t work.