NEREUS SONY PBUS LANCE CONTROL SETUP

Connect the lance Pbus serial output to the first Remote input on the Switcher chassis.  This is DCU1_Port 3_1.   (of course you also have to connect the remote ports from the Lance to the remote control ports of the Spotbox (usually 4 of them).  Also connect the lance control (TDC100) to the control port of the Lance mainframe.

Check the DCU Serial Port assign in the Sony Switcher.  It should say P-Bus for DCU1_PORT3_1 and VTR on the next 4 ports (for 4 channel control).  If it doesn’t; see below

Navigate to ENG Setup / PANEL / DEVICE INTERFACE and make sure PBus control is on “Timeline” and not “trigger”.  Enter the next Menu needed by pressing the DCU Serial Port/MPE Assign.

Use Menu 7325.4 (located from the above notes) to assign the Serial Port/MPE.  Assign PBus to port 3_1 and then DEV 1-4 to the VTR positions.

*SPOTBOX CONTROL SHOULD BE ODETICS 1-4