For Networking Prime and Red computers a ‘Virtual Network adapter’ is used between

the lower left USB ports of the two computers. (Titan is upper left as the lower 2 not working)

This should appear in Start/Settings/Control Panel/System/Hardware/Device Manager/Network Adapters. The Driver is in Vista admin tools driver pool.

Once the adapter is on line then the ip addresses need to be set.

in start/settings/network and dial up connections – right click the usb virtual network adapter and select ‘properties’

In properties dialogue box select ‘internet protocal (TCP/IP)’ and then select ‘properties’

In this dialogue box you can set the ip address for the virtual network adapter.

Primary – 192.168.250.250

Submask - 225.225.225.0 (default)

Red – 192.168.250.251

Submask - 225.225.225.0 (default)

(if a dialogue box appears saying that the IP address is taken by another adapter answer ‘No’ to assign address to current adapter.)

When both IP addresses are entered you should be able to go back to start/settings/control panel and from drop down TOOLS menu select ‘map network drive’

You should receive a dialogue box with drive letters and locations.

Drive: select ‘Z’

Folder: Select ‘//192.168.250.251\Backup

(If when drive is selected the folder is already on the line, it is already mapped)

Also ensure that in D950System.ini the line for RedundantPCIP=192/168.250.251 is uncommented and correct.

+in D950system.ini

Prime - ScoreIp1=192.168.1.60

Red -    ScoreIp1=192.168.2.60