Calrec Audio Ltd

Alpha and Sigma Bluefin Console

Software Installation

Software Team

Created: 		12 October 2004

Last update: 	03 November 2008

Software Installation	3

Important Notes	3

2.6	3

2.5	3

2.2c	3

2.2	3

2.00d	4

2.00	4

1.29	4

1.28	5

1.27	6

1.26	6

1.25	6

1.24	6

1.22	6

1.17	6

Upgrading from previous versions	7

Before Upgrading	8

Metering setup	8

Default Memory	8

Memories	8

Desk Options	8

Backup	8

PC logins	9

Upgrading PC software	10

Java Runtime Environment	11

Acrobat Reader	11

Config Application	11

Setup Application	11

Programming Utility	13

Memories & Runtime Options	14

Updating Individual Components	15

Updating the Bluefin DSP card	16

# Software Installation

Important Notes:

### 2.6

Note: New TFT firmware is required to display the track and aux cut icons.

If installing a second Elo touch screen for use with the lock screen feature see the section for 1.28 installation instructions down the page.

### 2.5

New hub boot loaders are recommended.

UN4873		V2.1

UN5221		V2.1

UN5221-2		V1.2

### 2.2c

From 2.2c onwards, moving coil meters will need to be recalibrated due to a software change/fix (BID1342/1674). VU meters will also need the 3k9 resistor removing from across the terminals (MN5066, MN5066-2). See dcc6602.

### 2.2

The lists have moved from setup into the FrontEnd.  The mic buss settings have moved from memories into Options.  The upgrade is described in detail in listUpgradeInstructions.doc which is on the network in R:\DATA\DOCS\Docs\V1_32_Development or in the docs directory on the Config CD.

Briefly:

- Before upgrading save a memory with label calrec description upgrade containing the mic open settings.  If pre 1.29 then rename the memory from num.mem to calrec-upgrade.mem Typically the memory saved will be the default memory.

- Continue desk upgrade as normal, when Setup is run it will create a file cust1/lists/defaultLists.ini containing the old list settings.

- Assuming hidden memory and options have been erased

- Run the FrontEnd

- Go to the Opt->List screen

- Load Options, this will take longer as the FrontEnd sorts out the new lists

- If there is no network then “Save to File/Load into Desk” the new list configuration

- If there is a network then load in the network configuration,  save it to the console, select to upgrade the network config.  This will add the network ports into their original lists.  Back to the Opt->List screen, name the new list configuration and “Save to File/Load into Desk”

- Save Options

### 2.00d

UN5221 requires new firmware to work in consoles with 8 hubs and this also requires the UN5469 to have kernel v1.3

### 2.00

Console and Rack

- Control processor UN4806 is replaced by new UN5469.

- Rack control processor UN4805 is replaced by new UN5384.

- All DSP cards (UD4796/UD5225) are replaced by one UD5533 (plus one for redundancy).

- JI5173 and OD5362 interface cards require new firmware.

- Panels require updating with new front panels, buttons and some with new software.  See document 926-096.pdf for upgrade information.

- TFT meters require software and firmware updates.

PC applications

- Java version is Java5, the new version of java must be installed.  To check the installed version: java –version from a cmd prompt should say  java version "1.5.0_06"

- config/config.cfg and progUtilLog.cfg file have been removed

- Setup logs are now in cust1/logs/setup and are in the same format as the frontend logs.

- progUtiliy/alphaprg.ini renamed to cust1/ports.ini.  The ports from a100fe1.ini have been moved to this file.  The new entries are:

PCPort=COM11

SecondaryPCPort=COM12

- cust1/A100fe1.ini renamed to cust1/frontend.ini and the ports information moved to ports.ini

- a100fe1 directory renamed to frontend and the a100fe1.jar renamed to frontend.jar

### 1.29

- ProgUtility has protection against reprogramming by accident.  To enable the programming buttons press “Enable FileUpload" then enter a supervisor username/password or  calrecservice/calrecservice.

To always allow programming put:

[DEBUG]

DEBUG=true

Into the alphaprg.ini

- To start logging as soon as the ProgUtility starts then alter the alphaprg.ini and set the relevant entry to true.  If the entries do not exist then it is assumed to be false.

[LOGGING]
ConsolePort=true
DspRackPort=true
IoRackPort=true
SecondaryConsolePort=true
SecondaryDspRackPort=true
SecondaryIoRackPort=true

UN4806-2 are required with Boot-loader 3.0 and Kernel 4.0.

The CA4724 panel requires software version 1.1.

### 1.28

- If an Elo touch screen is fitted, please note the following for Windows 2000 and later.

By default, buttons automatically release after they have been held down for 10 seconds, this time period is too short for the Calrec Applications.  To change the timeout to the maximum allowed of 2 minutes:

    - Login as calrecaudioadmin

    - Run regedit or regedit32 (click Start, select Run, enter regedit)

    - Drill down to:

HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Enum\SERENUM\ELOSERIAL\******* \Device Parameters

NOTE: The "******" entries are id numbers used by the Windows operating system

    - Double-click the UntouchTimeOut key

    - The Edit DWORD Value dialog box opens

    - Click the Decimal radio button

    - Value displays in decimal (default: 10000)

    - Enter the delay time in milliseconds 120000 (2 minutes, recommended)

    - Exit regedit when done.

    - Shut down and Restart the machine (a restart is required before the setting will take effect).

- New Key Caps Required

  - The CA4724 Functions Panel

Requires new key caps engraved as EXT MONO, EXT STEREO and EXT SURR.

- Remove the EXT O/P key cap

- Move the TONE CLEAR keycap to where the EXT O/P was

- Fit the EXT SURR where the TONE CLEAR was.

- Replace the VOC IDENT cap with EXT STEREO.

- Replace the EXT I/P with EXT MONO

  - The PY4885 Input\Output Panel

Requires new key cap engraved as “5.0 LEVEL”

- Replace blank key cap next to LFE on button.

### 1.27

- UN4873 and UN5221 should be upgraded to kernel V2.3

### 1.26

- UN4806 is no longer compatible, the UN4806-2 or later must be used. The boot loader needs to be v2, kernels need to be v3.0.

- Only packs E3, E4 and D3 are supported.

- For E3 and D3 audio packs, the DSP and I/O racks must be fitted with UD5225-3 DSP cards throughout with V1.1 firmware.

- For E4 audio pack, either UD4796, UD5225 or UD5225-3 may be fitted.

- E3/D3 Delay configuration: for the D3 and E3 audio packs, the delay configuration should specify “ALL” or “4” delay cards.  Do not set to “2”.

- E4 Delay Configuration: if delay is enabled, the configuration must specify “2” delay cards.  The UD5225 or UD5225-3 cards used must be fitted to slots 1, 3 and 7 (spare) in the I/O rack.  Note that if an existing desk is being upgraded, this will require the existing delay cards in the DSP rack to be swapped with cards from the I/O rack

### 1.25

This is a System Plus release.  Main changes are:

- alpha directory renamed to alpha, shortcuts on the desktop will need to be remade.

If an upgrade is performed, after uninstall  the ini files will be left in c:\alpha100\cust1 and C:\Alpha100\Alphaprg.  The cust1 directory should be copied to c:\alpha\cust1 and C:\Alpha100\Alphaprg\alphaprg.ini to c:\Alpha\ progUtility \alphaprg.ini

- Alphaprg directory has been renamed to progUtility

- The manual has been included in c:\alpha\manual acrobat needs to be installed to view.   Install c:\alpha\manual\AcroReader51_ENU_full.exe for Win95/98 or AdbeRdr70_enu_full.exe for 2000/XP.

### 1.24

The network software needs to be version 3.0E which requires new firmware, kernel, and software.

### 1.22

Take a note of the current meter configuration in setup.  Then start again with the 1.22 Setup and FE.

- In Setup assign the meter type to the correct slot

- Use the “Set DIP Switches to Defaults” button to allocated default settings for the DIP switches.

- Set the meter dipswitches to match the values in Setup using appropriate increments in dipswitch value for each panel type.

- After installing the desk run the FE and create the meter allocations on OPT->METER and “Send” then save into Options.

### 1.17

Update the Kernel on 4806, 4805, 4873

Check that the Calrec.dll in c:/windows/system has the date 3/10/02

new JRE to be installed

## Upgrading from previous versions

Upgrading the desk to use this version of software requires the following components to be updated:

- The PC software (Programming Utility, Config Application, Setup Application & Front-end Application)

- Update Config.ini

- Update setup.ini.

- The Core [UN5469] software (control.bin)

- The DSP Rack Control [UN5384] software (rack.bin)

- The Hub [UN4873/UN5221] software (hub.bin)

- The DSP card [UD5533] software and firmware (UD5533.zip)

- Existing desk memories & runtime options.

Note:  these instructions assume that the desk is being upgraded from the previous release of software.  Also, any firmware or hardware updates should be made before starting the software upgrade.

## Before Upgrading

### Metering setup

The Metering setup, in the Setup application, should be checked and noted.

### Default Memory

The default memory will need to be loaded onto the desk surface and then saved into a spare user memory for back up and memory conversion purposes.

### Memories

If any of the desk’s memories are to be retained then they must be backed-up to disk before starting the upgrade procedure.  This can be achieved as follows:

- Start the Front-end application if it is not already running.

- Ensure that the current desk settings are saved to a memory if they are to be retained after the upgrade.

- Go to the Mem screen.  Click Backup Memories.

- Click Select All and then choose a directory to backup the memories to (a sub-directory of C:\alpha\ is advised).

- Click Backup and wait for the progress bar to disappear.

### Desk Options

The desk’s runtime options (set via the Opt screen on the Front-end) will also be erased in the upgrade procedure.  These can be saved to the PC’s harddisk by going to Opt->File and clicking on Save options to Flash.  This will create a file called options.bin in the directory c:\alpha\cust1\options.

### Backup

For peace of mind it is advisable to backup the C:\alpha directory on the desk’s PC before upgrading the software.  This can be achieved as follows:

- Start Windows Explorer (Start->Programs->Windows Explorer).

- Select the directory C:\alpha in the left-hand pane of the Explorer window.

- Rename this directory to something like AlphaBackup

- If the memories backed-up above were not stored within the C:\alpha\ hierarchy these should be copied to another location to create a backup.

## PC logins

For PC running Windows 2000.

Username:	CalrecAudio

Password:		(no password)

Security:		“Power User” or “Standard User”.

Can install and run programs, but not change PC hardware settings, (i.e. set-up network, install drivers).

Notes:		This user is intended for normal running of PC.

Username:	CalrecAudioAdmin

Password:		calrec

Security:		“Administrator”

Full rights to PC, can install hardware, set-up network etc.

Notes:			This user is intended for use by customers to configure the PC for networking

etc.

Also to be used when installing software.

## Upgrading PC software

The first upgrade step is to ensure that the PC has all the correct software.  The Alpha Software installation CD has the latest versions of all the necessary software files.

The CD also contains the BIN and DSP files mentioned previously along with other necessary files.  It does not contain the config.ini (supplied by Calrec), setup.ini or config.bin files as these are unique to each desk.

To install the required files from the CD onto the desk’s PC do the following:

- Close any running programs.

- If  Windows 2000 login as: CalrecAudioAdmin, password calrec

- Insert the Alpha Software CD into the desk PC’s CD-ROM drive.

- The software installation wizard should start automatically if Autoplay is enabled for the CD-ROM drive.

- If not, run Windows Explorer and click on the CD-ROM drive icon.  Navigate to the install directory and double-click the icon labelled install.

- Once the Alpha PC Software installation has started, use the Complete installation option unless you need to install or ignore specific components.

- If  Windows 2000 logout and login as CalrecAudio no password,

Note:  The auxiliary card software files (e.g. kernels, bulk card software, bootloaders etc. - anything which is not one of the three main .bin files) can be found in a subdirectory bearing the card’s unit number.  For example, the kernel.bin file for the UN5384 can now be found in c:\alpha\un5384\.  To avoid confusion, delete the old-style subdirectories created in previous installations, namely:

.\4805Kern

.\bulk

.\CoreKern

.\dsp_boot

The desk is now ready to be upgraded.

## Java Runtime Environment

The Java Runtime needs to be upgraded.  The procedure is as follows:

- Insert the Alpha Software CD into the desk PC’s CD-ROM drive.

- The software installation wizard should start automatically if Autoplay is enabled for the CD-ROM drive.

- Cancel the Software Installation Wizard by pressing “Cancel” then “Exit Setup”, then “OK”.

- Start Windows Explorer (Start->Programs->Windows Explorer).

- Select the directory D:\JRE in the left-hand pane of the Explorer window.

- Double Click on jre-1_5_0_05-windows-i586-p.exe to launch.

- Follow the instructions in the wizard.

## Acrobat Reader

If the PC does not have Acrobat Reader installed then install c:\alpha\manual\AcroReader51_ENU_full.exe for Win95/98 or AdbeRdr70_enu_full.exe for 2000/XP.

## Config Application

Due to changes to the Config Application the config.ini file needs to be upgraded. This will be done by your Service Engineer.

- Insert Config Application CD

- Run D:/Config from Explorer

- Upgrade the config.ini following the instructions

- Do not make any other changes

- Save using File->Save

- Exit Config.

## Setup Application

Due to changes to the Setup Application the setup.ini file needs to be upgraded. The procedure is as follows:

- Launch the Setup Application by going to Start ->Calrec Alpha ->Setup Application.  It will take a few moments for the previous setup information to load.

- After a few moments the application will ask you to confirm the upgrade to the new version.  Click OK to proceed with the upgrade.  The Setup Application will then appear with the setup information.

- Unless any further changes are required you can now exit the application by going to File->Exit.  When prompted, select YES to save the new setup information.  This will create a setup.ini with the updated information.

The next stage is to upgrade the software components and configuration files of the various control processors in the Alpha desk using the Programming Utility.

## Programming Utility

The Programming Utility is used to program the processor control files (*.bin) and desk configuration files (*.ini) into the desk.

To reprogram the control processors the procedure is as follows:

- Run the Programming Utility by going to Start->Calrec Alpha -> Programming Utility

- Once started you’ll see one tab for each of the processors in the system.  Each of these processors will need various files to be programmed into them.

- Starting with the Console 1 tab click on Run Kernel to boot the card into Kernel mode.  Repeat this for Console 2 and DSP Rack 1 & 2.

- Go to the Console 1 tab.

- Click the Erase Hidden button to erase the hotbackup memory.

- In the Command box type UB [Send] to unlock the Flash memory.

- In the Command box type FE [Send] to erase the configurable Flash memory.

- In the Command box type EM [Send] to erase the user memories from Flash.

- In the Command box type LB [Send] to lock the Flash memory again.

- Click Send Hub s/w to send the hub software (hub.bin) to the processor.  Once this has finished, click Reprogram Hubs in Console to program hub.bin into each hub.

- Once this is complete click Send Core s/w to send the Core software (control.bin)

- Once finished, click Send Options to send the setup.ini and config.ini files to the Core.

- Repeat steps 5 to 12 for Console 2 (the backup Core processor).

Next, the DSP Rack Control processors need to be reprogrammed thus:

- Select the DSP Rack 1 (primary DSP Rack Control processor) tab

- Click Send Rack s/w to send the DSP Rack control software (rack.bin)

- Click Send Options to send the setup.ini and config.ini files

- The above steps should be repeated for DSP Rack 2.

Note:  the Hub tab in the Programming Utility is not functional in this release.

Finally, press the RESET button on the console and the Rack master reset button to reboot all the cards and start the console.  The Programming Utility may now be closed.

## Memories & Runtime Options

The final step is to restore any memories or runtime options that were saved before beginning the upgrade.

- Start the Front-end application (Start->Calrec Alpha ->Main Application).

- Go to the MEM->SETUP page then click on Restore Memories.  Use the Browse button to navigate to the appropriate directory (if necessary).

- Select the memories to restore from the list given.  Click Restore to start the restoration procedure.  This may take a few minutes depending on the number of memories and the size of each memory backup file.

- To restore options, go to the OPT.  Click on Load options from Disk to Flash.  The process should take just a few seconds to complete.

- Load the memory used to store the back-up of the default memory onto the console surface. Now go to the TECH->INFO page. Click Save Default Memory to save back into the Default memory.

This completes the upgrade procedure.

## Updating Individual Components

It may be necessary from time to time to update individual software components of the desk (e.g. a control processor [BIN] file).  The general procedure for this is as follows:

- Copy the BIN file into the C:\alpha\bin directory.

- Run the Programming Utility.  You will need to run the kernel (click
Run Kernel) of the appropriate cards depending on which component is being updated:

- CONTROL.BIN, HUB.BIN	Console 1 & 2

- RACK.BIN				DSP Rack 1 & 2

- Click on the following buttons depending on which component is being updated:

- CONTROL.BIN	Send Core s/w

- HUB.BIN		Send Hub s/w then Reprogram Hubs in Console

- RACK.BIN		Send Rack s/w

- If either the CONFIG.INI or SETUP.INI have changed then the Send Options button will need to be pressed for ALL of the control processor cards in the console and both racks.

- Updating individual PC applications will usually be done using an installation program similar to that used previously.  In any case, specific instructions for such updates will accompany the installation files.

Note that these are generalised steps.  In some cases alternative or more specific update procedures will be required in which case the Software Team will provide suitable instructions.

## Updating the Bluefin DSP card

Updating the DSP card requires a serial port to monitor progress and an ethernet connection (10baseT or 100baseT) to upload the new files.

The serial connection required is a straight through 9 pin m-f cable.  Hyperterminal should be used with normal Calrec settings (115200 8-N-1).

The ethernet connection requires a cross over cable (unless the PC has an auto MDIX adapter).  The card has a DHCP server built in to assign an IP address to the PC so no manual configuration is required.

- With both cables plugged in, reset the DSP card (button marked RESET) while holding down the button marked CONFIG and do not release this button until four of the five green LEDs are strobing.  The serial terminal will show ‘***** CONFIG MODE *****’ and then initialise the network interface.  When this is done it will show ‘Download server started’.

- On the connected PC at a command prompt: Configure the IP address by typing ‘ipconfig /renew’ and then send the update file via TFTP by navigating to the drive and folder where the update file is and typing ‘tftp –i 192.168.1.1 put UD5533.zip’.  The serial terminal will show the file size that has been received.

- The serial terminal will continue to show the progress of the update as all the component changes are implemented.  This can take many minutes if the update involves a firmware change.  Do not unplug the card, reset the card or switch the power off during this time.

- When the update is complete a message will be displayed ‘Programming complete’.  The card has to be re-powered for the changes to take effect and this is done by unplugging the card and replugging it shortly after.

- Repeat on the redundant card (if fitted).