- Calrec Audio Ltd

Internal Use Only

- Bluefin Software Release

V2.8A

- Software Team

Created: 	12 October 2004

Last update: 	27 February 2009

Introduction	6

New features	6

2.8	6

Support for 33 SDI boxes.	6

2.7	6

Support for 8 input SDI box.	6

2.6C	6

Hydra route checking	6

2.6B	6

Offline Editor	6

2.6	6

Track Output Cuts	6

Cut State on TFT meters	6

Fader Layer Locking \ All A, All B exclusion	6

Support for Omega 4th bulk PSU	7

2.5A	7

Ross Video	7

2.5	7

SDI  - Not Supported	7

2.4	7

Touch Disable	7

Pseudo Mono Channels	7

Configurable Channel Buttons – Alpha Only	7

192 Auto Faders	7

Force Ownership Drop – Hydra Outputs	8

Grab Ownership – Hydra Outputs	8

GPIO Test	8

PFL Backstop Fader Assignment	8

TB Button – Time reduction	8

Filter Off past endstops	8

Tech Mode – Clear Memories	9

2.3B	9

Omega	9

2.3A- Beta	9

Omega	9

FrontEnd COM ports	9

2.3	9

Down Mix	9

Spill Mains	9

Eq on Mains + LFE filter	9

Expander on Mains	9

Insert send delay on Mains	9

TFT - CRLS Meter and Meter 1-3 (or Main, Anc1, Anc2)	10

Replay	10

Track output screen	10

Routing matrix	10

Divergence on Stereo Channels and Groups	10

Omega Bluefin Panels	10

Omega Bluefin DSP	10

2.2	11

Expanded delay features	11

Self healing DSP	11

Multiple Channel Selection	11

Override Auto Fader	11

Port Lists	11

PC Panel Screens	11

2.1A	11

2.1	11

Sigma Bluefin DSP	11

Auto faders	11

Dynamics Sidechain listen	11

Dynamics - Soft Knee and Recovery Hold	12

Shaft Default Press	12

Patch Dependent Input	12

Sigma Bluefin Panels	12

2.00	12

Alpha Bluefin DSP	12

PC Applications	12

1.29	12

Memory Restore	12

Backup to Removable Media	13

Programming Utility	13

Support for Front End Fader	13

Front end Wild Delay Assign	13

Surround External Meters	13

Local Inputs to Hydra Outputs	13

Multiple Functions per GPI	13

Jump to Assigned Fader	13

Screen Display Maximise	13

Sorting Inputs and Outputs	13

Layer Views	13

Move Fader Strip	13

1.28	14

5.0 Level Control	14

1.27	14

1.26E	14

IU5531 Support	14

1.26D	14

1.26C	14

1.26B	14

1.26	15

5.1 Surround channels	15

Surround channels and groups	15

1.25	15

New Fader Panels	15

System Plus	15

1.24	16

Hydra Redundancy	16

Hydra I/O Issues	16

1.23	16

Monitoring System	16

New Delay	17

Metering System support	17

1.22	17

Metering System	17

Nexus Router support	18

Input 1/Input 2 Gain Linking	18

Performance Improvements	18

Config Application & Programming Utility	18

1.19	18

Vca Groups as Slaves of Another	18

Partial memories	18

Cross fades from tallies	18

MADI	18

1.18D	19

MADI	19

1.18	19

On/Off control from Wilds	19

New Panels – IU5219 Channel fader, IM5415 Main fader, IA5392 Assigned fader	19

Linked Input Gains	19

Separate Wilds for Input 1 and 2 Gains	19

Non Switcher Network	19

1.17	19

Hydra Audio Networking Stage 2	19

1.16A	20

MADI support added	20

Joystick support added	20

New PC user interface	20

New PC Front End Application	20

Input 1, Input 2 Switching	20

Preview Memory	20

Exec Led	20

Studio LS Monitor	21

Pfl	21

Allocated Metering System and External Meter Inputs	21

1.12d	21

Bird Beater	21

Monitor Outputs	21

Mix minus Direct Input	21

Reverse Interrogate Routing	22

Audio package configurations	22

New PC screens	22

Dynamics	22

Monitor panel	22

I/O Matrix panel	22

Group master fader panel	22

Issues Resolved	23

2.8A	23

2.8	23

2.6F	23

2.6E	23

2.6D	23

2.6C	23

2.6B	23

2.6A	24

2.6	24

2.5A	24

2.5	24

2.4A	25

2.4	25

2.3D	25

2.3C	25

2.3B	25

2.3A	26

2.3	26

2.2E	26

2.2D	27

2.2C	27

2.2B	27

2.2A	27

2.2	27

2.1B	28

2.01	28

2.00E	28

2.00d	28

2.00c	28

2.00b	28

2.00a	28

1.29	29

1.28B	29

1.28A	29

1.28	30

1.27	30

1.26E	30

1.26D	30

1.26C	30

1.26B	30

1.26	30

1.25	31

1.24	31

1.23C	31

1.23B	31

1.23A	31

1.23	32

1.19	32

1.18D	32

1.18B	32

1.18A	32

1.18	32

1.17B	33

1.17A	33

1.17	34

1.16B	34

1.16a	34

1.16	34

1.15b	34

1.15a	34

1.12d	35

1.12c	35

1.12a	35

1.12	35

# Introduction

The software release document details the new features in this release. Information on upgrading and installing the software can be found in Bluefin_installation.doc. A list of the software modules can be found in Xxxxx_sw_modules.doc (Xxxxx=desk type).

# New features

## 2.8

### Support for 33 SDI boxes.

## 2.7

### Support for 8 input SDI box.

## 2.6C

### Hydra route checking

After every memory load, a request is made to all IO boxes to send routing tables to the console if they have routes made there.  The results are checked against the known routing for errors which are corrected and logged if found.

## 2.6B

### Offline Editor

The offline editor is now available to allow operators to configure show setups on a PC remote from the Calrec console.

## 2.6

### Track Output Cuts

Track outputs can now be cut.

### Cut State on TFT meters

If a track or aux is cut an icon is displayed beneath the appropriate leg of each TFT meter.

### Fader Layer Locking \ All A, All B exclusion

An option can set so that selected faders are excluded from the layer switching actions of the “All A” or “All B” buttons on the console. When this options is active an additional option can be set to prevent pressing fader assign buttons from changing layers. These two options combine to ensure critical audio is always active on a particular fader minimising the danger of accidentally being switched to the wrong layer.

Lock screen

The user can now lock a screen to a second monitor if one is available and it is enabled in config.

### Support for Omega 4th bulk PSU

Omega can now support 4 separate bulk supplies for use when a separate console and rack powering system is used.

Support for surround Vu meter

Surround Vu meters can now be configured using the meter page on the frontend

## 2.5A

### Ross Video

Our consoles now interface with Ross Video Synergy systems.

## 2.5

### SDI  - Not Supported

## 2.4

### Touch Disable

A command line only feature has been included to disable the control software’s response to touch messages from the fader panels. The setting is saved in options. The commands can be typed in the control window of the programming utility. Command ‘touchdis’ will display the current setting, ‘touchon’ enables touch processing, ‘touchoff’ disables touch processing. After a change the run time options must be saved for the effect to be retained.

The state of the fader panel touch LED is set to off when user memories are loaded. The touch state is always set to off when a secondary control processor take over occurs. The fader panel touch LED’s are turned off after a memory preview.

### Pseudo Mono Channels

The use of 2 stereo channels with the same stereo port attached as 2 mono channels. LB is selected on one channel and RB on the other.

Each channel is now able to set their gain with no affect on the other.

### Configurable Channel Buttons – Alpha Only

The four buttons found on the channel control panel are now all set from the Configuration. A new option of LB, RB has been included.

This feature requires a new Configuration, Programming Utility and Front End.

### 192 Auto Faders

The number of auto-faders available has been increased to 192.

### Force Ownership Drop – Hydra Outputs

It is now possible to force ownership to be dropped on networked outputs, this just removes the ownership from the port; audio routing is not changed, the port is then available for any networked console to route to.

### Grab Ownership – Hydra Outputs

It is also possible to grab networked output ports. To grab a networked output port is must appear as patched on the output patching screens and it must not already be owned by the grabbing console. If these two conditions are fulfilled then the port can be added to the grab list. The port can then be grabbed from the grab list output page. The process has deliberately been made a multistage process because grabbing outputs does change the audio routing and could potentially disrupt the broadcast from another console.

### GPIO Test

There is now a means of testing GPI actions from the Front End by simulating Opto events and a way of viewing Opto status by simulating Opto status LEDs.

There is also a means of testing GPO actions from the Front End by simulating relay events and a way of viewing the status of the functions associated with a relay by simulating function status LEDs.

### PFL Backstop Fader Assignment

As well as initiating PFL for a particular fader, the PFL backstop, while held down, now also changes the currently assigned fader for the desk to the one being PFL-ed.

All controls and displays present or operate on the data associated with the PFL-ed fader.

When the PFL backstop control is released the desk reverts the currently assigned fader back to the one that was assigned before the PFL backstop operation.

This feature is configurable so that a user can switch it off from the Front End.

The feature defaults to disabled.

### TB Button – Time reduction

The timer for the talk back panel buttons was reduced to make it more difficult to unintentionally latch the talkback on when just issuing a short response. The button has to be stabbed on and off for a very short time to cause the talkback to latch.

### Filter Off past endstops

The low frequency filter has an extra frequency setting, as it is turned fully anti-clockwise the filter is turned from 20 Hz to being flat.

The high frequency filter has an extra frequency setting, as it is turned fully clockwise the filter is turned from 20 kHz to flat.

The feature allows the operator to switch the filters in using the single in\out button and then effectively turn off one of the filters.

The LFE spill of a surround channel is defaulted to have the low frequency filter switched out, all other channels default with both filters active.

### Tech Mode – Clear Memories

The clear all memories button on the mem screen is disabled unless the user is in tech mode.

## 2.3B

### Omega

Introduction of Omega  with Bluefin.

## 2.3A- Beta

### Omega

Introduction of Beta Omega  with Bluefin.

### FrontEnd COM ports

If the entry “SecondaryPCPort” is left blank in cust1/ports.ini then the FrontEnd will only connect to the Console through the Primary Port (PCPort entry).  This can be used if running the FrontEnd on laptop so only one COM port is available.

## 2.3

### Down Mix

When a Surround path is selected its spill legs display on the spill fader panel. When the down-mix button is selected the spill faders will switch to allow adjustment of each individual spill paths contribution to the down-mix.

This will cause the LoRo Main Buss Output to have the following Down mix levels applied:-

LR	was	0	now	-4.5

C	was	-3	now	-7.5

LsRs	was	-6	now	-10.5

LFE	No change

### Spill Mains

When a Surround Main is selected its spill legs will display on the spill fader panel, allowing adjustment of each main spill leg (LR, C, Lfe, LsRs) as well as the overall surround main.

### Eq on Mains + LFE filter

Mains have full EQ capability ( 6 bands + 4 bands for dynamics sidechain ).

The HF filter range has been expanded down to 50Hz for LFE legs that are independent of the surround master to provide a low pass LFE filter. This defaults to IN and set to 80Hz for main outputs. The default LFE filter setting for channels and groups is 80Hz and out.

### Expander on Mains

The dynamics expander has been implemented for main outputs. Same as channels/groups.

### Insert send delay on Mains

Assignable delay has been implemented for main insert sends. Same as channels/groups.

### TFT - CRLS Meter and Meter 1-3 (or Main, Anc1, Anc2)

Choice of meter type ‘Surround’ has changed to ‘Surround, Stereo or Mono’ when this option is selected the meter changes its display to show the width of the source being metered.

### Replay

Channels assigned to replay switch to input 2 when replay is turned on and switch to input 1 when replay is turned off. A channel can be assigned to replay only from the front end screen. Replay is turned on and off be selecting the “Replay” button and then the “Exec” button before the timeout occurs.

### Track output screen

A screen to display and control all the track outputs has been added to the FE under the panels section.

### Routing matrix

A screen to allow the user to toggle, route or un-route channels has been added under the user section on the FE. The screen allows the user to click on a single cell which will toggle the route or select multiple cells before deciding what action to perform on those cells.

### Divergence on Stereo Channels and Groups

Divergence control is now available on stereo channels to control the centre level relative to left and right output to a surround main. At one extreme the centre is a mono mix of the left and right leg which can be balanced using the pan control, at the other extreme the output is stereo in left and right only which can be balanced using the pan control, half between the centre, left and right all have audio which can be balanced using the pan control.

### Omega Bluefin Panels

Support for the new Omega panels.

Auxiliary Route Main Panel	AM5630

Monitor Memory Panel		MP5631

Input Output Eq Dyn Panel		PY5632

### Omega Bluefin DSP

This provides:

- 160 mono channels which are organised into 48 stereo and 64 mono channels  allowing up to 24 surround channels or

240 mono channels which are organised into 72 stereo and 96 mono channels allowing up to 36 surround channels.

- 3 selectable meters from monitor panel.

- Other parameters as Alpha/Sigma DSP.

## 2.2

### Expanded delay features

Delay now available on group inputs, direct outputs, insert sends and returns for channels/groups and insert returns for mains.

### Self healing DSP

Self healing DSP. Reset of faulty DSP after failure and improved auditing of potential DSP problems. Number of DSP cards can be specified in the setup application.

### Multiple Channel Selection

Ability to select multiple channels on the User->Chan page which can then be cleared, or have their path type changed.

### Override Auto Fader

The user is now able to enable/disable the auto faders from the console surface, when they are attached and are being controlled by the Optos, without having to un-patch either the auto fader or the opto.

A new button on the Input/Output panel, next to the Insert In/Out button can now be used to toggle the new Override Auto Fader mode and the current Insert In/Out mode. When in this new mode the Insert In/Out button will enable or disable the currently attached auto fader instead of the usual In/Out operation.

### Port Lists

Allocation of Input/Output ports to lists is now handled entirely within the FrontEnd.

The network ports are also allocated to lists on the same Opt->Lists screens.

The ports lists which are not “Front End only” lists are saved in Options.

Attachment of Mic Busses to Input ports is done from the Opt->Lists screens.

### PC Panel Screens

The Front End input output panel screen is now available.

## 2.1A

New Calrec logos

## 2.1

### Sigma Bluefin DSP

This provides:

- 320 mono channels which is organised into 108 stereo and 104 mono channels allowing up to 54 surround channels.

- Other parameters as Alpha DSP.

### Auto faders

Implemented for Bluefin DSP.

### Dynamics Sidechain listen

Selectable from any path to feed the PFL buss.

### Dynamics - Soft Knee and Recovery Hold

### Shaft Default Press

To allow the shaft press of a rotary control to set the value of the control to a default. This behaviour will be enabled/disabled via the front end.

### Patch Dependent Input

The selection of either input 1 or input 2 will only be allowed from the channel control panel if the input being selected has a port attached. The ability to select either input 1 or 2 regardless of port attachment would still be available from the Input Output panel.

### Sigma Bluefin Panels

Support for the new sigma Bluefin panels.

Routing Matrix panel		RY5586

Auxiliary Main Panel		AM5589

Equaliser Dynamics Panel		QF5587

Talkback memory Panel		TB5588

## 2.00

### Alpha Bluefin DSP

This provides:

- 480 mono channels which is organised into 162 stereo and 156 mono channels allowing up to 78 surround channels.

- 256 output or external meter legs for TFT metering.

- 6 band EQ and filters in channel and 4 band EQ and filters in dynamics side chain.

- Fast redundancy swap of complete desk processing in case of any failure.

- All direct outputs available

- Up to 5.76s of delay on channel inputs.  Delay is assignable to different paths through the system.

- 192 insert sends and returns.

- All groups are separately switchable mono/stereo/surround.

- EQ on groups.

- 4 selectable meters from monitor panel.

Up to 8 WAB cards can now be fitted.

Auto faders are not supported in this release, this is to follow later.

### PC Applications

- FrontEnd has a new look and feel

## 1.29

### Memory Restore

The Restore Dialog has substantially changed to allow memories to be restored to any location.  Due to this memories are now saved called  <label>_<description>.mem  instead of the old <location>.mem

### Backup to Removable Media

Setup contains a new option to specify the drive to save backed up files to.  All the files created by the PC applications will be saved in the backup location.

### Programming Utility

See Installation Document, have to be in supervisor mode or use calrecservice/calrecservice to enable the programming buttons.

### Support for Front End Fader

Support for the Front end to use a separate fader from the console has been added to the delay and routing screens.

### Front end Wild Delay Assign

A new button has been added to the Front End user channel screen in the wild assign frame to allow customers without a delay panel to assign delay control as one of the wild controls.

### Surround External Meters

External meters can now support mono, stereo, and 5.1 widths.

### Local Inputs to Hydra Outputs

The output level when patching a local input to a Hydra output has been corrected.

### Multiple Functions per GPI

Each of the remote general purpose inputs can have up to ten functions attached. When the input is activated all the functions are activated. A function can only be connected to one general purpose input. One use of this is to allow all the components (four spill legs) of a surround channel can be simultaneously cut from one remote input.

### Jump to Assigned Fader

When the consoles assigned fader was changed the Front End channel input screen automatically jumped to the same fader. It is now possible to turn this feature off and on.

### Screen Display Maximise

Problems with some screen not being displayed correctly when maximised has been dealt with.

### Sorting Inputs and Outputs

When Diagnostic mode is chosen the input and output lists on the user channel screens can be sorted by rack, slot, and port or number or name.

### Layer Views

The action of the all A and All B buttons can be set up so that a short press changes the fader layer as in earlier versions but a long press allows a temporary or lathed view of the  chosen layer without destroying the current layer selection.

### Move Fader Strip

The action of the IO matrix panel To Fader button can be configured to move the whole fader strip or just one path as it does now. When the whole strip is moved the paths on both layers are swapped and the wilds are swapped.

## 1.28

### 5.0 Level Control

The 5.0 level control adjusts the gain to every leg except the LFE. In order to route a channel to only the LFE of a surround main then surround 5.0 can be turned off (faded out) and the LFE level turned up.

## 1.27

Additional Audio Packs A1,B1,B2,C1,C2,D1,D2 and E1 implemented.

## 1.26E

### IU5531 Support

Support for IU5531 spill panel added

## 1.26D

None

## 1.26C

None

## 1.26B

D3 pack is supported.

1.26A

Only pack E3 is supported.

Partial mem screen on FE has been removed until testing is completed.

Group talkback is now supported.

Isolates are supported.

Surround channel master fader meters now show a composite signal in Input, Output and Dynamics modes.  The meter shows the maximum input, output or gain reduction of all 4 spill leg faders.  Previously, only the front LR fader meter signal was shown on the master fader meter.

Preview mode is supported.

Delay is supported on each input of channels but not supported on groups.

If delay is made available the configuration application must used to set the number of delay cards to 4.

Each 5.1 surround channel has its own dynamics link bus available.  Each spill and/or the surround master can also be linked to the 4 user dynamics links.

Support for the IU5472 and CC5471 panels has been added.

## 1.26

### 5.1 Surround channels

5.1 Surround Channels provide the ability to control a discrete 5.1 source as a single channel. The system can have up to 20 surround channels. A surround channel is assigned from available mono and stereo channels. One fader path is chosen by the user to be the surround master. This path holds the master control settings which affect the operation of the surround channel.

Two stereo channels are used for each surround master to provide the LR and LsRs audio and two mono channels to provide the C and LFE audio. Each of these can be spilled on to a panel to provide individual control of the audio.

### Surround channels and groups

New screens on the Frontend at User->Spill to enable routing of surround legs and to mimic the spill panel.

Only pack E3 is supported.

The following features do not work or are unfinished in this release and so should not be used:

- Group Talkback

- Surround master fader bargraph only reflects front LR fader signals rather than a composite of all the surround leg fader bargraph signals

- Dynamics linking including the linking of surround channel spill faders

- Isolates

- Partial memories

- Preview

This release requires the use of UN4806-2 or later, UN4806 core processors are no longer compatible.

The UN4806-2 boot loader needs to be v2, kernels need to be v3.0.

The DSP cards must be UD5225-3 with firmware V1.1.

## 1.25

### New Fader Panels

Supports the following new fader panels:

IU5472	Channel Fader

IA5530	Assignable Fader

IM5529	Main Fader

These panels have the larger Cut/On buttons and their A, B Assignment buttons stay illuminated, Green or Amber dependant on the selected layer.

The Current Assignment is now illuminated Red.

### System Plus

New splashscreens for Frontend and new background for PC. Alpha100 directory renamed to Alpha.

The Operators manual has been included in c:\alpha\manual, along with acrobat installers.  It will appear on the Start Menu with the other Calrec Applications.

## 1.24

### Hydra Redundancy

The network software needs to be version 3.0E which requires new firmware, kernel, and software.

New screen added to FE Net->Device Status which shows the preferred port and which ports can be seen by the console.

Networking screens altered to allow a second IP address to be assigned to a I/O Box

The console and rack software now provide a network redundancy feature which monitors active network paths and re-routes audio automatically if the network path fails.

### Hydra I/O Issues

Remote mic cut system has been increased to 5 Mic Busses and now works with networked inputs. It also now takes into account the Channel/Group Cut button.

Networked Inputs can be patched directly to Local Outputs.

Networked Inputs now work with Opto’s (channel cut’s) and Relays (fader starts)

## 1.23

### Monitoring System

Introduction of Assignable Monitor panels.

The Panel fitted must now be chosen in the configuration. The old and new style panels cannot be mixed in a console.

The setup is used to select:-

The width, phantom centre and lfe off settings of the monitor outputs.

The Pfl Rtb LS.

The Sw 3 Input.

The Main Line Internal /External settings.

The New Monitor panels Output 1-6 and Meter 1-3 Labels.

Which Monitor Outputs work the Mic Open system.

Which Monitor Outputs work with Studio TB.

There are now 4 Surround and 4 Stereo Outputs.

On the Older panels the Main LS, Small LS, Desk H/P and Studio LS1 can be Surround, but Studio LS2 and Studio Phones can only be Stereo.

On the Newer panels the Main LS, Small LS, Monitor Output 1 and 2 can be Surround, but Monitor Output 3, 4, 5, 6 can only be stereo.

The monitor selector buttons are now allocated at runtime. No defaults are provided.

The selections allocated are saved with the options.

The monitor button selection screen has been moved from the set-up to the OPT->MON I/P+TB screen on the FE.

The user can now allocate 7 Banks of 16 selector buttons from a list of outputs. Each bank can be labelled using the ‘Change Bank Label’ button.

The Front End must be in ‘Technician Mode’ to set the monitor selector buttons on the older style panels.

The newer style monitor panels are updated when the ‘Save to file Load into Desk’ button is selected.

The LS Monitor Insert is now working (both with the new panels and the old).  The send ports are patched on the I/O \ Output \ Mon, TB & Osc screen.  The return ports are patched on the Options \ Mon i/p & TB \ Mon Sel (ext i/p) screen.

### New Delay

Delay has to be enabled and the number of DSP delay cards fitted (excluding the spare) set in the configuration utility before delay is available.

Delay controls can be assigned to wild controls. If the wild shaft push feature is enabled on the front end options misc. screen, pressing a wild delay control shaft will toggle the delay in and out.

The front end program now has a delay panel screen with a rotary control to mimic the delay shaft. To operate the delay shaft on the screen (using the mouse or touch screen) press within the button area then drag in a circular motion. Whilst dragging moving further from the centre of the button’s centre changes the delay value more slowly. The button press to assign wild delay controls is not implemented on the front end screen.

The buttons on the front end delay panel screen mimic those on the delay panel on the Alpha and Sigma. In addition the front end delay panel screen has buttons to select the display units as mS, PAL frames or NTSC frames. Changing the display units also affects the resolution of the delay shaft, nudge up and nudge down buttons accordingly.

The delay interrogate button on the front end panel screen lights when pressed but the panel button does not.

The Zeta does not have a separate delay panel. The delay control shaft and delay in and out button are available on the input\output\eq panel when the wild control display mode is selected.

### Metering System support

Old meter types are now fully supported including trimod displays on Alpha consoles.  These use the same allocation system in options->meters on the front end as the new TFT meters.

## 1.22

### Metering System

Introduction of TFT meter panels.  All meters have to be allocated at runtime.  No defaults are provided.  The allocations are saved in options.

The metering allocation screen has been moved from setup to OPT->METER on the FE.   Setup is used specify the position of the meter panels, (both TFT and old style Meter panels).  The FE is then used to configure the layout of the TFT meter panels and allocation to all meters.

Trimod labels for non-TFT meters are not supported in 1.22.

### Nexus Router support

New FE screen at OPT->SERIAL to allocate labels to input ports on a Nexus Router.  These will replace the input port labels on the faders when the router is online.

### Input 1/Input 2 Gain Linking

The ability to link the input 1 and input 2 gains by pressing the inp1 and inp2 buttons on the Input Panel simultaneously.  This behaviour is turned on by using the screen USER->STATE on the FE.

### Performance Improvements

Memory load times decreased.

DSP link usage optimized.

### Config Application & Programming Utility

The Config Application and Programming Utility have been redesigned.  The  functionality has not changed.

## 1.19

### Vca Groups as Slaves of Another

An extra layer has been added to the VCA group system so that a VCA master can be added as a slave to another VCA master.  A setting to disable the VCA edit function has been added to the front end to prevent accidental group creation.

A Front end screen is provided in State to Enable/Disable this feature as specified in the VCA Group As Slave specification document:..\..\..\Feature Specifications\V1_19 NBC Olympics release\VCA_groups_as_slaves.doc

### Partial memories

A Partial memory feature has been added which allows the user to select which audio paths and which parts of those audio paths are to be saved in a partial memory.  This allows the user to load settings only on part of the console.

A front End screen is provided in Mem/Part Mem, for user interaction with this feature: The screen specification can be found in the Partial memory specification document, paragraph 9 :..\..\..\Feature Specifications\V1_19 NBC Olympics release\Partial memories.doc

### Cross fades from tallies

This feature allows automatic fade in and out of any channel or group under control of a GPI opto input.  See : ..\..\..\Feature Specifications\V1_19 NBC Olympics release\Cross-fades from Tallies.doc

### MADI

Added support for improved MADI hardware using OD5362-2 and JM5407.

Removed support for obsolete OD5217 and JM5222.

## 1.18D

The Front End is not able to label ports on Remote IO Boxes if they are being used as an expansion rack.  For this feature to be available the checkbox in config must be check to say the FE is not on the NE.  The port labels can be edited in the NET->Conf screen in the LHS table.

### MADI

IOCONTRL.BIN updated to support using OD5362-2 and JM5407 MADI Version1.01 firmware to allow console to be informed if the JM5407 box has been reset in order for gains to be replayed .

## 1.18

### On/Off control from Wilds

Limited to Aux Sends and Front Pan.

This feature is enabled / disabled from a button found on the OPT / MISC Screen, and the modes of this button is saved in the Options. When enabled any Aux Send or Front Pan that is assigned to a Wild control can now be toggled On/Off (In/Out) by pressing the Wild Control Knob.

### New Panels – IU5219 Channel fader, IM5415 Main fader, IA5392 Assigned fader

These three panels have been incorporated into the Alpha Code, they now have the Surround Leds and the 7 tri-colour led metering instead of B/Gs.

NOTE: Old and new fader panels will not operate correctly when plugged into the same Hub.

### Linked Input Gains

Linking of Input 1 & 2 Gains on channels, when linked both gains are adjusted at the same time. To link hold down one input selection and at the same time press the other. This can also be done from the Channel Control panel if the channel buttons mode is set to be Input1 Input2 on separate buttons (will not work if selected on just the one button).

### Separate Wilds for Input 1 and 2 Gains

Input 1 Gain and Input 2 Gain can now be assigned separately to wilds, in addition to what is already available. This is done by selecting wild mode as normal then holding down the input button required while pressing the wild gain shaft.

### Non Switcher Network

When the network does not have a Switcher, the Bulk card is connected to an individual Hydra and Remote Box. The allocation is set from the Front End - Net Screen.

## 1.17

### Hydra Audio Networking Stage 2

- Stage  1 networking features with fixed routes has been removed.

Stage 2 networking has been added allowing dynamic routing of signals to and from remote input output boxes (RIOB).

The configuration of the RIOBs can be done from the network editor integrated into the PC front end.

- The Un4806 and Un4873 required updated firmware and kernel software.

Input routing from RIOBs to Opto functions, Relay Functions, direct input to output routing, Mic open bus and external IO (cue director) are not supported.

- Dynamic networking from desk to desk is not yet released.

- New screens on Front End, accessible via NET button.

## 1.16A

### MADI support added

Wide area bulk cards can be used in the digital rack (physical card slots 15,  17 & 19) to provide connection via the SCSI connectors to a MADI interface box.

### Joystick support added

### New PC user interface

Faster, easier to use PC user interface.

### New PC Front End Application

Faster startup time, sub 2 minute.

Faster memory load time, same speed as desk.

Multiple patching on IO screens.

New colour screen.

Replacement of drop down boxes with buttons which are easier to use on a touch screen.

Bird Beater available on all Auxes.

### Input 1, Input 2 Switching

Option added to now allow input 1,2 Switching on just one button on the wild control section of the fader panel.

Led off 	– Input 1

Led on 	– Input 2

### Preview Memory

Memories can be previewed onto the channel strips, and non assignable panels

When a memory is selected and the Preview button is pressed the selected memories settings will be displayed on the console surface. Any assignable panels will be blanked.

Upon release of the preview button the surface reverts back to the live memory.

Preview mode takes into account any isolations on faders.

All A, All B (LHS, RHS) buttons can be selected when in preview mode.

### Exec Led

MY4883 – Exec led should now light. – BID 683 (only applicable if this Alpha panel is used as a spare for Alpha)

### Studio LS Monitor

New option to choose monitor buttons in the Studio LS Bank  to follow Selector 1 or Selector 2.

### Pfl

Pfl has been stopped from coming out of two Loudspeakers at the same time.

If Pfl to Mon or if CRLS following Sel2 and Sel2 on PFL – PFL only comes out of the CRLS should not be heard on the Pfl LS or Small LS. – BID 676

### Allocated Metering System and External Meter Inputs

The metering assignments are now controlled by the setup application.  The following features are provided:

- Each meter slot (64 pairs) can be fed from any signal source selected from a list.

- Each pair of meters has individual meter response selector (PPM/VU, BG/MC, M/S etc.) to cater for customer’s individual requirements.

- The last 3 slots (6 stereo legs) are assigned permanently as phase meters.  To enable these it is necessary to set the meter bargraph module DIP switch to the value as given in the setup application.

- Each meter slot may also be assigned as an external meter input.  Patching of external inputs is achieved via a new page in the Front End application in the Options Screen.  The patches are saved and restored along with all the existing run-time options.

Note: the setup conversion will configure the metering system to mimic the fixed metering set-up provided in previous versions of the software, however, some adjustments may be necessary.  These are detailed below :

The meter slots 122..127 (Meter DIP settings 0x7A, 0x7C and 0x7E) previously occupied by Main 4 L/R pre, Mix Minus, ANC1 Lo/Ro are now fixed phase meter slots.  If any of these sources are metered on a desk, then they will have to be moved to a new meter slot (i.e. new DIP switch address) using the setup application.  Choose meter slots carrying signals which are not currently metered on the desk.  Refer to the meter slot spreadsheet for detailed information.

It is necessary to check all meters following the software upgrade to ensure that a) the response (VU/PPM etc) and the signal feeding the meter is present and correct.

## 1.12d

### Bird Beater

The bird beater option for Aux1 can be enabled using the Front End, User, Busses screen.

When this is enabled an aux pre-fader send will only be on when the channel is cut or the fader is closed.

### Monitor Outputs

Pfl override to headphones has been enabled.

Studio headphones output enabled (no level control).

### Mix minus Direct Input

- This input (at line level) is permanently enabled while an input is routed.

### Reverse Interrogate Routing

While interrogating mains, groups and tracks, routing can be applied by pressing the appropriate fader assign button for fast setup of many channels to one destination.

### Audio package configurations

Eight audio packages have been implemented offering varying numbers of channels from 36 to the upper limit of 72.  The maximum number of channels for the selected pack will always be available no matter how many are selected, only the mono/stereo divide can be moved.  The pack is selected in the config application.  Existing customers should retain the number of stereo channels (for memory compatibility) but may benefit from an increase in mono channels dependent on the number of DSP cards fitted to their system (this governs which pack must be selected during the upgrade).

### New PC screens

- Copy screen

Oscillator screen

PFL to mon & PFL to headphones on Options screen

Track post fader send follows channel/group L-R pan/balance and mono option for stereo channel sends all on Options screen.

### Dynamics

Gate on channel & groups.

Soft ratio on channel & group expander.

Auto recovery on channel, group and main compressors and channel and group expander/gate.

### Monitor panel

Support for new monitor panel (MY6048) in the setup application and on the Options screen.

### I/O Matrix panel

Support for new panel (MY6046) that allows channels/groups to be allocated to the control surface and input/output ports assigned.

### Group master fader panel

- Support for short fader panel (IU5284) without routing LEDs or wild controls.

# Issues Resolved

## 2.8A

2751 - Isolating Groups not working properly on memory load

2752 - Divergence/Convergence settings not copied between channels when copying pan settings.

2754 - Insert return patching not displayed in Offline Editor

## 2.8

BID2740 - The "lock screen" does not display EQ/Dyn buttons correctly when you first lock the screen.

BID2739 - On FE boot up, the Eq pages show assigned fader info when set to PC Fader

BID2738 - Eq Dyn page and routing panel page refreshing problems

BID2736 - FE not booting when no Local I/O in config

## 2.6F

BID2701 - Mono Tone Output was really fed from Surround Tone L output or followed Ext Surr L Tone input

Compressor Attack - default changed to 30ms So that Sigma and Omega’s “slow” setting is 30ms

BID2712 - "opto Dims CRLS" not cleared if "(Studio) TB Dims CRLS” is toggled whilst "opto Dims CRLS" is on.

## 2.6E

Created release of cumulative bug fixes provided previously as patches to 2.6D to avoid confusion for service and support and distributors.

## 2.6D

BID2680 OE - unbundling to the same folder will warn of existing data but then delete ALL files in that folder (very serious if the unbundling was done to c:\).

BID2633, BID 2634 The fix for these bids(2.6C) has been re-done to allow labels etc. to still update.

## 2.6C

BID2633, BID 2634 When a Monitor.ini was loaded the external monitor inputs would reset the gain on analogue inputs.

BID2459 Routing leds not blanking on B layer if Unassigned.

Logging improved for important messages from programming utility output to fix timestamp accuracy.

## 2.6B

BID2638 When a serial port function's hub is changed the old hub now stops processing the protocol before the new one starts.

BID2652 Network routing fault occurred when a left input is already patched to a mono channel and then the left and right of that input is then patched to a stereo channel.

BID2603 There is no opportunity to save the Tech mode passwords unless you go back to user mode. i.e exiting the FE in tech mode means you lose any tech-users you have entered.

BID2609 FE - Outputs Patching - Mon/TB and OSC - can patch from "Port Conn" columns - can give ownership with nothing showing as patched.

## 2.6A

Hub updated to track Ross fader level requests whilst the level is below the fader open level.

Ross stopped sending the board configuration request on a regular basis so Calrec turn on the unsolicited messages as soon as any valid message is received from Ross.

BID2600 Insert returns can be patched.

BID2614 Ross fader levels are transmitted after a Ross enabled memory is loaded onto a console with Ross disabled or after a secondary take over.

BID2612 When a mono is patched to the right leg of an input the second character of the input label is no longer corrupted.

## 2.6

BID 2563 Label for SDI box needs to be more generic to cover different types of boxes.

BID 2537 on FE screen OPT->Misc, Wild Control Push Switch has two options which can be enabled separately but one overrides the other - only one is really active

BID 2484 Memory Conversion - main spill and group spill paths

BID2538 and BID2238 ensure Sigma panels using multiple state memories update correctly.

BID2553 Down mis fader LCD displays improved.

BID2554 Omega displays the delay for the assigned path not always the first path.

## 2.5A

After memory upgrade –

BID 2484

Changing mains to surround left C, LFE and LsRs spill legs silent until master adjusted.

Groups had audio out of all their spill legs even though the master was faded out fixed when master was adjusted.

BID 2486 Assigning new surround channels showed audio on spills even though master was faded out fixed when master adjusted.

Both these BIDs are fixed with a new control.bin and a babbage.jar (for the memories)

BID2478 APFL is updated when viewed auxiliary is changed in master mode.

BID2400 When a surround channel is removed from a fader the LFE spill filter in is cleared.

BID2520 On Omega when front pan is in, rear pan follows front pan.

BID 2512 Fix for desk tone out of PFL LS after a desk reboot.

## 2.5

BID 2463 Mains not implemented for PFL overpress fader assignment and associated preview memory problem

BID 2428 Warning Messages were being issued for valid Buss IDs of NOT AVAILABLE.

BID 2418 HF Frequency around the 10k mark was adjusting differently going up and down.

BID 2436 Main Fader touch was not updating till fader moved

BID 2449 Group Master Assign Leds were flashing in Aux interrogate mode when group was not routed to an Aux.

BID 2411 Sigma Routing Matrix panel RY5586 was not displaying the top half of its fader input display.

## 2.4A

BID 2494 AFL not working from VCA master when there is no path under it

BID 2475 SDI Pre Decoder Input patches were being lost on a memory load.

BID 2480 Dynamics on Mains and Groups was not active when width of changed until a snapshot had been forced.

BID 2495 Divergence "in" status cancelled when upgrading from 2.2G to 2.4

BID 2491 Old memories with illegal chars in the description/label can be restored but not then backed up.

## 2.4

Network messages sent to WAB card could be overwritten when high volume of messages occurs.

BID 1986 Blip heard on PFL when PFL listen level is low and monitor level is high

BID 2122 Splats on monitoring when changing from an internal buss to an external source

BID 2298 Networked direct inputs are disconnected when the group width is changed.

BID 2300 PSU fail and AWACS relay state updates mode on the Front End GPO screen.

Omega rack no longer stalls when the fast link receive over runs.

BID 2364 Expander is available on groups and mains.

BID 2376 IP Address not clearing in Config App

BID 2389 Stereo led on A layer of 3rd fader (alpha only) was being turned off if the B layer was unassigned.

BID 2403 Main Insert Send Delay was not being cleared when the mains width changed.

BID 2435 Bluefin 2.3/2.5- Move Strip button on states page not activating

BID 2358 Omega wilds were not displaying ‘Auto’ when overridden by the Auto Button.

BID 2441 IU4722 B layer assign buttons light

BID 2442 Fader null up and down LED’s work correctly in reverse fader mode.

BID 2458 Main to Main Routing (surround) was not working.

BID 2233 Rear AFL led no longer lights unexpectedly for VCA masters without a path.

BID 2241 Front end doe not clear output port lists when an empty memory is loaded.

BID 2131 Auto-fader 4 no longer assigns itself to fader 1 path 0.

BID 2278 Ensure input output panel always show correct layer and colour.

BID 2378 VCA masters without a path can be cut.

BID 2434 Isolate page updates correctly.

BID 2354 Auto-fader null lights show an assignable fader.

## 2.3D

BID1986 & BID2122: Added v fades to monitor route switching to prevent audio blips when switching between sources at wildly different levels.

BID1960 Fixed external meter outputs at wrong level.

## 2.3C

BID 2402 Omega only: Inputs without a source patched had audio from 19-11R on them.

BID 2391 Main expander buttons on dynamics screen are greyed out

BID 2392 when shutting down Omega front-end a dialog box pops up displaying Zeta text

## 2.3B

BID 2300 PSU fail and AWACS relay state updates mode on the Front End GPO screen.

BID 2315 config.jar cannot locate config.ini in Cust1 folder

BID 2257 Omega rack no longer stalls when the fast link receive over runs.

BID 2364 Dynamics should not be restricted to channels any more (Omega only bug)

BID 2364 Expander is available on groups and mains.

## 2.3A

BID2235 Divergence buttons on joystick panel repeated too rapidly.

BID2237 Groups (stereo or mono) to track Pan not working.

BID2239 Groups to Track Pan not working from a wild

## 2.3

PC backs up config to the backup drive

BID 2207 Niplut not sending everything needed if boxes removed.

BID 2165 Gain interpolators not switching off when they should.

BID 2170 LFE Filter on mains has 20Hz HPF in addition to the 80Hz LPF.  This has been temporarily dealt with by forcing all channel, group and main 20Hz HPF and 20KHz LPF filters to have a flat response.  The proper solution will be provided in v2.4.

BID 1804 Phase meter display problem when loading meter configs

BID2047 When a ch/gp is cleared from the surface its partial memory settings remained.

Causing the ch/gp to still save in any subsequent partial memories in their default state.

BID2046 Audio O/P (1-6) Label was displaying pfl override when it should not.

BID2053 To make the meter label more informative swapped labels:-

Follow CRLS to CRLS Follow

Follow LS Sel to LS Sel Follow

BID2059 Partial memory load of group paths was ignoring the isolates.

BID2054 Delay Interrogate mode can be left stuck on. So - If in delay interrogate mode don’t allow selection change.

BID2070 Group phase meters display incorrectly when group is not set as stereo

BID2081 Meter Assignments were not seeing the apfl override when following the CRLS.

BID2062 Console clear now clears vca cuts and levels on spill paths.

BID2120 Channel clear on a spill no longer clears a VCA primary or secondary master cut or level.

BID2124 VCA cuts and levels are maintained on spill legs when the group width is changed.

BID2037 Options saved with PSU fail relay function as “PSU fail LED” are restored correctly.

BID2163 Input Output screen - does not when changing assign from Group master to a Group spill.

BID2061 Surround fader labels not being cleared when surround channels are cleared.

BID 2049 Stopped spill faders jumping when surround assign button selected (only happened on surrounds assigned using FE start/end selection)

BID 2084 (part 2) Delay display was changing after a while if shaft pressed and turned at the same time.

BID2199 To reduce confusing message when memories have not been erased before an upgrade wrong memory headers for cleared memories are not reported in the console debug.

## 2.2E

Rack stalling fixes merged in from Classic.

BID 2112 Debug assert fired during hot-takeover relating to main fader assignments

BID 2104 Talkback pre-selectors were not lighting other panels buttons during selection.

Fixed bug in selectable meters following external monitor inputs for mono inputs.

BID 2095 Monitor:- 	External sources input gain needs removing from output if afl/pfl 					override.

Afl Pfl levels should not be seen by Ls Mon Ins or CRLS pre.

BID 2126 Pfl override wins insert override if pfl to small ls enabled.
BID 2118 Listening to a stereo signal using MONO alt listening mode produced silence if phantom centre not used.
BID 2123 Surround tone incorrectly used for tone inject to stereo main buss output.

BID 1976 Ancillary meters not showing correct levels when monitoring external inputs.

## 2.2D

BID 2103 Input 1/2 switching could be a little quicker/smoother (NBC report)

BID 2102 Memory issues CalrecDLL.dll upgrade

BID 2101 When working with sets on Port Order page, text has a tendency to turn white making it  very difficult to read

BID 2099 Desk sets OHIO 12/4 card inputs to LINE instead of MIC input

## 2.2C

BID2085 Group dynamics meter stopped working when group was not on control surface.

BID1342/1674 - M/C VU and PPM meter scale/ballistic problems (M/C meters require recalibration, VU’s require removal of 3k9 resistor dcc6602).

## 2.2B

BID 2078 Stereo or Mono Groups when removed and reassigned from console surface were losing the ability to route.

BID 2052 Stereo or Mono Groups when routed to a stereo aux the aux pan did not work.

## 2.2A

Delay nudge button does not nudge on release.

BID 2065 Delay resources became corrupted during memory loading if input delay was allocated to an input and a channel’s input selector was changed to the alternate input before saving.

BID 2066 prevent front end channel to track level control flicker on a surround group in stereo mode by never sending the spill path routing state message.

BID 2072 Front End input panel LFE nudge buttons no-longer change the 5.0 level when 5.0 level is selected on the desk panel.

BID 2055 Delay marker gets upset when changing time formats

BID 2058 Mains buttons on delay screen

BID 2051 - WAB timeslots not updating on Net->Status screen

BID 2050 Track display not working correctly on surround master

BID 2057 pc fader follows desk A B selection on routing screen

BID 2069 FE Aux screen style nudge (inc/dec) buttons flicker

## 2.2

BID 2015 Fader meters could stop working after a memory load.

BID2014 and BID1267 Main insert return gain is restored correctly from memories. There is only one main insert return gain for all six legs the last leg to be updated determines the gain, do not mix port types, legs with no port patched do not update the gain.

BID2020 Prevent fader assign buttons from locking up if a control secondary take over occurs during VCA interrogate. Copies or moves are also cancelled after a secondary take over.

Fixed an issue that prevented stereo direct output meters from working correctly.

BID 2000, 2013, 2022, 2023 Delay on wilds not working correctly.

BID 2036 Fader Input Label on TFT kept losing the LR.

BID 2039 Network Direct outputs were being isolated, and leaving audio behind – not allowed networked outputs cannot be isolated.

BID 2040 Isolating one direct output was isolating multiple direct outputs.

BID 2041 Clearing a channel did not clear its networked direct outputs.

BID 2045 Debug asserts were being called when partial memories loaded

## 2.1B

BID 2015 Fader meters could stop working after a memory load.

BID 2016 Fader meters flicker with data from previous path.

BID 2017 Direct output meters on paths 106 and above do not show the correct meter data.

## 2.01

BID1839 - Delay nudge control sticks on if fader assignment changed during nudge.

BID1850 - Madi types on all spill path's stopped the surround master gain from working

BID1915 – Main Paths - When compressor recovery control turned to auto, fast attack led lights, but does not light if try to select.

## 2.00E

BID1851 fix – Blip heard on monitoring when switching between desk surround and desk mono.

BID1772 fix - Alt listen mode mono was -6dB too low.

BID 1897 - Memory Conversion - converting inserts on groups 5-8

BID 1901 – Memory Conversion – no audio on Groups 1-8

## 2.00d

Font alignment across all projects and panels for the new delay characters require some panel software changes to match.

Some shaft press messages on EQ panel missing from panel.

Hub firmware changes to allow reliable access in any position across the backplane.

Aux ‘on’ LEDs did not always update when in mono mode.

System synchronisation from AES inputs did not work if more than one input from cards in the range 10 – 14 had been selected.

Fixed fault whereby group dynamics meters do not update if group faders are not shown on the current fader layer.

## 2.00c

Main and group noise occasionally found after powering up DSP card fixed by implementing a new width control.

## 2.00b

Allowed extra time for DSP card reset and re-enabled some debugging to track DSP card problems.

New DSP card interface firmware to fix false hot swaps (error writing to card test latch).

## 2.00a

BID1830 Secondary Console Proc has difficulty communicating with UN5221's at the far left of console rack.

Layer view options not as customer required so buttons have been hidden on PC FE.

## 1.29

BID1664 front end pop-up dialog for too many output meters updated to include external input meters.

BID1665 5.0 level is copied in the pan segment.

BID1668 TFT channel input meters are updated if On Air or Rehearse mode causes the tone to be disabled.

BID1675 update tx\reh after load but before snapshots.

BID1676 prevent debug assert when program is loaded.

BID 1343 The 16 way bar graph meter representation on the FE does not look like the 16 way bar graph meter

BID1642 Eq flat button can be selected on group path

BID 1735 - Down-mix levels inconsistent

BID 1743 When small ls c/o selected crls ass 1(pre sel 1) was overriding the selection of crls ass 2 (pre sel 2)

BID 1746 Changing monitor selector buttons from External by sending new monitor selections, loading options or changing main lines (from setup) was not fully clearing the networked patch.

BID 1760 When changing to surround groups the dynamics settings only got applied to the LR spill leg, should go to any leg that is not independent.

BID 1770 Dynamics settings were not changing from group spills when changed from surround.

## 1.28B

BID1661 GPO patches not being shown on Zeta

BID1660 Memory upgrade problem. Input Output patching not being upgraded.

BID1659 Memory upgrade problem.  The problem is the assigned fader layer B does not

BID1658 Only either the main or sub main meter value is displayed on the Sigma assignable fader meter not both which caused flickering meter displays.

BID1657 If aux width is changed whilst displaying direct inputs the aux display now updates with the correct display.

BID1656 When the Sigma assignable fader is locked to a main fader, moving the assignable fader only moves the main fader if it is showing the locked layer.

BID1655 Aux screens (on Sigma FE) different to from control surface

## 1.28A

Setup crashed if the PC did not have a network Interface

Problem in old memories which had corrupted the Output Lock State Memory fixed in memory conversion

BID1643 Use the correct function when assigning channel delay in PAL frame display mode.

BID1635 Turning level shafts slowly from	-9,9 dB to -10.0dB does not jump back to 9.8dB

Control debug command pclvl added.

BID 1650 (from V1-17H)

Some input gains were being reset to zero or changed when port ownership grabbed.

Added checks and constraints against invalid input settings from a RIOB.

## 1.28

BID1566 Joystick panel does panel does not show strange LED’s and LCD displays when a fader with no path is repeatedly pressed. Joystick cannot be locked to an unassigned path.

BID1572 VCA slave fader on LED always lights when the VCA master is faded up slowly.

BID1574 Alt listen modes on Small LS were not selecting correctly

BID1576 Joystick fader always displayed on bottom row of LCD.

BID 1580 Filexfer system needed to clear when Front End Starts.

BID 1287 - Even mono port phantom power does not work on TB inputs.

BID 1601 - Setting stereo channels to 0 in config causes problems reading run-time options

BID 1608 - Stereo insert gives just Left to both sides when used on Groups 1 to 4

BID 1597, 1602, 1604, 1605 AWACS relay LED function corrections

BID1609 LB RB on surround groups in stereo mode now works

BID 1622 Networked Direct Outputs not clearing when group width was changed and Networked Group Direct Outputs (surround capable ones only) losing audio if network lost and reconnected.

## 1.27

BID 1557

## 1.26E

BID 1559 Spill independence screen showing ON buttons reversed

BID1555 if the assigned fader is a spill fader the first time the front end start up the LR C LFE or LsRs buttons on the PC screen fader panel are lit appropriately.

BID 1556 Initialise group LR spills Aux send levels to Line up.

Updated Hub startup.asm to access dpram semaphores correctly (hub software update fault).

## 1.26D

BID1553 IA5530 locked button always lights red.

## 1.26C

The On button of the IO Matrix panel will not be lit and has been disabled when the first direct output port is a network port not in a visible list (LCD shows “N’Work” on the bottom row of its LCD), to prevent the processor locking up.

## 1.26B

BID1537 Selecting an Insert Position on the Input Output Panel was turning the display of the group insert off on the front end.

BID1544 Groups allocated as surround but set to stereo should not display master on the wild direct input gain displays.

BID 1541 Monster channels not working in 1.26A

BID 1545 Main insert sends out of step

Partial memories now supported.

## 1.26

BID1390 AFL’s were remaining illuminated after AFL clear of VCA master.

BID 1403 Remote (opto) cuts were being left on faders after the channel had been cleared and then re-assigned.

BID 1392 Pfl to Small Ls (with pfl not to mon option) did not work after erase hidden.

## 1.25

BID 1403 Remote (opto) cuts were being left on faders after the channel had been cleared and then re-assigned.

BID1538 Losing a leg of a networked insert on a channel when trying to parallel with a main insert.

BID1539 Partial memory load of a networked input lost audio.

BID 1525 Tone needs clearing when input is cleared.

BID1548/BID162 Controls in left hand hubs do not change audio smoothly when secondary processor is running.  Backplane timing adjusted to fix.

## 1.24

Memory reductions of the equaliser and dynamic state memories has been done in this release as a maintenance issue.

BID1314 TFT Main, Anc1, Anc2 Meter labels were not refreshing after a hot takeover or a (erase options) load options. (refreshed with a mon sel button press so was not critical) BID1319 TFT Main, Anc1, Anc2 Meter labels were not refreshing when first set up from the Front End. (refreshed with a mon sel button press so was not critical)

BID 1377 Direct Output screen not updating correctly when path with multiple patches is moved in a memory load

BID 1412 Fixed Opto dim not working.

BID 1422 Fixed Local Input to Networked Output patching not working.

1.23C

BID1321 FE not upgrading 1.16 memories correctly

BID1322 Setup not dealing with Anc1 correctly, being set to Surround only

Memory reductions of the equaliser and dynamic state memories has been done in this release as a maintenance issue.

## 1.23B

Hub’s Uuart receive buffer improved preventing overflow messages.

File transfer to PC now implements a retry mechanism on block transmit failure.

javaApps

BID 1302 FE flooding core with messages when labels assigned on Serial Int screens

Console

BID 1296 - Intialised audio and meter output to fix lack of audio through monitor after erase hidden and before a mon sel button press.

BID 1309 Main and anc meters following a sel button(18) after an erase hidden

BID 1311 Added function calls to refresh meters after console clear

BID1313 PC does not report the 'out of delay resources' message.

## 1.23A

javaApps

BID1246 Channel input meters need removing from LED bargraphs

BID1263 When a memory is upgraded from V19 to V1.22 the mic open busses are not converted properly

Control

BID 1255  Now updates Crls pre meter with apfl overrides.

BID 1256  New monitor assign panel was not updating apfl clear led correctly – It was dependant on which hub it was plugged in to. Can now go in any hub.

BID 1260  Fixed TB SRC/phantom power not switching for networked inputs and TB SRC switching other TB ports SRC as well as the selected port.

BID 1262  Stopped apfl settings disappearing on a hot-takeover.

Zeta requirement. Added Comms for Ls Mon Ins In button to the FE - for old panels only

BID 1277  Sel1 and Sel2 function ids to studio ls on old mon panels caused a debug assert. This bug did not affect the actual selection.

BID 1259 Incorrect levels occur for a brief time on Sigma inputs with delay assigned following an input type (AES/MIC etc.) switch.

BID 1257 CRLS PRE output splats when crls insert is switched in.

BID 1292 Tone (mono) crackles when routed to WAB to wall box.

## 1.23

BID 1001 - Tone was not loading correctly with port isolates.

BID 1018 - Mic live was not clearing when the desk had been reset and the settings had not been saved in options.

BID 1029,1051 – Alt wild on CC was not lighting when alt wild selected from functions panel

BID 1013 – Vca masters with No path were not moving to correct position when reverse fader enable was selected.

BID 1022 - Unassigned paths under masters with wild channel fader were not updating with an All A or B press.

BID 1020 - Pfl LS now mutes if there is pfl on the CRLS (via selection or override).

BID 857,220  - TB/RTB had audio bursts even though pot set to off.

BID 1048 – When fader path cleared delay resources are removed and the resources used is updated

BID 1243 – Channel and console clear do not un-assign delay resource just set the delay to minimum and set delay out

BID 1264 – Networked external monitor inputs were losing audio upon reset.

BID 1278 – Old Mon Panels apfl o/rides not refreshing tft’s following crls

BID 1280 – Incorrect debug assert firing

## 1.19

BID 898,919,909,929

## 1.18D

BID964 old data left on channel control displays after a memory load.

BID965 Wild Channel Fader display refreshes now when copying.

## 1.18B

Meter labels added for second ESPN console and Game Creek console.

JD4915 2.9a code released to fix impedance relay fault (independent of console software).

## 1.18A

MADI interface completed.

1.18

BID899 - Netlists now sent to dsp rack for Hot_takeover.

BID 895

Metering can get out of step in high load situations fixed.

BID 927

Talkback input gain corrupt @ -97cB

BID 884 - Pfl to desk headphones is not now muted when pfl to mon selected

## 1.17B

- BID820

Label on IO Matrix panel now displays the correct Port label for network patches.

- BID834

Memory Loads, resets etc. no longer send disconnects then connects for networked input timeslots still in use.

- Patrick did something to the rack

- FE: Drain the serial connection before sending another packet – stop overflowing problem

## 1.17A

- BID 782

When a remote mic (RMW's) box was reset the console debugging reported the error "Wab index not within bulk card ranges 18" has been corrected.

- BID789

Misleading error messages in the rack when a networked input was disconnected have been removed

- BID801

- Networked outputs can now be patched to insert sends.

- BID802

- Aux direct inputs connected to networked MIC inputs now apply correct sharc gain.

- BID803

Network outputs which are no longer patched after a memory load have the buss labels removed.

- BID804

- Matrix panel lists corrected.

- BID805

- When networked inputs timeslots are reallocated the ownership is now reclaimed.

- BID807

- External monitor networked inputs now initialise with the sample rate converter on.

- BID808

- Options now ownership padlocks update after options load

- BID809

- Networked inputs connected to talkback now update the options are loaded

## 1.17

My6006 SLS button leds light.

## 1.16B

Routing from S2 and S1 to M2 and M1 is possible.

## 1.16a

BID 714 – Studio Ls Outputs now cut separately. Mic Buss 1 cuts SLS1 Mic Buss 2 cuts SLS2.

BID 737 – Reverse Faders are now remembered in the Hot_swap of the Core Processor.

BID 748 – Preview memory speeded up.

## 1.16

BID 682

Control s/w now has test code that will examine and correct all assignment errors.  Invoked from programming utility by typing ‘TEST’.

BID 708 – Direct outputs sometimes losing audio. Caused by directs re-assigning themselves and not clearing the previous assignment correctly (not visible to user, and not consistently – hence sometimes)

All memories need fixing with the above programming utility ‘TEST’ and resaving, this will recalculate the direct outputs and the fix will stop them getting out of step again

BID 683 – See new features.

BID 676 – See new features.

BID 660 – Fader ‘on’ leds were not copying everytime.

BID 691 – After memory load sometimes Mains or Sub Mains assignment leds were being left illuminated.

BID 702 – Clearing paths now updates the free path count on the Front End.

BID 692 – While holding down a Fader Assign Button and assigning a path(mono,stereo or group) at the same time it is no longer possible to create a VCA Master with only the new path underneath itself as a Slave.

## 1.15b

BID 644 - Matrix panel in Direct Output mode now displays ‘IN USE’ for any output port patched not just Direct Output patches.

BID 656 - Paths cannot be cleared if their Direct Outputs are Locked, Groups cannot change state (mono, stereo) if their Direct Outputs are Locked

## 1.15a

Analogue gain did not operate when level control was increased and jumped to correct level when turned back.

- BID 689 – Sluggish response of EQ controls fixed.

## 1.12d

Support for UD5225 DSP cards added – this allows UD5225 to be fitted as an alternative to the UD4796.  For this release to work on existing desks fitted with UD4796 DSP cards, the UD4796 DSP cards must be upgraded to firmware v1.13.

BID 644 - When the Port Display(Matrix Panel) is showing Direct O/P ports, the display now indicates “IN USE” for any Output Port already patched – not just Direct Outputs.

- BID 689 - EQ/Filter control response improved.

- BID 618, 643 - AES input freq. response problem fixed.

BID 694 – Monitor problem which causes outputs to be muted following audio pack re-configuration fixed.

BID 565 - When Input 1 (or 2) is ON on the Isolate screen, the port screen shows that the port is isolated.  It shouldn't be as there are now separate buttons  on the Isolate screen, for the Input ports.  Fixed.

BID 662 - When patching a source fader open to a relay, half the line inputs and all of the dig inputs source labels do not appear in the relay column, though the function actually works. The same input sources do not appear for channel cut opto assignments.  Fixed.

## 1.12c

Some fixes from the previous version have been removed because it was found that when a new AES source was plugged in, the output from the same device muted for a short time.  The original frequency response problem may now occur occasionally when an AES source is plugged in but this will be cleared on rack reset or power up.

## 1.12a

Correction to prevent loading of reverse fader options if not made available in configuration settings.

Reverse interrogate of mix-minus bus corrected. Previously once this had been used the routing to the mix-minus bus was always toggle when ever any reverse interrogate routing was changed even to other busses.

## 1.12

Improved locked fader performance.

Errors in the new memory panel save and load user interface have been corrected. Whilst in stack mode with load or save active, the ‘>‘ key to can no longer scroll beyond the stack (this prevents corruption of the display), pressing the ‘Remove’ key no longer loads or saves the selected memory unintentionally. Whilst not in stack mode pressing the ‘<‘ or ‘>‘ buttons does not unintentionally load the selected memory. Dialog boxes with memory label changes are not left on the front end screen.

- Front end reaction time after pressing ALL ALT when available has been improved.

- A VCA master cutting an aux. send is now recalled when a memory is loaded.

- Channel panel input 1 and 2 LED’s are not turned off during an interrogate.

Analogue input gain changes now have better digital / analogue gain step handovers due to timing changes within the bulk card software and changed control methods.  This requires new bulk card firmware (v1.6) and software (v2.5).  Without this, the input gain control will produce high input levels.

- VCA slow performance on fader and cut resolved.

VCA masters could be left without slaves if a slave fader was deleted from the ‘B’ layer.  This can no longer happen and existing faults with VCA masters can be corrected with a command line operation which re-evaluates all the VCA groups (type ‘vca’ at the command prompt of the programming utility (console tab)).

- Option save failed to work on the 5th to 8th save after a flashROM erase, this is now fixed.