Upgrading Hydra from pre-1.11 systems	2

Upgrading Hydra from 1.11 systems	2

Upgrading from Hydra 1.6a to 1.6b	2

# Upgrading Hydra from pre-1.11 systems

When upgrading hydra from systems that are pre-1.11, please upgrade the fixed format IO boxes first.

This is so that the upgrade process does not stall with an Error 4 condition. This is where the information is sent too quickly for the system to process and ends in an incomplete state. This condition is more prone to occur after the router speedup changes implemented as part of the 1.11 work. A fix in Hydra 1.6a will stop it from causing a problem, but until the system is upgraded to 1.6a, the act of upgrading may cause the problem. Therefore it makes sense to upgrade the IO before the console (which contains the 1.11 speed up)

# Upgrading Hydra from 1.11 systems

In this case, the act of upgrading IO boxes may cause an Error 4 condition. This is where the information is sent too quickly for the system to process and ends in an incomplete state. This condition is more prone to occur after the router speedup changes implemented as part of the 1.11 work. A fix in Hydra 1.6a will stop it from causing a problem, but until the system is upgraded to 1.6a, the act of upgrading may cause the problem.

If it does occur, just clear the entry from program updater and retry until successful. It may take a number of attempts. Do not reset the IO box until it has completed 100% as doing so may leave the box in an incomplete way. Once it has updated, power cycle the IO box by physically unplugging and replugging it. Program updater will only show the correct new version number after the IO box has been power cycled.

# Upgrading from Hydra 1.6a to 1.6b

For a desk that has had Hydra2 1.6a installed (will be showing 2 x H2LinkDriver in Prog Updater), to upgrade to 1.6b you need to select the LinkDriverMadi manually:
- click right on the Fixed format box heading in Prog Updater
- “Add Application”
- select the 2.12.0 LinkDriverMadi version from 1.6b
- click Download