# Active Antenna

# Dect Update Procedure

## Active Antenna DECT download/debug mode

To set an Active Antenna into DECT debug mode, open the case and put links on LK4 and LK2, as shown below. These are the same links set to upgrade or debug the DECT firmware.  These links should be removed for normal operation.

## Running the dect loader

Install the dect loader onto a PC with a serial port. Connect the PC to the BP using the BP data cable.

Start up the Flashloader program, set as follows with correct “Target Final Flash Executable” the Download should start automatically:---

Power up the BP and the DECT should start updating, the status will change from “idle” to “erasing flash”.

The ildr_425.hex can be found in the DECT images directory.

After the DECT is upgraded. Turn off the AA, disconnect the power and remove the jumpers from the LK2 and LK4.

Check that Jumper LK1 is shorted as this activates the watchdog.

Connect a terminal emulator (ie. Hyperterminal) set at 19200,8,N,1. Power up check that the SW versions are correct.