For a 576x1728 system which has

*1-576 bnc outputs

*577-1152 DD outputs

* 1153-1728 xlink outputs

All three xpt are 576x576.

The redundancy for top and bottom half are independent of each other.

When DD card is used in any slot redundancy for that slot is not allowed.

No redundancy for DD outputs

No redundancy for xlink outputs.

EQX-V4 Note: Define the output priority “Xpt Redundancy Output Priority” to indicate which output should take the shared path. For example outputs 1-288 and 577-865 share the middle xpt, so to support redundancy for 1-288, “Xpt Redundancy Output Priority” should be enabled for outputs 1-288. In order to support DD outputs 577-865, “Xpt Redundancy Output Priority” should be enabled for outputs 576-865 instead.