This is meant for a 26RU Dual Frame EQX-V4 System.

- Outputs 1-876 are fully redundant

- 877-1152 no redundancy

- 1153-1440 no redundancy but Main feed interruptible by 1-288 redundancy.

This system has 288 shared outputs. Frame-1 and Frame-2 Middle xpt is shared between Normal outputs 1-288 and DD outputs 1153-1440. It provides redundant feed to 1-288 outputs as well as Main feed for DD outputs 1153-1440. In other words outputs 1-288 share the path RI/REI with 1153-1440 .

Some points to understand the map

- MI: Main Feed to Normal outputs from SAME Frame.

- EI: Main Feed to Normal outputs from OTHER Frame.

- RI: Redundant Feed to Normal outputs from SAME Frame. And Main feed to DD outputs from SAME Frame.

- REI: Redundant Feed to Normal outputs from OTHER Frame. And Main feed to DD outputs from OTHER Frame.