This is meant for a 16RU XL2 frame. Redundancy is supported for outputs 1-288.

Top Xpt need not not be software map supported. Middle & bottom xpts needs to be software map supported.

The top xpt drives the top output cards, outputs 1-288.

The middle xpt drives xlinks numbered 865-1152 and provides redundant feed to outputs 1-288. The bottom xpt drives xlinks, numbered 289-576. The xlink outputs are NOT numbered linearly.

If the xlink connectors are numbered as follows

- 1-9 from bottom

- 10-18 from the middle

Then connector 1 pin 1 is 322 and so on, and connector 10 pin 1 is 913.