This map file is for 26RU Xlink Frame for 55x52x52 configuration.

All xpts needs to be software mapped.

The top xpt drives the top output cards, outputs 1-288. It also drives the xlinks numbered 1153-1440. The xlink outputs are numbered linearly. If the xlink connectors are numbered as follows

- 1-9 from Top

Then connector 1 pin 1 is 1153, connector 1 pin 2 is 1154 and so on.

The middle xpt provides redundant feed to outputs 1-288 and outputs 289-576. Redundancy is shared by the top & bottom half of the outputs, and priority can be defined in the winsetup if needed.

The bottom xpt drives the bottom output cards, outputs 289-576.