This is meant for a 16RU XL2 frame, with three 288x288 xpts. There is no redundancy at all.

The top xpt drives the top output cards without redundancy, outputs 1-288. Top xpt need not be software map supported. As the map is same as the default old system it should work.

The middle & bottom xpts needs to be software map supported. The middle xpt drives xlinks numbered 865-1152. The bottom xpt drives xlinks, numbered 289-576. The outputs are numbered linearly.

If the xlink connectors are numbered as follows

- 1-9 from bottom

- 10-18 from the middle

Then connector 1 pin 1 is 289 and so on, and connector 10 pin 1 is 865 and so on.