This is meant for a 16RU XL3 frame. There is no redundancy at all.

All xpts needs to be software map supported.

The top xpt drives the top output cards without redundancy, outputs 1-288. It also drives the xlinks numbered 1153-1440.

The middle xpt drives xlinks numbered 865-1152. The bottom xpt drives xlinks, numbered 289-576. The outputs are numbered linearly.

If the xlink connectors are numbered as follows

- 1-9 from bottom

- 10-18 from the middle

- 19-27 from top

Then connector 1 pin 1 is 289 and so on, and connector 10 pin 1 is 865 and so on and connector 19 pin 1 is 1153 and so on.