Brief test procedure for the 3410 Multivert

- Observe Power circuitry for obvious faults

- Plug in 1 supply.   Measure +5V at TP1 and –5V at anode of D16.  Repeat for the other supply.

- Place shunts in the OFF position of J47 and the OFF position of J46.

- Properly connect JTAG programmer cable to J3.   With power applied use the JTAG software to program CPLD U18 with file “bulken2.jed”

- Properly connect JTAG programmer cable to J1.   With power applied use the JTAG software to program CPLD U21 with file “bulkencd.jed”

- Power down the unit and properly connect the WINISP cable to J45 apply power to the WINISP cable then apply power to the 3410.  Be sure J46 has its shunt in the OFF position.  Follow the WINISP rules of programming the 89c51rc+ microcontroller with the software “be34a1.hex”

- Power down the unit.  Remove the WINISP cable and any JTAG cables. Place a shunt on the pedestal ON/OFF pins according to whether NTSC pedestal is required by the customer.

- Place J46’s shunt in the ACTIVE position and power the unit.  Observe the rapid “Christmas tree lights” on the front of the unit.  If you observe  red “runway lights” check that the Watch Dog shunt J46 has been properly set to the ACTIVE position.

- Observe supply LEDS function.  Check that all front panel NO SIGNAL LEDs illuminate.

- Apply SDI to the input of each channel.  Verify that the signal present LED on the back of the unit illuminates.  Observe the front panel LEDS change to indicate which standard (NTSC/PAL) has been applied.  Verify SDI output is functioning.

- Take 1 output of the channel to an oscilloscope.  Use Pot R? to set the level of the output to the analog standard.  Take the other output to the scope and verify that it is also set properly.

- Repeat steps 10 and 11 for all of the channels.

- Switch incoming SDI to PAL.  Repeat step 12.

- Prepare unit for “burn in”.  Place TEST J47’s shunt in the ON position.  Apply video to all the SDI inputs.  Terminate all the analog outputs.  Apply power to BOTH supplies.  Place as an enclosed unit in the burn in cabinet.  Any failed channels will show up on the front panel with its LEDs  flashing.

- After burn in remove unit reset the TEST J47 to the OFF position.  Double check that the Watch Dog jumper is set in the ACTIVE position.