8075 Software Upgrade

Installation Information

# UPGRADE KIT CHECKLIST

The enclosed items allow you to upgrade your 8075 units to the current firmware level.  We have checkmarked the items that we believe are required for your units.

Please check that you have received these items as shown below:

Req'd	Shipped	Rec'd

8025 Main Board Firmware 27C512 EPROM CK87D2 -990721

8038 Sub-Module Firmware 29F010 FLASH FX87D4 -981030

8025 Main Board Firmware 27C512 EPROM CK87D3 -990316

# Preparation

In order to perform this upgrade, the following tools will be required for a smooth installation:

- PLCC Puller

- 3/16 Hex Driver

- # 6 Phillips Screwdriver

- Small Screwdriver or EPROM Puller

# Disassembly of the 8074 for Prom Installation

Before proceeding with this installation make sure that Power is disconnected from the Unit.

?

Electronic devices are subject to damage by electrostatic charge build-up, which can occur with improper handling.  The devices should always be carried and stored in the anti-static carrier tube.

?

Required	Completed

Remove the 8 top cover screws on the unit. Remove the top cover and set aside. Turn the unit so that the rear panel is facing you.

Remove the 3/16 hex nuts from Port B and Port C DB9 connectors. NOTE: No other hex nuts need to be removed to complete this installation.

Turn the unit so that the front panel is now facing you. Remove the 4 Phillips screws which are holding down the 8038 module.  At the rear of the module, there is a parallel port ribbon cable connected to J2 (A 10-pin header). Carefully remove it and place the cable over the rear of the unit. Remove the short 4-conductor Power cable that connects the 8038 module to the 8039 module (J14 on the 8038 and J13 on the 8039).  The 8038 module may now be removed by carefully holding and lifting either side of the front edges. This will disconnect the 8038 from the 40-pin connector. A side to side rocking action will help facilitate the removal of this module. Set the module aside for the next operation.

# Upgrading The Firmware

Using a small screwdriver, carefully lift out the EPROM U21 being careful not to bend the pins.  Store the old EPROM in electrostatic conductive foam or tube.

Install the new EPROM, labelled CK87D2 or CK87D3, in the socket U21, so that the notched end is in the same orientation as the silk screening on the circuit card.

Using a PLCC puller, carefully lift out the FLASH U3 on the 8038 sub-module being careful not to damage the socket.  Store the old FLASH in electrostatic conductive foam or tube.

Install the new FLASH, labelled FX87D4, in the socket U3 of the 8038 sub-module, so that dot on the FLASH is in the same orientation as the Pin # 1 silk screening on the circuit card. This can also be confirmed as both the socket and the FLASH have one corner which is cut on a diagonal. Both the socket and FLASH will line up to this diagonal when inserted correctly.

Turn FASTBOOT mode on by closing dip switch #6 on SW1, on the 8025 board.

Make sure that all of the pins are inserted correctly into the socket.  Severe damage may result from an improperly installed integrated circuit.  Double-check the installation before proceeding.

?

# Reassembling The 8074

Reinstall the 8038 sub-module by placing the rear of the module under the Parallel port connector at the rear of the unit. Be sure that the mini co-axial cable is beneath the sub-module and will not be caught on the placement of the sub-module. Align the sub-module up using the screw holes as a guide. Once you are sure of correct alignment, press the front edge of the 8038 sub-module down onto the 40-pin connector. Reconnect the 4-conductor cable that was previously removed to J13 on the 8039 module and J14 on the 8038 sub-module. Reconnect the parallel port ribbon cable to J2. Install the 4 Phillips screws that secure the 8038 in place.

Install the 3/16 hex nuts from Port B and Port C DB9 connectors.

Replace the top cover and install the 8 top cover screws on the unit.

Power-up the unit and be sure that unit is operational. The upgrade is now complete.

NOTE: If you are upgrading more than one unit, simply repeat Sections 3, 4, and 5.