This release contains the following files for the

NEW STANDARD 8070 (center daughter board is 8033A)

RL8070.DOC - These release notes.

SOFTUPD.TXT - Text file describing how to subscribe to a automatic

service which will notify you of software updates to

this product.

DK87D1.HEX  - An ASCII Intel Hex file suitable for most PROM burners.

The PROM (a 27C512) belongs in the U21 the 8025 Digital

Keyer board (the main board).

FX87D1.HEX     - An ASCII Intel Hex file suitable for most PROM burners.

The FLASH PROM (a 29F010) belongs in the U5 socket on the

8033A RAM submodule (the center daughter

board).

All PROMs should checksum to the "DATA CHECKSUM" value shown in the comments

at the end of each .HEX file.

If your PROM burner calculates a 4 digit checksum, they should match

the least signifigant 4 digits of the checksum shown in the comments

at the end of each .HEX file.  The .HEX files define the values for

all locations in the PROM, so no pre-burn filling is required.  The

.HEX files are INTEL HEX compatible (ie. they are NOT binary format

files)

The HEX files contain comments at the end of the file

which indicate what size PROM should be used, and the

name and version of the software.  Please copy the NAME

and VERSION information onto the prom label.

(Some prom burners will complain about the comments but virtually

all prom burnerts will accept these files.