Router labeling utility: Overview

HyperTerminal

Configure serial port

The labeling utility allows the user to change the label of any of the video/audio inputs using a .ini style file loaded through HyperTerminal.

- Launch HyperTerminal. Connect a serial communication cable from the router to the PC host. Use settings of 38400 baud, 8 data bits, no parity, 1 stop bit and no flow control

- Note: Before transferring files, the GVG protocol should not be running

- From the menu, select “Upload file?” to send the file to the router. In HyperTerminal a prompt will appear with “Upload file now” and the file can then be sent. Selecting “Download file?” will send the file to HyperTerminal.

- The file structure is as follows:

[label_a]<CR><LF>

var_a1=value 1<CR><LF>

var_a3=value 2<CR><LF>

var_a2=value 3<CR><LF>

[label_b]<CR><LF>

var_b1=value<CR><LF>

var_b2=value<CR><LF>

var_b3=value<CR><LF>

<CR><LF>

<CR><LF>

Note: <CR><LF> = Enter key

Each label is associated with a set of variables. The variables must follow their associated label in the file. The variables represent items such as label names that can be changed in the router. The value which follows the equals sign is assigned to the variable. There is a 16 character maximum length for the value. Alphanumeric characters such as a-z, A-Z, 0-9 and punctuation are valid (byte values 32 to 126 are valid). The square brackets and equals sign may not be used in the label or variable since they delimit the file.

INPUT LABEL FILE

Upload file?

Router

HyperTerminal

Transfer

Send text file

Download file?

HyperTerminal

Wait for file to appear