GUIDE

What is RCP Soft

RCP SOFT(Grass Valey Software Product) is a Java Program that can be run on any O.S that will send command to a gateway in the Kaleido-X series.

The Kaleido-X series uses a web server installed on the card that listens permanently through a specific gateway for a user command.

The Command send to the Kaleido-X are the same command used in the XEDIT. These less extensive command give the user another way of using the Kaleido-X series.

Here is a view with all the different panel of the Java Program opened at once.

Visual Overview of the RCP Soft.

Steps to be consider

The only glitch that we have seen so far concern’s the Ethernet Adapter chosen by default.

Sometime to make the Java Program start the Communication Protocol you will have to switch from one network card to another in the drop down list of network card available.

You do this even if it is to come back to the first choice. This should “kick start” your session, we are looking for the message “Querying Rooms Please Wait…“

If there is only one card available you may not be able to kick start the Application.

First step :

The very first step when opening the RCP soft java Program is to configure the IP of the Kaleido-X.

| In the main window Push the Enter button for 10 second | You are looking for the message “Configuration UNICAST IPs” | Use the Arrow down from the Keyboard | This will bring you inside the IP configuration. You need the Kaleido Frame IP address. |
| --- | --- | --- | --- |
| Use the Keyboard Pad again to modify the number to get the right IP address. | Once you have the right ip address written… | Use the Escape button as many time needed to return to the main message. | You can do this for 2 other Kaleido-X system (The RCP accept 3 IP addresses) |

Before even login in you have to choose a room.

Choose a room

| Click the ROOM SELECT button | This should appear in the message window | Then the name of the different layout should be available |
| --- | --- | --- |
| Use the Keypad to travel between the available Layout from the KAleido-X |  |  |

Then you log in. In the Kaleido by default the username is Admin with no password.

Login in

In order to access the layouts of a room you will have to log in. Each time you change room you have to log in.

How to log in

| User Login | Show the Username, you can modify it if you need it | Enter | If no Password don’t write anything | Enter |
| --- | --- | --- | --- | --- |
| Access granted message means success |  |  |  |  |

The rest of the software is working like the RCP2 model.

PS:

The F1 to F10 key of the keyboard can be used to switch between available layouts.