P1K FLASH INSTRUCTIONS

- Connect BDM to Adapter Board.

- Connect Power Cord

- Turn Power On

- Execute the P1K App batch file on the desktop

- Let it run until it has completed flashing.

- Remove the BDM from the adapter board.

- Connect the special serial cable to the adapter board.

- Using Hyperterminal, set the connection properties to 19200 baud, no flow control, 8 data bits, and 1 stop bit. There may even be a P1K hyperterminal icon on the desktop.

- Turn the unit back on.

- Enter in the Serial Number and the Mac Address and press ‘Y’ to commit.

- At this point the unit should boot.

- The default IP address starts at 192.168.1.2xx. The xx is determined by the dial:

Dial #			2xx

  - 201

  - 203

  - 205

  - 207

  - 209

  - 211

  - 213

  - 215

To change the IP address, go to the Microsoft Command Prompt.

Type in “telnet 192.168.1.2xx”, where xx is the ip address from the above.

Then type “pesa –ip” at the prompt.

The prompt will ask you for the first octet, type 192, then 168 for the second, and then 2 for the third. The prompt will ask you if you want to commit, and type in “y”. Reboot the system, and now the unit will come up at 192.168.2.2xx.