PESA

Cheetah DRS Embedder

FPGA 10.11.11

Product Functional Definition and Validation

| Written By: | Richard Hunter |
| --- | --- |
| Revisions By: |  |
| Version | 0.0.1 |
| Date | 9/27/2013 |

# Table of Contents

Table of Contents	1

1.0	Introduction	2

2.0	Functional Description	2

2.1	System Diagram	2

2.2	Functional Diagrams	2

2.3	Control interfaces and protocols	2

2.4	Electrical Interfaces	2

2.5	System power	2

3.0	Controls	2

3.1	GUI Controls	3

3.2	System Controls	3

3.3	Switches and Jumper settings	3

4.0	Validation	3

4.1	System configuration	3

4.2	System Power	3

4.3	Control Verification	3

4.3.1	GUI Controls	3

4.3.2	System Controls	4

4.3.3	Switches and Jumper settings	4

4.4	Electrical verification	4

4.5	Mechanical verification	4

4.6	Thermal verification	4

4.7	Agency verification	4

# Introduction

This update is to correct an issue concerning audio dropout using some external equipment which includes the C58 Xstream PUMA.

# Functional Description

FPGA revision 10.11.11 corrects the timing issue which causes some equipment to have audio dropout issues.

## System Diagram

NA

## Functional Diagrams

NA

## Control interfaces and protocols

NA

## Electrical Interfaces

NA

## System power

NA

# Controls

| MUX Controls |  |  |
| --- | --- | --- |
| Information | General Board information including Software revisions, serial number,I/0 Range, Reference, temperature, and Power rail status |  |
| Output Settings | Level adjust | Allows adjustment of all channels by +/- 6db |
|  | Phase Invert | Inverts phase of the selected channel |
|  | Audio Delay | Displays the amount of delay assigned to each channel |
|  | Stereo Remedies | Allows manipulation of selected channel by being Normal, Swapped, (L+R)/2, and (L-R)/2 |
| Output Delay | I/O Channel | Selects channel to be delayed |
|  | Delay Amount | Selects delay amount for each channel |
|  | Delay Mode | Determines total number of channels which can be delayed |
|  | Delay Units | Selects between frames and milliseconds for measuring delay |
| MUX Setup | Pass-Through |  |
|  | DRS Group Assignment | Manual selection of channels for video groups |
|  | Default Config | Populates groups 1 and 2 for all video for 128 channel mode, and populates all groups for 256 channel operation |
| MUX Timecode | Enable | Enables Timecode for selected video |
|  | Timecode Channel Assignment | Assigns DRS channel for timecode |
|  | Apply Changes | Applies changes made on each page |

## GUI Controls

Cattrax.

## System Controls

NA

## Switches and Jumper settings

NA

# Validation

## System configuration

Complete listing of all hardware and documentation used for validation

Hardware/Software used for evaluation

| Part Number | Agile Revision | Agile Description |
| --- | --- | --- |
| 81905607750 | B | CHEETAH DRS EMBEDDER/DEEMBEDDER CONFIG CPLD SOURCE CODE |
| Preliminary | 10.11.11 | **CHEETAH DRS EMBEDDER BOARD CONFIGURATION FLASH ROHS |
| 81906531510 | AA | _CHEETAH OUTPUT COMBINER EMBEDDER HD MULTI RATE |
| DRS-DXE-1310 | C | CHEEETAH DRS DXE WITH 1310NM SM FIBER |
| DRS-EDXE-1310 | D | DRS ENTERPRISE DXE W/1310NM SM FIB SUPPORTS 1536 IN 1536 OUT ONE CNTRL P/S |
| 81905607330 | AB | CD ISO IMAGE FOR CATTRAX V3.3.0 |
| XSTREAM-US | B | XSTREAM WITH US POWER CORD |

## Test equipment used for validation

| Manufacturer | Model Number | Function |
| --- | --- | --- |
| Leader | LV-5800 | Video/Audio waveform monitor/analyzer |
| Phabrix | SxA | Video/Audio Generator/Analyzer |
| Wohler | ATSC-2 | Audio Monitor |

## System Power

NA

## Control Verification

### GUI Controls

Test results for all listed GUI controls.

### Output Mux

| Control Name | Control type | Number of elements | Results |
| --- | --- | --- | --- |
| Output Settings | Level adjust | 128 | Verified OK |
|  | Phase Invert | 128 | Verified OK |
|  | Stereo Remedies | 128 | Verified OK |
| Output Delay | I/O Channel | 128 | Verified OK |
|  | Delay Amount | Variable | Verified OK |
|  | Delay Mode | 4 | Verified OK |
|  | Delay Units | 3 | Verified OK |
| MUX Setup | Pass-Through | 64 | Verified OK |
|  | DRS Group Assignment | 64 | Verified OK |
|  | Mute | 64 | Verified OK |
|  | Default Config | 1 | Verified OK |
| MUX Timecode | Enable | 16 | Verified OK |
|  | Timecode Channel Assignment | 128 | Verified OK |
|  | Apply Changes | 1 | Verified OK |

### System Controls

All system controls and functions performed as expected in both standard and enterprise DRS.  The audio issues exhibited in the input section of the PESA Xstream are completely corrected with the implementation of rev 10.11.11 FPGA code.

### Switches and Jumper settings

NA

## Electrical verification

NA

## Mechanical verification

NA

## Thermal verification

NA

## Agency verification

NA