PESA

Enterprise DRS

PERC1500 Version 2.2

Product Functional Definition and Validation

| Written By: | Richard Hunter |
| --- | --- |
| Revisions By: |  |
| Version | 0.0.1 |
| Date | 2/22/2012 |

# Table of Contents

Table of Contents	1

1.0	Introduction	2

2.0	Functional Description	2

2.1	System Diagram	2

2.2	Functional Diagrams	2

2.3	Control interfaces and protocols	2

2.4	Electrical Interfaces	2

2.5	System power	2

3.0	Controls	2

3.1	GUI Controls	2

3.2	System Controls	2

3.3	Switches and Jumper settings	4

4.0	Validation	5

4.1	System configuration	5

4.2	System Power	5

4.3	Control Verification	5

4.3.1	GUI Controls	5

4.3.2	System Controls	5

4.3.3	Switches and Jumper settings	5

4.4	Electrical verification	7

4.5	Mechanical verification	7

4.6	Thermal verification	7

4.7	Agency verification	7

# Introduction

The purpose of the document is to validate the updated release of the Enterprise DRS controller main application to version 2.2

# Functional Description

This release adds support for the Generation II Embedder and DeEmbedder, in both 128 and 256 channel mode.

## System Diagram

N/A

## Functional Diagrams

N/A

## Control interfaces and protocols

## Electrical Interfaces

Ethernet

## System power

N/A

# Controls Detailed

3.0.1  GUI Controls

3.0.2  Main EDXE Screen

A.  EDXE Information

Displays system information for both Primary and Redundant EDXE

- Primary and Redundant Controller Status

- EDXE Output range

- Condition of power to EDXE

- Serial Number

- Sync status

- FPGA revision

        - EDXE Status

Displays status information for both Primary and Redundant EDXE

- Power Status of all power rails within the EDXE

- Temperature and Temperature Status of EDXE

- Fiber Link Status for each Fiber Port

  - Status up or down

  - RX Power

  - TX Power

  - SFP Temperature

  - Wavelength

- Physical Port Connections for each of 24 ports

- Data Control for each of 24 ports

        - Port Configuration

- Graphical reference of port status

- Port configuration table listing Primary and Redundant EDXE

- Port configuration controls

  - Refresh Port Configuration data – retrieves currently stored system port data.

  - Send Port Config Data to DXE – sends configuration changes to DXE.

  - Copy Board Type – copies detected board type information to I/O board type column.

    - I/O Ports Screen

    - Summary

Summary table of all Port Activity

1.  Port ID – Port number of unit.

      - Active – Whether port is active or not.

      - Type – Indicates type of I/O device attached to port.

      - Range – The range of inputs or outputs used by port.

      - Sync Reference – origin of sync reference for I/O.

      - Temperature – Temperature reported by I/O board.

      - PS1/PS2 – displays whether Power supply for I/O is present and active

      - Fan #1/Fan #2 – displays fan activity.

        - Individual Port controls

There are several types of I/O cards for the EDXE system.  There are input, output, and boards which share inputs and outputs.  While the boards vary, there are some common items which are available on each individual port tab.

  - Information – Identifies board type, I/O range, power supply condition, serial number information, code revision, reference information, and the condition of the individual power rails

  - Input, Output, or Input and Output settings – This screen gives information about each port of the particular I/O, including Channel assignments and delay settings.  On this screen adjustments are also available for Level, Phase inversion, and Stereo Remedies.

  - Input or Output Delay – on this screen delay can be set for 32, 64, or all 128 channels of the particular I/O card.

3.0.4  PERC1500 Frame Controller

A.  Information – gives information on:

1.  Type

2.  Serial Number

3.  IP Address

4.  MAC Address

5.  Fan

6.  Power Supply

7.  Bootstrap

8.  Main APP

9.  Factory App

10. Redboot

NOTE:  This information is provided for both the primary and the redundant PERC1500 in the system.

A.  Control

1.  Active/Standby - This allows the selection of the active controller.

B.  Configuration

1.  Router/Switch Test – used for system switching tests.

C.  Status

Provides the status of power supplies and their respective fans

## System Controls

Interface port and detailed list of commands

  - Switches and Jumper Settings

N/A

# Validation

## System configuration

Complete listing of all hardware and documentation used for validation

Hardware/Software under validation

| Hardware | Part Number | Agile Revision | Agile Description |
| --- | --- | --- | --- |
| DRS-EDXE-PSFC | DRS-EDXE-1310 | A | DRS ENTERPRISE DXE POWER SUPPLY WITH PERC1500 FRAME CONTROLLER |
| Main App | 81905608100 | ? | P1500 SOFTWARE APPLICATION IMAGE V2.2 |

Software / FPGA code

| Part Number | Agile Revision | Agile Description |
| --- | --- | --- |
| 81905607930 | A | COMMON REDBOOT FIS INFO IMAGE |
| 81905608070 | F | SOFTWARE DRS EDXE AND JAGUAR PERC1500 SYSTEM CONTROLLER |
| 81905608080 | A | BOOTSTRAP IMAGE WITH CPLD, AT91SAM9260 |
| 81905608090 | B | COMMON BOOTLOADER IMAGE WITH CPLD, REDBOOT V2.0 |

### Supporting hardware:

| Hardware | Part Number | Agile Revision | Agile Description |
| --- | --- | --- | --- |
| PERC 2000 Controller | 81906527030 | N | _PERC2000 SYSTEM CONTROLLER |

## System Power

N/A

## Control Verification

### GUI Controls

Test results for all listed GUI controls.

| Control | Version | Results |
| --- | --- | --- |
| Cattrax | 3.2.4 rev 4050 | All Cattrax functions OK |

### System Controls

Tests performed in Cattrax unless otherwise noted

      - Verify EDXE Information, Status, and Configuration

Methods

Get and verify initial system information

Verify status and changes within the system

Make system changes Port Configuration changes and verify results

Verify that system sees and configures new Gen II I/O cards

Results

- Information is correct for all system information

- Made system changes which affect Status and noted Status changes.

- Changes reflect correctly on both working physical I/Os, and on status

- Both 128 and 256 channel Gen II Embedder and DeEmbedder cards were handled correctly by the system.

      - Active/Standby status

Methods

Change active/standby PERC1500 controllers on controller and in Cattrax

Results

- Changed controllers and verified status change on both controller and in Cattrax

      - Ports

Methods

Changed Port configuration

Results

- Changed port configuration and noted changes were reflected in EDXE I/O configuration.

      - Fiber Ports

NOTE:  The EDXEs in the lab have no working fiber ports and therefore could not be verified.

      - Redundancy

Methods

Verify that EDXE works properly in redundant mode

Results

System functions correctly with redundant EDXE configuration.

### Switches and Jumper settings

N/A

## Electrical verification

N/A

## Mechanical verification

N/A

## Thermal verification

N/A

## Agency verification

N/A