Telex Communications Inc.

Pro Sound & Entertainment Group

How To Guide #

Update Adam Master Controller Firmware Via

ADAMEDIT

Author:-	Struan Robertson

Date:-		05 May 1998

Issue:-		1.0

# Table Of Contents

1. Table Of Contents

2. Issue Status

3. Requirements

4. Procedure

# Issue Status

| Issue | Date | Reason |
| --- | --- | --- |
| 1.0 | 05 May 1998 |  |

# Requirements

- Computer running ADAMEDIT

- The new combined Hex file for the Controller card

# Procedure

The following has to be carried out to update the Controller firmware in the Adam controller card.

Please follow the following procedure:-

- Save the configuration that is currently in the ADAM to a file on the computer. This needs to done as the upgrade to the controller firmware clears the memory in the Controllers.

- Place the combined Hex file into a temp directory

- Open ADAM EDIT and ensure that you are online.

- If you have two controllers remove the slave one (only has on red LED on).

- Select Status from the pop downs along the top of the screen.

- Select Software Versions.

- Select Master Controllers.

- It should show the current version

- Highlight the line by  clicking on it with the cursor.

- Now do the following keystroke Ctrl + Shift 5 and D.

- A dialogue box named Firmware Download should appear.

- From the Temp directory created earlier select the file combined Hex select OK

- You now should have a dialogue box named Download Device Firmware.

- Select Begin Download.

- The download bar should show the % . This takes a few minutes.

- When this is complete the system will reset.

- Wait a minute and re-connect ADAM EDIT

- Confirm that the Master Controller Version is now the new version .

- Send saved configuration file.

- Now remove the update controller and insert the slave and repeat steps 3, 5 to 19.