| Release Title: | 5.2.0 |
| --- | --- |
| Release Date: | 11-Feb-2015 |
| Version: | 5.2.0 |

# 1. Remarks

The following document contains the release details for the Version 5.2.0 for OMI and OKI and OEI-2 version 1.4.0.This release is major release for RTS based OMNEO products especially OMI and OKI as this fixes many of the issues related to

  - Firmware Upgrade for the OMI cards

  - Mis-match of IP address and Device Name between OMI Controller and OMI-Audio Device

  - Display of IP address on AZEdit and IPEdit

  - Audio Route Errors due to network disconnection /reconnection

  - Intermediate Reset of the OMI

  - Switching between OMNEO and AIO mode

  - Stability of OMI-OMI audio route connections

  - Issues related to swapping of the OMI cards

  - Issues related to Trunking (Display of Remote Trunked Alphas)

  - OEI-2 support for all panel types (MKP4, KP32 Classic Ver 2.3.0,…)

  - Control Bus errors resulting in the freeze of OMI cards

  - OKI device reporting  –103DB gain issue

# 2. New features

  - Support for connecting to OEI-2 Device is provided from this version onwards. Previous 4.x.x version of OMI does not support connecting to OEI-2 and should not be used

  - Support for KP4000/5000 series new key panel . Previous 4.x.x version of OMI does not support these key panels.

  - Audio Routes set-up between OMI and other Dante devices made are retained/saved across resets and power-ups. Note only Audio Routes set-up via Dante Controller are saved.

  - Capability to display and edit both the OMI IP address. This needs AZEdit Version 4.7.0 and IPEdit 3.1.0. Do not use previous version of AZEdit and IPEdit with this version

  - Capability to display the MAC Address of the OMI-Controller and OMI-Audio Device

  - Capability to display both the OMI-Controller and OMI-Audio Device version in the version info string on AZEdit and IPEdit.

  - AZEdit displays both the Data Connection and OMNEO Audio Connection loss in the Alarm status window per port

  - Support for 8-bit protocol in the OMI and the OKI.

  - Support for 255 Intercoms

  - Add following new LED indication on the OMI card for Diagnostic

    - RED LED 19 will be on if the OMI-Controller Device name and OMI-Audio Device do not sync

    - RED LED 18 will be on if the OMI-Controller IP is not in the same range as OMI-Audio Device. If both the IP is in the same range then the GREEN LED 18 will be on

    - RED LED 0 will be blinking indicating that the OMI-Card is working normally. If it stays solid or is off it indicates there is an error

    - During the Firmware Upgrade of the OMI-Controller RED Led 0-6 will sequentially turn on and off.

    - RED LED 1 will blink to indicate that the communication with OMI-Audio device is normal.

    - RED  LED 2-6 is used for OMI internal firmware status

    - RED LED 7 ON indicates the OMI is configured in static mode

  - OMNEO platform migration to version 3.05 from existing version 3.01

# 3. Known Issue

- Under Some conditions of reset of the OMI card, the OMI-Controller is not able to resolve the OMI-Audio device. This would be indicated by RED LED 0 staying either solid or off.

  - Workaround is to manually reset the card.

- When upgrading from 4.x.x version to V5.2.0 , a manual reset of the OMI card is required once the complete upgrade process is completed (Upgrade for both the OMI-Controller and OMI-Audio device)

- If the IP address of the connected KP is changed from DHCP to static, the OMI would take 5 minutes to connect back again.

- Upper case and lower case Device Name for the OKI connected KP still not handled. The user can still give “MARY” and “mary” from the OKI/OEI-2 connected Key panel and it will be accepted. However this would not be allowed if changed from IPEdit/AZEdit.

# 4. Software Compatibility Matrix

The Version 5.2.0 firmware requires following version of the software/firmware

- OMI 				5.2.0

- OKI					5.2.0

- OEI-2				1.4.0

- IPEdit				3.2.0

- AZEdit				5.0.0

- MCIIe				3.0.4 or above

- FWUT				3.30.2877

- Bosch DNS-SD 			3.30.2877

- SetupOMNEONiosFirmware 	3.05

- SetupOMNEOOEI2Firmware	3.05

- SetupOMNEOOKPFirmware	3.05

# 5. Firmware Versions

From this release onwards it is possible to update the OMI card with the single firmware file (capfw). The following constraints still exist

- If the OMI card version is 4.0.0, then separate capfw files are required to update the OMI card to Ver 5.2.0. The OMI-Controller should be updated first

  - OMI-Controller. capfw

  - OMI-Audio-Device.capfw

- For OMI card with version 4.02 and above single capfw file is required. It will update both the controller and audio device on the OMI.

The following table gives firmware version

| Component | Version |  |  | Comment |
| --- | --- | --- | --- | --- |
| OKI-Firmware_V5.2.0.capfw | 5.2.0 |  | This file is used for upgrading the OKI board set with the above versions using the FWUT |  |
| The following firmware file contains images for the both the OMI controller and Audio Device and should be used for upgrading OMI card with Versions 4.02 and above. |  |  |  |  |
| OMI-Firmware_V5.2.0.capfw | 5.2.0 |  | This file has to be used for upgrading OMI cards with version 4.02 and above. Please read the FWUT instructions |  |
| OMI-FPGA.capfw | 5.2.0 |  | This updates only the NIOS FPGA |  |
| Component | Version |  |  | Comment |
| The following firmware file contains images for the both the OMI controller and Audio Device and should be used for upgrading OMI card with Versions 4.0.0 |  |  |  |  |
| OMI-Firmware_Controller_V5.2.0.capfw |  | 5.2.0 | This file is used for upgrading the OMI board set with the above versions using FWUT. This only updates the NIOS processor |  |
| OMI-Firmware_AudioDevice_V5.2.0.capfw |  | 5.2.0 | This file is used for upgrading the OMI board set with the above versions using FWUT |  |

| Component | Version |  | Comment |
| --- | --- | --- | --- |
| The following firmware file contains images for the OEI-2 device from version 1.2.3. Versions prior to V1.2.3 are not supported. |  |  |  |
| OEI2-Firmware_V1.4.0.capfw | 1.4.0 | This file is used for upgrading the OEI-2 using the FWUT |  |