| Release Title: | 5.2.5 |
| --- | --- |
| Release Date: | 05-May-2015 |
| Version: | 5.2.5 |

# 1. Remarks

This release contains bugfixes and improvements for the OMI, OKI and OEI-2.  This release onwards contains RSTP and noRSTP version of the capfw file for all the devices.

The major issues fixed are listed below

OMI

- When OMI is allocated timeslots 445-461 then Audio is not available on these ports.

- OMI-Controller sometimes not able to resolve its Audio device on reset or power-up.

- Log file overflow resulted in OMI connection drops to the Key panels have been addressed

- OMI-controller not renewing its DHCP address on network link  down has been fixed

- Download of large font files has been fixed

OKI

- Ability to restore default Authentication table for OKI connected to Keypanel. It is now possible to send a command from the Keypanel to restore the default authentication table. This needs an update of the Key panel Firmware also.

- Switching from OMNEO to AIO addressed. Earlier it was observed if the user delayed activating the change, the panel would not switch to AIO mode.

- Device name change ignored if there is no difference

OEI-2

- Sometimes OEI-2 connected Keypanels do not make data connection. This has been fixed.

# 2. Previous Release

V5.2.1

The following document contains the release notes applicable to OMI firmware only. In the OMI firmware there was one issue resolved. The following issue was resolved

- OMI card does not come up from reset in the scenario if the right-hand side MCIIe is active (Physical Slot 20) and left hand-side (Physical Slot 19) MCIIe is in standby mode. This issue is only observed when both active and standby MCIIe are in the frame and Physical Slot 20 MCIIe is the active MCIIe and only if OMI resets. This is not an issue when only single MCIIe is used.

- In addition this release also contains NORSTP version of the OMI-Firmware. These are separate capfw files that need to be uploaded on to the OMI card by using the Firmware Upgrade Tool. Please refer to section 6 of this document.

V5.2.0

The following document contains the release details for the Version 5.2.0 for OMI and OKI and OEI-2 version 1.4.0.This release is major release for RTS based OMNEO products especially OMI and OKI as this fixes many of the issues related to

  - Firmware Upgrade for the OMI cards

  - Mis-match of IP address and Device Name between OMI Controller and OMI-Audio Device

  - Display of IP address on AZEdit and IPEdit

  - Audio Route Errors due to network disconnection /reconnection

  - Intermediate Reset of the OMI

  - Switching between OMNEO and AIO mode

  - Stability of OMI-OMI audio route connections

  - Issues related to swapping of the OMI cards

  - Issues related to Trunking (Display of Remote Trunked Alphas)

  - OEI-2 support for all panel types (MKP4, KP32 Classic Ver 2.3.0,…)

  - Control Bus errors resulting in the freeze of OMI cards

  - OKI device reporting  –103DB gain issue

# New features added in V5.2.0

  - Support for connecting to OEI-2 Device is provided from this version onwards. Previous 4.x.x version of OMI does not support connecting to OEI-2 and should not be used

  - Support for KP4000/5000 series new key panel . Previous 4.x.x version of OMI does not support these key panels.

  - Audio Routes set-up between OMI and other Dante devices made are retained/saved across resets and power-ups. Note only Audio Routes set-up via Dante Controller are saved.

  - Capability to display and edit both the OMI IP address. This needs AZEdit Version 4.7.0 and IPEdit 3.1.0. Do not use previous version of AZEdit and IPEdit with this version

  - Capability to display the MAC Address of the OMI-Controller and OMI-Audio Device

  - Capability to display both the OMI-Controller and OMI-Audio Device version in the version info string on AZEdit and IPEdit.

  - AZEdit displays both the Data Connection and OMNEO Audio Connection loss in the Alarm status window per port

  - Support for 8-bit protocol in the OMI and the OKI.

  - Support for 255 Intercoms

  - Add following new LED indication on the OMI card for Diagnostic

    - RED LED 19 will be on if the OMI-Controller Device name and OMI-Audio Device do not sync

    - RED LED 18 will be on if the OMI-Controller IP is not in the same range as OMI-Audio Device. If both the IP is in the same range then the GREEN LED 18 will be on

    - RED LED 0 will be blinking indicating that the OMI-Card is working normally. If it stays solid or is off it indicates there is an error

    - During the Firmware Upgrade of the OMI-Controller RED Led 0-6 will sequentially turn on and off.

    - RED LED 1 will blink to indicate that the communication with OMI-Audio device is normal.

    - RED  LED 2-6 is used for OMI internal firmware status

    - RED LED 7 ON indicates the OMI is configured in static mode

  - OMNEO platform migration to version 3.05 from existing version 3.01

# 3. Known Issue

- Sometimes all Audio Routes are not established when OMI is reset.

# 4. Software Compatibility Matrix

The Version 5.2.5 firmware requires following version of the software/firmware

- OMI 				5.2.0/5.2.1/5.2.5

- OKI					5.2.0/5.2.5

- OEI-2				1.4.0/1.4.2

- IPEdit				3.2.0 and above releases

- AZEdit				4.7.0/5.0.0 and above releases

- MCIIe				3.0.4 or above  releases

- FWUT				3.30.2877

- Bosch DNS-SD 			3.30.2877

- SetupOMNEONiosFirmware 	3.05.2903

- SetupOMNEOOEI2Firmware	3.05.2903

- SetupOMNEOOKPFirmware	3.05.2903

- K32-CLD				V1.6.0 onwards Omneo Support Added

- KP12-CLD				V1.3.0 onwards Omneo Support Added

- KP32-Classic			V2.4.0 onwards Omneo Support Added.

Requires update to boot loader

# 5. Firmware Versions

From this release onwards it is possible to update the OMI card with the single firmware file (capfw). The following constraints still exist

- If the OMI card version is 4.0.0, then separate capfw files are required to update the OMI card to Ver 5.2.5. The OMI-Controller should be updated first

  - OMI-Controller. capfw

  - OMI-Audio-Device.capfw

- For OMI card with version 4.02 and above single capfw file is required. It will update both the controller and audio device on the OMI.

The following table gives firmware version

| Component | Version | Comment |
| --- | --- | --- |
| OKI-Firmware_RSTP_V5.2.5.capfw OKI-Firmware_noRSTP _V5.2.5.capfw | 5.2.5 | This file is used for upgrading the OKI board set with the above versions using the FWUT |
| The following firmware file contains images for the both the OMI controller and Audio Device and should be used for upgrading OMI card with Versions 4.02 and above. |  |  |
| OMI-Firmware_RSTP_V5.2.5.capfw OMI-Firmware_noRSTP_V5.2.5.capfw | 5.2.5 | This file has to be used for upgrading OMI cards with version 4.02 and above. Please read the FWUT instructions |
| OMI-FPGA_Firmware_V0.48.capfw | 0.48 | This updates only the NIOS FPGA |
| Component | Version | Comment |
| The following firmware file contains images for the both the OMI controller and Audio Device and should be used for upgrading OMI card with Versions 4.0.0 |  |  |
| OMI-Firmware_Controller_V5.2.5.capfw | 5.2.5 | This file is used for upgrading the OMI board set with the above versions using FWUT. This only updates the NIOS processor |
| OMI-Firmware_AudioDevice_RSTP_V5.2.5.capfw OMI-Firmware_AudioDevice_noRSTP__V5.2.5.capfw | 5.2.5 | This file is used for upgrading the OMI board set with the above versions using FWUT |

| Component | Version |  | Comment |
| --- | --- | --- | --- |
| The following firmware file contains images for the OEI-2 device from version 1.2.3. Versions prior to V1.2.3 are not supported. |  |  |  |
| OEI2-Firmware_RSTP_V1.4.2.capfw OEI2-Firmware_noRSTP_V1.4.2.capfw | 1.4.2 | This file is used for upgrading the OEI-2 using the FWUT |  |