KPG-23D Help File Version 1.01

File name: KPG-23D.HLP

September-09-1994

Y57-3190-20

Modification History:

Version	Release Date	Description

1.00	05/19/'94	KPG-23D First Version.

1.01	09/09/'94	Add the Space Bar TX RX control Help.

General Help

*** [ General Help ] ***

Along the top border of the main program screen (Channel Info.) are the

"drop down" menus. Press ALT; and use the cursor keys --> <-- to select the

desired program section. Help is available for each menu by pressing the F1

function key. Pressing the F1 Function key while in the Channel Info Screen

will load it's Help screen. To get familiar with the function keys used by

the KPG-23D software press the ALT key, use the cursor keys to select

Keyboard Help; and press ENTER.    Within a Help screen use cursor keys

up/down to scroll; F5 to print a hardcopy; and ESC or ALT to escape the

screen. Refer to the KPG-23D's Instruction Manual for additional help.

**For Basic Instructions (use F1 for Help in each program section below)**

A. [CREATING PROGRAMMING DATA FOR A RADIO]

1. Open the menu marked "MODEL" and select the appropriate model type.

2. Open the menu marked "EDIT" and select "Key Selection." Assign

functions to each key on the top panel head. (Hint: use ALT to return

to EDIT menu)

3. Open the menu "EDIT-Scan Information." Set the SCAN features &

parameters.

4. Open the menu "EDIT-Feature Option."    Set the general built-in

features.

5. Open the menu "PROGRAM Test mode." Set specific radio adjustments.

6. Hit ESC. the cursor will move to the "CHANNEL INFORMATION SCREEN, the

main screen. Set all per-channel and/or channel group information.

B. [PROGRAMMING A RADIO]

1.    Open the "PROGRAM" menu.

2.    Select "Write to radio".

C. [SAVE PROGRAMMING DATA TO A DISK FILE]

1. Open "FILE" menu.

2. Select "SAVE" for quick save to the "default" or "current" filename.

or "SAVE AS" to name or rename the file..

*Organizational Chart of the KPG-23D*****

[ MAIN SCREEN ]

+----- CHANNEL----+-- Receive & Transmit Frequencies

|        INFORMATION    +-- QT/DQT Decode & Encode

|                                  +-- Channel Name (Alphanumeric Display Info.)

|                                  +-- Channel Delete/Add for Scan

|                                  +-- RF Power High, Med or Low

|                                  +-- Option Signaling (Built-in 2-tone A,B,C

|                                                                                          or DTMF decoder)

|                                  +-- Busy Channel Lockout Y/N (Y=enabled, N=disabled)

|                                  +-- CPU Clock Shift Y/N (Y=enabled, N=disabled)

|                                  +-- Scrambler Y/N (Y=enabled, N=disabled)

|                                  +-- Scrambler Code (CODE 1-4)

|                                  +-- Priority Channel set

|                                                (press F10 to set Fixed Priority Channel)

|                                  +-- Group Delete/Add for Group Scan

|                                  +-- (IF Frequency & Ch Step) (CAUTION: Needs hardware

|                                                                                              modifications DO NOT USE)

|

+----- FILE ----+-- New

|                              +-- Load

|                              +-- Save(=F4)

|                              +-- Save As

|                              +-- Delete

|                              +-- Print(=F5)

|                              +-- Go to DOS

|                              +-- Exit(=F3)

|

+----- MODEL----+-- TK-250    32 Ch

|                              +-- TK-250 160 Ch

|                              +-- TK-350    32 Ch

|                              +-- TK-350 160 Ch

|

+----- EDIT-----+-- Tuning Frequency

|                              |        (Maximum 10 frequencies)

|                              |

|                              +-- Key selection---+- Scan

|                              |                                      +- D/A

|                              |                                      +- Talk Around

|                              |                                      +- Low

|                              |                                      +- Monitor

|                              |

|                              +-- Scan -----------+- Drop Out Delay Time

|                              |      Information          +- TX Dwell Time

|                              |                                      +- Priority Channel

|                              |                                      +- Look Back Time A

|                              |                                      +- Look Back Time B

|                              |                                      +- Revert Channel

|                              |                                      +- Group Scan

|                              |

|                              +-- Feature Option--+- Time Out Timer

|                              |                                      +- Power Save

|                              |                                      +- Power On Tone

|                              |                                      +- Control Tone

|                              |                                      +- Warning Tone

|                              |                                      +- Call Tone

|                              |                                      +- Call LED

|                              |                                      +- DTMF Monitor

|                              |                                      +- Busy LED

|                              |                                      +- Clone

|                              |                                      +- Test Mode

|                              |                                      +- Self Programming

|                              |                                      +- Main Programming

|                              |                                      +- 2-Tone Decode

|                              |                                      +- DTMF Decode

|                              |                                      +- DTMF Encode

|                              |                                      +- DTMF Pre-stored Number

|                              |

|                              +-- Embedded Message

|                                        (64 characters max. electronic serial # etc.)

|

+----- PROGRAM -+-- Read From Radio

|                              +-- Write To Radio

|                              +-- TEST MODE -- PC Tuning ------+- Squelch

|                                                                                                +- RF Power High

|                                                                                                +- RF Power Medium

|                                                                                                +- RF Power Low

|                                                                                                +- MAX Dev.

|                                                                                                +- DQT Bal.

|                                                                                                +- Tone Fine Dev. QT

|                                                                                                +- Tone Fine Dev. DQT

|                                                                                                +- DTMF Dev.

|                                                                                                +- Print Tuning Data

|

+----- SETUP----+-- Communication Port

|                              +-- Display Color --+- Black & White

|                              |                                      +- Display Back Color

|                              |                                      +- Input Data Color

|                              |                                      +- Feature Highlight Color

|                              |                                      +- Screen Border Information Color

|                              |                                      +- Window Menu Color

|                              |                                      +- General Message Color

|                              |                                      +- Windows Color

|                              |                                      +- Help Window Color

|                              +-- Help Path

|

+------ HELP---------+-- GENERAL HELP

+-- COMPUTER KEYBOARD HELP

Keyboard Help

*** [ Computer Keyboard Help ] ***

Available key functions are displayed at the bottom of the screen.

ESC: Returns to the main screen,    called the Channel Information Screen.

(exits menus & sub-screens).

ALT: "Go to Menu" or "Go back" to previous screen.

Arrow Keys: Move the cursor within a data input field;

scrolls help screens.

PgUp & PgDn:    Pages up/down Channel Group & Help screens.

F1: Opens a Help screen.

( F2: through F10: Select labeled functions displayed at bottom of

the screen.)

F2: Toggles Yes/No or increments selection in data fields.

F3: Exit from this program and returns to the DOS PROMPT.

F4: Saves the current program data with current file name.

F5: Prints the current data or Help screen.

F7: Next page/ channel group (same as page up).

F8: Previous page / channel group (same as page down).

F10: Set the Priority channel.

Shift+F2: Opposite function as the F2 key.

Shift+F3: Copies an entire channel to another group channel.

Shift+F4: Copies an RX frequency column entry to a TX frequency column.

Shift+F5: Copies a QT/DQT decode column entry to the encode column.

Shift+F6: Changes the IF mixer input Frequency to the upper or lower mixer

Frequency. (Needs hardware modifications, see S/M for details)

Shift+F8: Copies an entire group to another group (only 160 channel

version).

Shift+F9: Channel Insert (inserts a blank channel row & advances

the current channel data(such as Ch name) & all subsequent

channels to the next channel row).

Shift+F10: Channel Delete (deletes the current channel data(such as Ch

name) row and decrements all subsequent channel rows by one).

HOME: Jumps to (group 1) channel 1 of the Channel Info. screen,

or to page 1 of a help screen.

END: Jumps to the last channel(Ch 32 or Gr10 Ch16) of the Channel Info.

screen.    Or to the last page of a help screen.

ENTER: Enters data into a program field, or opens a selection window.

SPACE BAR: Toggles Yes/No selections, or increments selections.

(same as F2)

Delete pre-programmed data.

TX and RX control while the Test and Tuning mode.

Shift+SPACE BAR: Opposite function to the SPACE BAR.

New File Help (CLEAR FILE)

Selecting the NEW function clears any

currently loaded, read or write file

data.    All program

data screens will be set to their

defaults.

Load Help

The LOAD function is used to retrieve

a stored data file from a hard disk

or floppy disk. Afterwards,    the file

can be    modified, viewed, used to

program a radio (written),    or printed

out.    A directory of stored files,

sorted alphanumerically,    are

displayed for selection.

Save Help

Saves the current program data

under the current file name.

Save As Help

Saves with the option of

assigning a "new" file name, or

renames an existing file.

Delete Help

The DELETE function is used to remove

any stored data files from the

directory containing stored files.

Print Help

Prints all radio programming

data currently loaded into the

program:

- Model Type

- File Name

- Embedded Message

- Model Type Configuration

- Key Selection

- Scan Information

- Feature Option

- IF Frequency

- Priority Channel Information

- Channel Information

- Test Mode

Go to Dos Help

To temporally go to the

DOS prompt. Open the menu "FILE-Go to

dos" and press ENTER.

To return to the KPG-23D Program,

type EXIT and press ENTER.

Note: The KPG-23D program can support

this command only if your computer has

sufficient memory.

Exit Help

Quits the DOS prompt and return to

the DOS prompt, type EXIT and

press ENTER.

Channel Name Help

Enter a maximum of 4 characters.

The RX frequency must be entered

BEFORE the channel name.

Note: The channel name assigned to the

RX frequency.

Frequency Help

Type a Receive and Transmit frequency

directly.    Receive-only channels are

allowed.    Frequencies can program

within the displayed band range of

the PLL, and the proper step size.

QT/DQT Help

Type tone or code directly.

QT: 60.0 to 300.0 Hz (in 0.1 Hz steps)

DQT: codes 000 to 777

Enter D###N    for    (Normal)

D###I    for (Inverted)

(codes default to Normal if ( I )

is    not entered)

-or-

Press F2 key, open QT/DQT chart,

(QT: 67.0 to 250.3,)

(DQT: 023 to 754 in the Chart)

then select.

Scan Help

Add: Adds the CH to the SCAN map.

(default)

Del: Deletes the CH from the SCAN map.

Press the F2 key or SPACE BAR to

alternate between selections.

Power Help

Select High,    Medium,    or Low Power.

High Power:        5 Watts (VHF)

4 Watts (UHF)

Medium Power : 2 Watts

Low Power :        1 Watt

Press the F2 key or SPACE BAR to

alternate selection of High,    Med, or

Low Power.

Option Signalling Help

Select one of    four built-in types of

DTMF or 2-tone(A,B,C) decoder per

channel.

Set the decode tone or code in the

Feature option menu.

If    QT or    DQT decode is also

programmed both the OPT SIG & QT/DQT

tones must be correctly decoded before

the audio unmutes.

Press the F2 key or SPACE BAR to

alternate between the selection of

DTMF, 2ToneA, 2ToneB, 2ToneC,

or NO.

Busy Channel Lock Out Help

TX inhibited when the channel is busy.

Press the F2 key or SPACE BAR to

alternate between the selections.

Shift Help

Shifts the microprocessor system clock

frequency per channel to prevent

interference.

Press the F2 key or SPACE BAR to

alternate between the selections.

V/SC: Scrambler Yes/No Help

Enables or Disables the installed

scrambler option board per channel.

Press the F2 key or SPACE BAR to

alternate between the selections.

Code: Scrambler Code Help

Selects codes one (1) to four (4).

Group Del/Add Help

Selects Del/Add:    To Add or Delete

the group from the scan list.

If DEL is selected, the radio will not

scan any channel in the group.

Available in the 160 channel version

only.

TK-250 Help

TK-250 ( 32ch): Maximum 32 channels.

TK-250 (160ch): Maximum 10 Groups

16 Channels per group.

Frequency band

F1: 146-174MHz

F2: 136-150MHz

Sp:    Special tuning.

The model name arrows a programmable

frequency range of KPG-23D.

TK-350 Help

TK-350 ( 32ch): Maximum 32 channels.

TK-350 (160ch): Maximum 10 Groups

16 Channels per group.

Frequency band

F1: 450-470MHz

F2: 470-490MHz

F3: 490-520MHz

F4: 400-430MHz

Sp:    Special tuning.

The model name arrows a programmable

frequency range of KPG-23D.

Scrambler Unit Help

Select "Yes" If a Voice Scrambler

unit is installed in the radio.    If

one is not installed,    select "No".

Scan Help

The Scan key can be programmed for

either Enable or Disable.

Press the F2 key or SPACE BAR to

alternate between selections.

Enable:            Enables the scan

function.

Disable:          Disables the scan

function.    If

programmed, sounds

an alert tone when

the scan key is pressed.

D/A Help

The D/A key can be programmed for one

of three functions; D/A1, D/A2

or disable.

Press the F2 key or SPACE BAR to

alternate between selections.

D/A1:                  Temporary nuisance

channel "Delete".

D/A2:                  Memorized Add/Delete

of a Channel in the

scan map.

Disable:            Disables D/A function.

and sounds an alert

tone,    if programmed,

when the key is pressed

Talk Around Help

TALK AROUND ON/OFF; simplex operation

(receive frequency) on any programmed

channel.

The TA key can be programmed for one

of six functions;    Talk Around,

Channel Name, Group UP, Group DOWN,

Scrambler or Disable.

Press the F2 key or SPACE BAR to

alternate selection.

Talk Around:    Switches the Talk around

function ON/OFF.

Channel name: Alternate Channel name

switch.

Group UP(160 Ch version only):Group UP

switch.

Group DOWN(160 Ch version only):Group

DOWN switch.

Scrambler(Scramble unit installed

version only):Voice

Scrambler unit ON/OFF

switch.

Disables:          Disables TA function

and sounds an alert

tone when the key

is pressed.

Low Help

RF LOW POWER ON/OFF; RF power will be

set to LOW on all channels.

The LO key can be programmed for one

of six functions Low, Channel name,

Group UP, Group DOWN, Scrambler or

Disable.

Press the F2 key or SPACE BAR to

alternate between selections.

Low:                  Low Power function

ON/OFF switch.

Channel name:Alternate channel name

switch.

Group UP(160 Ch version only):Group UP

switch.

Group DOWN(160 Ch version only):Group

DOWN switch.

Scrambler(Scramble unit installed

version only):Voice

Scrambler unit ON/OFF

switch.

Disable:          Disables the LO function

and sounds an alert

tone, if programmed,

when the key is pressed.

Monitor Help

MONITOR ON/OFF; supplies a reset

signal to the Option Signalling board.

The monitor switch also enables/

disables QT / DQT tone squelch.

The MON key can be programmed for one

of four functions Monitor 1,

Monitor 2, Monitor 3 or Disable.

Press the F2 key or SPACE BAR to

alternate between selections.

Monitor 1:        Carrier Squelch function

while pressing the key.

Monitor 2:        Toggles between Carrier

Squelch/Tone Squelch

function.

Monitor 3:        Unmute function while

pressing the key.

Disable:            Disables the MON

function and sounds an

alert tone when the key

is pressed.

Drop Out Delay Help

Drop out delay time interval from

the end of a received call to the

time scan automatically resumes.

Press the F2 key, SPACE BAR, <- or ->

to increment the selection.

Selectable 500-5000ms in 250ms steps;

TX Dwell Time Help

TX dwell time interval from the end

of the transmit to the time scan

automatically resumes.

Press the F2 key, SPACE BAR, <- or ->

to increment the selection.

Selectable 500-5000ms in 250ms steps;

Priority Help

Priority channel selection feature.

Press the F2 key,    SPACE BAR, <- or->

to alternate between the selections

of None, Fixed , or Selected.

none:          Priority Channel is

deselected.

Non-priority scan only.

fixed:        You must select one channel

for priority channel.

(To set as the Priority

channel, press F10 key)

selected: The selected channel before

scan is initiated becomes

the Priority Channel.

Look Back Time Help

Priority channel checking intervals

while receiving on a non-priority

channel. The radios CPU chooses either

Lookback time A or B depending on the

priority channel activity --Program

both times.

Press the F2 key, SPACE BAR, <- or ->

to increment selection.

[Lookback A]: Priority channel is not

busy (no carrier detected). This time

is typically set shorter than Lookback

B, since the priority ch. should be

checked more often to prevent

missing a call.

Lookback A: 350-700ms in 50ms steps.

2.    Default=500ms.

3.    (=B) setting    makes Lookback A

same as Lookback B's setting.

[Lookback B]: The priority channel is

busy, but tone squelched. This time

is typically set long because the

priority ch. is busy with another

caller using a different tone.

Lookback B: 750-5000ms in 250ms steps;

default=2000ms

Revert Channel Help

Revert Ch. is the transmit destination

channel during scan, depending on the

signal condition and priority

selection.

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

----:          Selected or talkback

channel.      Default when

Priority Channel    (None)

is selected

Priority: Priority channel at any

time.

Talkback: Priority or talkback

channel.

Group Scan Help

Allows scanning of the currently

selected channel group or

all channel groups.    Available

only on the 160 channel

version (10GR. X 16CH.version).

Press the F2 key, SPACE BAR, <- or ->

to alternate selection.

[Single]-Scans only the currently

selected channel group.

[Multi]--Scans all available channels

within all groups.

Time Out Timer Help

Microprocessor controlled to limit

lengthy transmissions.

Press the F2 key, SPACE BAR, <- or ->

to increment selection.

TX time out can be set from 30 to

300 seconds in 30 sec. steps.

default: 60 sec.

Power Save Help

Automatic battery saver during standby

operation, the receiver circuit is

cycled on & off to conserve battery

life.

The "on" time is automatically

selected by the current signal

conditions.

Press the F2 key, SPACE BAR, <- or ->

to increment selection.

The "off" time is selectable as below

[off], [short], [medium] and [long].

Power On Tone Help

Beep tone at radio power up.

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Control Tone Help

Keys and selector knob echo-back

beeps.

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Warning Tone Help

Time-out timer & Busy

ch. lockout alert tones.

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Call Tone Help

Alert tones will sound after

receiving the correct selective call.

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Call Indicator Help

Call indicator will flash after

receiving the correct selective call.

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

DTMF Encode Sidetone Help

Enable or disable the DTMF encode

sidetone monitor function.

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Busy LED Help

Enable or disable the Busy LED

function.

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Clone Help

Enable or disable the Clone function.

(see S/M for details)

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Test Mode Help

Enable or disable the Test mode

function (factory use only).

(see S/M for details)

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Self Programming Help

Enable or disable the Self Programming

function.

You must also select Test Mode =

Enable when Self Programming = Enable.

(see S/M for details)

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Main Programming Help

Enable or disable the Main Programming

function.

This data is for factory use only.

Select DISABLE before delivering the

radio to the end user to avoid

unintentional changes of the

RADIO's OPERATING SYSTEM.

(see S/M for details)

Tone 1 Help

Set the first tone frequency of the

two tone sequential signaling from

282.2 Hz to 3487.0 Hz in 0.1 Hz steps.

Press <- or -> to increment selection

or directly enter the frequency.

Tone 2 Help

Set the second tone frequency of the

two tone sequential signaling from

282.2 Hz to 3487.0 Hz in 0.1 Hz steps.

Press <- or -> to increment selection

or directly enter the frequency.

Group Call Help

Select the group call function by

receiving either a long TONE 1 or

long TONE 2.

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Call/Transpond Help

If "CALL" is selected.    when a

selective call is correctly decoded

the call indicator flashes until

reset.

If "TRANSPOND" is selected, the

radio returns a transpond tone to

the caller.

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Primary Help

The primary code consists of one to

seven digits and is user-definable.

Reception of this code activates the

call function.

The decoder may be reset locally by

pressing the monitor key or remotely

by sending the primary code

followed by the "#" key.

Enter the required code directly.

Secondary Help

The secondary code consists of one to

seven digits and is user-definable.

Reception of this code activates the

call function.

The decoder may be reset locally by

pressing the monitor key or remotely

by sending the secondary code

followed by the "#" key.

Enter the required code directly.

Dead Beat Disable Help

The Dead Beat Disable code consists of

one to seven digits and is

programmable.

Reception of this code activates the

transpond function and then places the

radio in the Dead Beat Disable mode.

While in the D.B.D mode, the radio

ignores transmit requests.

Reception of the D.B.D code followed

by the "#" key will reset this

function.

A Primary or Secondary code must be

programmed with D.B.D., otherwise the

radio will remain muted on any channel

programmed with OPT SIGNALLING = DTMF.

Enter the required code directly.

Keypad Auto-PTT Help

DTMF tones are sent out exactly as

entered via the keypad. When using

Manual Dial with Auto Dial, Store

& Send, and Last Number Redial(all

explained below), the [*] key cannot

be used to send the "*" DTMF tone.

Available only with the keypad model.

Example: Send: "123-4567".

Enter: 1234567

Sent:    1234567

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

12 Key / 16 Key Help

Selection of the 4th column key board

function.

12Key is used with auto dial.

16Key is used with manual dial.

Available only with the keypad model.

12Key:    4th column set for function

keys.(see the Instruction

Manual for detailed

information).

16Key:    4th column set for DTMF

manual encode key.

(A, B, C and D)

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Keypad Conn./Disc. ANI Help

Enable or disable key board

controlled ANI transmit function.

Available only with the keypad model.

Press the F2 key, SPACE BAR, <- or ->

to alternate selection.

Press "**" for transmit Connect ANI.

Press "*#" for transmit Disconnect

ANI.

Auto Dial Help

Enable or disable automatic

DTMF transmit function.

Available only with the 12 key model.

The 16 key model cannot accepts

"Yes" selection.

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Programmable nine memory area.

Manual Dial Help

Enable or disable manual DTMF

transmit function.

You must also select 16KEY.

Available only with the keypad model.

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Autodial Memory Protect Help

Enable or disable operator

re-programming autodial memory area.

You must also select 12KEY.

Available only with the keypad model.

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Autodial Memory Help

Programmable    memory area to send

out a prestored DTMF number.

The radio has 9 areas for programming.

Each area holds a maximum 16 digits.

Available only on the 12 key model.

Enter the required code directly.

Connect ANI Help

Enter the radio's    programmable

Connect ANI data.(maximum 16 digits)

Enter the required code directly.

Disconnect ANI Help

Enter the radio's    programmable

Disconnect ANI data.(maximum 16

digits)

Enter the required code directly.

Conn. ANI sent at PTT-On Help

Allows the connect ANI to be sent

when PTT is engaged.        (B.O.T.).

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

Disc. ANI sent at PTT-Off Help

Allows the disconnect ANI to be sent

when PTT is released.        (E.O.T.).

Press the F2 key, SPACE BAR, <- or ->

to alternate between selections.

DTMF Speed Help

When using manual dial, the DTMF Speed

is the minimum time period between

digits if the keys are pressed

consecutively.

This time is designed to reduce

false decoder operation by providing

a fixed time period between digits,

similar to DTMF minimum duration and

first digit time.    Selectable 6, 8,

10 or 15 digits per second.

Press the F2 key, SPACE BAR, <- or ->

to increment selection.

1st Digit Help

First digit time is the tone

length of the first digit when using

auto dial. Like the DTMF minimum

duration and the DTMF rise time, first

digit time is intended to reduce the

possibility of improper decoder

operation. This feature extends the

send time of the first digit compared

with subsequent digits.

However, if a transmission starts with

the "*" or "#" tone, the radio

compares the preset first digit time

with the "*" and "#" tone duration,

and adapts the longer of the two.

Selectable 0, 100, 500 or 1000 mSec.

Press the F2 key, SPACE BAR, <- or ->

to increment selections.

* and # Digit Help

The tone duration is the send time of

the "*" and "#" tones. This function

is required by some systems in which

these tones must be set longer than

numeric tones. This setting defines

how much longer the "*" and "#" tones

will be.

However, if a transmission starts with

the "*" or "#" tone, the radio

compares the tone duration with the

preset first digit, and adapts the

longer of the two.

Selectable 0, 100, 500 or 1000 mSec.

Press the F2 key, SPACE BAR, <- or ->

to move between    selections.

Ch Frequency Help

Select the desired frequency from

the Test Frequency table.

Use the cursor keys or enter desired

number.    Then press the Enter key.

Signalling Help

Select desired signalling.

1....None

2....QT Encode/Decode

3....DQT Encode/Decode

4....2-Tone Decode

5....DTMF Decode

6....DTMF Encode 1

7....DTMF Encode 2

8....Square Wave Encode

(see S/M for details)

Use the cursor keys or enter the

desired number.    Then press

the Enter key.

SQL Help

Preset Squelch level adjustment.

Press <- or -> to increment selection.

RF Power High Help

Preset RF Power High level adjustment.

Press <- or -> to increment selection.

RF Power Medium Help

Preset RF Power Mid level Adjustment.

Press <- or -> to increment selection.

RF Power Low Help

Preset RF Power Low level adjustment.

Press <- or -> to increment selection.

Max Dev. Help

Preset Maximum Deviation level

adjustment.

Press <- or -> to increment selection.

DQT Bal. Help

Preset DQT encode balance adjustment.

Press <- or -> to increment selection.

Tone Fine Dev. QT Help

Preset QT encode level adjustment.

Press <- or -> to increment selection.

Tone Fine Dev. DQT Help

Preset DQT encode level adjustment.

Press <- or -> to increment selection.

DTMF DEV. Help

Preset DTMF encode level adjustment.

Press <- or -> to increment selection.

Print Tuning Data Help

Print out a currently memorized tuning

data.

Embedded Message Help

Up to 64 alphanumeric characters can

be programmed into the radio. This

can be used for an electric serial

number, program file name notation,

property ID., etc..

It will appear as part of the header

statement on a program data printout

along with the model type and

filename.

Type required message directly.

Black & White Color Help

Monochrome display mode.

Display Back Color Help

Main window's background color.

Press the F2 key, SPACE BAR, <- or ->

to alternate selection.

Input Data Color Help

Input data cursor color.

Press the F2 key, SPACE BAR, <- or ->

to alternate selection.

Feature Highlight Color Help

Sub-window's selected feature

cursor background color.

Press the F2 key, SPACE BAR, <- or ->

to alternate selection.

Screen Border Info. Color Help

Outside of main window's border

information background color.

Press the F2 key, SPACE BAR, <- or ->

to alternate selection.

Windows Menu Color Help

Main menu bar sub windows background

color.

Press the F2 key, SPACE BAR, <- or ->

to alternate selection.

General Message Color Help

Important information window's

background color.

Press the F2 key, SPACE BAR, <- or ->

to alternate selection.

Windows Color Help

Sub menu bar sub window's background

color.

Press the F2 key, SPACE BAR, <- or ->

to alternate selection.

Help Window Color Help

Help window's background color.

Press the F2 key, SPACE BAR, <- or ->

to alternate selection.

Priority Channel Help

Fixed Priority select function.

If the Priority Channel is set to

Fixed, select a priority channel

before writing to the radio or saving

to the database.

Use the F10 key to set select the

Priority channel.

Channel Copy Help

Copies the entire channel data to

another channel(s).

Use SHIFT + F3 to copy a channel.

Frequency Copy Help

Copies a channels RX frequency

to the TX column.

Use Shift + F4 to copy a RX frequency.

QT/DQT Copy Help

Copies the data of QT/DQT from

a QT/DQT decode column to

the encode column.

Use Shift + F5 to copy decode tone.

IF & Heterodyne Help

Do not change IF & Heterodyne in

normal use to avoid DAMAGE of DATA.

This data is for factory use only.

Press the F2 key, SPACE BAR, <- or ->

to increment selection.

Group Copy Help

Available only for 160 channel radios.

Copies all data of an entire group

to another group.

Use SHIFT + F8 to copy a group.

Read From Radio Help

Read all program data from the radio--

-- select & hit enter.

Write To Radio Help

Writes all currently loaded program

data into the radio's EEPROM memory.

Before writing to the radio, Verify

the model type selected in your

current data file matches the    model

of target radio.    ---Then    hit Enter

to program the radio.

Communications Port Help

Select the COM level of the computer's

serial communication port for

programming.

These are two choices, COM1 or COM2.

COM level setting's in your computer

may be dependent on internal hardware

and/or setup software settings; check

your computer manuals for details.

Press <- or -> to alternate selection.

Help Path Help

Help message files are contained in

this program under

drive:\KPG23D\KPG23D.HLP. The drive

and path statement that this program

uses to access these messages is

initially set to A:\KPG23D.

Enter the required path name directly.

Test Frequency Help

The radio can be programmed with test

frequencies for alignment purposes.

Additionally required frequencies

can be directly entered below.

General Help

Along the top border of the main program screen (Channel Info.) are the

"drop down" menus. Press ALT; and use the cursor keys --> <-- to select the

desired program section. Help is available for each menu by pressing the F1

function key. Pressing the F1 Function key while in the Channel Info Screen

will load it's Help screen. To get familiar with the function keys used by

the KPG-23D software press the ALT key, use the cursor keys to select

Keyboard Help; and press ENTER.    Within a Help screen use cursor keys

up/down to scroll; F5 to print a hardcopy; and ESC or ALT to escape the

screen. Refer to the KPG-23D's Instruction Manual for additional help.

End Of Help File