KPG-34D Instruction  Manual

PROGRAMMING SOFTWARE

KPG-34D

__________________________________________________________

INSTRUCTION MANUAL Ver. 2.00

KENWOOD CORPORATION

- 1 -

KPG-34D Instruction  Manual

SOFTWARE LICENSE NOTICE

1.  The copyright for this software is owned by KENWOOD CORPORATION.

2.  The software and manual, or any part thereof, must not be used nor copied

except as authorized in writing by KENWOOD CORPORATION.

3.  The contents of the manual are subject to change without notice.

4.  KENWOOD CORPORATION accepts no responsibility for the effects caused by

the use of the software and manual.

5.  The specifications of the software are subject to change without notice.

6.  The product has been carefully designed; however, after reviewing the

supplied documentation, if you have questions, encounter mistakes, or

discover that necessary information is absent from the manual, please

contact the KENWOOD CORPORATION.

IBM is a trademark registered by International Business Machines Corporation.

HANDLING OF FLOPPY DISKS

Observe the following points when handling the programming software disks:

* Never touch the magnetic surface of the floppy disks.

* Keep magnets and magnetic objects away from the floppy disks.

* Do not bend the floppy disks.

* Do not place the floppy disks in areas that are unusually hot, humid, cold

or dry.

* Always insert the 3.5" floppy disk back into its case after use.

- 2 -

KPG-34D Instruction  Manual

CONTENTS

==================================================================

1   INTRODUCTION

FUNCTIONAL SUMMARY

SUPPLIED ITEMS

SUPPORTED TRANSCEIVERS

HARDWARE/SOFTWARE REQUIREMENTS

COMPUTER

OPERATING SYSTEM

COMMUNICATION PORT

COMMUNICATION INTERFACE

2   CONNECTING THE EQUIPMENT

3   INSTALLING THE PROGRAM

INSTALLING ON A HARD DISK

INSTALLING ON A FLOPPY DISK

4   PROGRAM OVERVIEW

SYSTEM FLOW CHART

PROGRAMMING METHODOLOGY

CREATING AND WRITING NEW DATA TO A TRANSCEIVER

MODIFYING DATA PREVIOUSLY WRITTEN TO A TRANSCEIVER

MODIFYING DISK DATA THEN WRITING TO A TRANSCEIVER

5   BASIC OPERATIONS

STARTING THE PROGRAM

INITIAL SCREEN

MENU BAR

CHANNEL INFORMATION WINDOW

MENU OVERVIEW

FILE

EDIT

PROGRAM

SETUP

< F1=HELP >

SELECTING AND ENTERING DATA

KEY SUMMARY

PROMPTED INPUT

NON-PROMPTED INPUT

EXITING THE PROGRAM

6   CHANNEL INFORMATION WINDOW

DATA FIELDS

CHANNEL NUMBER

FREQUENCY (RECEIVE/TRANSMIT)

QT/DQT SELECT NO.

QT/DQT SIGNALING DATA (DECODE/ENCODE)

FUNCTION KEYS

[F3] : EXIT

[F4] : SAVE

[F5] : PRINT

[Shift]+[F3] : CHANNEL COPY

[HOME] : HOME

[END] : END

- 3 -

KPG-34D Instruction  Manual

7   USING THE MENU

FILE

New

Load

Save(=F4)

save As

Delete

Print(=F5)

Go to dos

eXit (=F3)

EDIT

Key assignment

Optional features

PROGRAM

Read from radio

Write to radio

Test Mode

Tuning Mode

SETUP

Communication port

Display color

Help path

F1=HELP

Help Hot Key [F1]

General help

Keyboard help

Keys Active During Help

- 4 -

KPG-34D Instruction  Manual

1   INTRODUCTION

==================================================================

<< FUNCTIONAL SUMMARY >>

KPG-34D software is a tool for programming the TK-261/361 transceiver.  The

software allows a user to enter programming data into an appropriate

computer, and then transfer this data to a transceiver.  The entered data can

also be printed out to provide a permanent record.  Additionally, data that

is already programmed in a TK-261/361 transceiver can be retrieved and edited

using a computer that is running the KPG-34D software.

<< SUPPLIED ITEMS >>

Your programming software package includes the following:

*  floppy disk: 3.5", 1.44 MB

"KPG-34D   PROGRAMMING DISK"   containing:

* KPG-34D program files

* Instruction manual file

<< SUPPORTED TRANSCEIVERS >>

This programming software supports TK-261/361 transceivers.

<< HARDWARE/SOFTWARE REQUIREMENTS >>

< COMPUTER >

The software was designed for use on IBM PC, XT, AT, PS/2 and compatible

computers with RAM (memory) capacity of at least 400 kB.

< OPERATING SYSTEM >

The operating system must be IBM DOS Version 3.1 (or later) or equivalent.

< COMMUNICATION PORT >

Data is transferred between the computer and transceiver via a serial

communication port on the computer.  The port must be compatible with the IBM

asynchronous communications adapter.  Either the COM1 or COM2 port on the

computer may be used.

< COMMUNICATION INTERFACE >

A KPG-8 or KPG-22 cable connects the computer to the transceiver.

- 5 -

KPG-34D Instruction  Manual

2   CONNECTING THE EQUIPMENT

==================================================================

Connect the KPG-8 or KPG-22 cable between the transceiver and the computer

before attempting to use the KPG-34D programming software.  The KPG-8 cable

requires a 13.8 V power supply.

1)   Switch OFF the power to the computer.

Refer to the computer operating manual for set-up details.

2)   Connect the 25-pin connector on the KPG-8 or KPG-22 cable to the COM1 or

COM2 connector on the computer.  If the computer has a 9-pin connector

for COM1 or COM2, use a commercially available 9-pin to 25-pin

adapter.

3)   Switch ON the power to the computer.

4)   Install and start the KPG-34D program.  Refer to sections

"3 INSTALLING THE PROGRAM" and "5 BASIC OPERATIONS" for details.

5)   Switch OFF the power to the transceiver.

6)   Connect the other end of the KPG-8 or KPG-22 cable to the

speaker/microphone jacks on the transceiver.

Note:  When data is transferred from the computer to the transceiver (Program

Read, Program Write), the transceiver automatically will enter PC mode.

CAUTION:  Always connect a 50 ohm dummy load prior to transmitting in Test

mode.

- 6 -

KPG-34D Instruction  Manual

3   INSTALLING THE PROGRAM

==================================================================

After completing all physical connections, you are ready to install the

programming software.  The first procedure describes installing on a hard

disk while the second procedure describes installing on a floppy disk.

<< INSTALLING ON A HARD DISK >>

This procedure assumes the hard disk is installed as drive C.

1)   Insert the "KPG-34D PROGRAMMING DISK" (source disk) into drive A.

2)   Type A: then press [Enter].

"A:\>" appears on the computer display.

3)   Type INSTALL C: then press [Enter].

The installation program copies the necessary files from the source disk

in drive A to the hard disk.

If the KPG-34D program has already been installed on the hard disk, an

overwrite warning message will open.  Press "Y" to continue or "N" to

cancel the installation.

<< INSTALLING ON A FLOPPY DISK >>

This procedure assumes the source drive as A, and the destination drive as B.

1)   Insert the "KPG-34D PROGRAMMING DISK" (source disk) into drive A.

2)   Insert the floppy disk that will receive the software (destination disk)

into drive B.

3)   Type A: then press [Enter].

"A:\>" appears on the computer display.

4)   Type INSTALL B: then press [Enter].

The installation program copies the necessary files from the source disk

in drive A to the destination disk in drive B.

- 7 -

KPG-34D Instruction  Manual

4   PROGRAM OVERVIEW

==================================================================

<< SYSTEM FLOW CHART >>

This chart provides an overview of all functions offered in the menus of the

programming software.

+-Channel Information--+-Frequency

|                      +-QT/DQT Select No.

|                      +-Signalling QT/DQT

|

+-Menu-+-File--+-New

|       +-Load

|       +-Save(=F4)

|       +-save As

|       +-Delete

|       +-Print(=F5)

|       +-Go to dos

|       +-eXit (=F3)

|

|

+-Edit--+-Key assignment-----+-Moni key

|       |

|       +-Optional feature---+-Time out timer (TOT)

|                            +-tot Pre-alert

|                            +-tOt rekey time

|                            +-tot rEset time

|                            +-Squelch level

|                            +-seLf-program

|

+-Program--+-Read from radio

|          +-Write to radio

|          +-Test mode-------+-F10:Tuning Mode--+-RF High Power

|                            |               +-RF Low Power

|                            |               +-Battery level

|                            |               +-Squelch level (Tight)

|                            |               +-Squelch level (Open)

|                            |               +-QT Deviation

|                            |               +-DQT Deviation

|                            |               +-Print Tuning Data(=F5)

|                            +-F5:Print

|

+-Setup----+-Communication port

|          +-Display color

|          +-Help path

|

+-F1=Help--+-General help

+-Keyboard help

- 8 -

KPG-34D Instruction  Manual

<< PROGRAMMING METHODOLOGY >>

This section describes the logical process for three typical programming

operations.

< MODIFYING DATA PREVIOUSLY WRITTEN TO A TRANSCEIVER >

1)   Read the data from the transceiver. (Program/Read from radio)

2)   Modify the data. (Channel Edit)

3)   Write the data to the transceiver. (Program/Write to radio)

4)   Save the data to a disk. (File/save As)

< MODIFYING DISK DATA THEN WRITING TO A TRANSCEIVER >

1)   Read the data from a disk. (File/Load)

2)   Modify the data. (Channel Edit)

3)   Write the data to the transceiver. (Program/Write to radio)

4)   Save the data to a disk. (File/save As)

- 9 -

KPG-34D Instruction  Manual

5   BASIC OPERATIONS

==================================================================

<< STARTING THE PROGRAM >>

With the software correctly installed, you can now proceed to running the

program and learning the necessary techniques for programming the TK-261/361

transceiver.

1)   Make the directory containing the installed programming software the

current directory.

If the software was installed on a floppy disk, first insert the floppy

disk into a drive, then make that drive the current drive.

2)   Type KPG34D then press [Enter].

The program starts and the Introductory Screen appears.  If an error

occurs try reinstalling the program (see "3   INSTALLING THE PROGRAM").

<< INITIAL SCREEN >>

The primary display is referred to as the Initial Screen.  The Initial Screen

is the first screen to appear after the Introductory Screen.  After starting

the program, press any key to clear the Introductory Screen and open the

Initial Screen.

The Initial Screen consists of three parts:

* The top part contains the Menu Bar.

* The central part contains the Channel Information window.

* The bottom part contains a Key Summary.

Brief introductions to each of the three parts of the Initial Screen follow;

however, more detailed information is provided later in this manual.

< MENU BAR >

The Menu Bar contains the titles of the various menus available.  The menus

are used to open windows to edit and/or process data, and to execute

commands.

< CHANNEL INFORMATION WINDOW >

The "Channel Information" window is used to specify frequencies and

Signalling information in specific channels of the TK-261/361 transceiver.

<< MENU OVERVIEW >>

The Menu Bar is at the top of the Initial Screen, above the "Channel

Information" window.  There are six menu titles on the Menu Bar.  To activate

the Menu Bar press [ALT].  Use the right- or left-arrow key to select a

different menu title.  Press [ALT] or [ESC] to deactivate the Menu Bar.

< File >

The File menu allows you to create, open, save, delete, and print data files.

This menu also provides two methods to exit the program.

- 10 -

KPG-34D Instruction  Manual

< Edit >

The Edit menu allows you to assign functions to specific keys on the

transceiver, and configure transceiver functions that are not programmed from

the "Channel Information" window.

< Program >

The Program menu allows you to read data from and write data to memory

(EEPROM) in the transceiver.  This menu also accesses the Test mode.

< Setup >

The Setup Menu allows you to select the communication port on the computer

that is used for communicating via the KPG-8 or KPG-22 interface cable with

the transceiver.  Other configurable items include the color scheme for the

screen as well as the drive and path name for access to the Help file.

< F1=Help >

The Help menu offers general and keyboard help.  The [F1] Help Hot Key

provides help messages pertaining to the highlighted data field of the open

window.

<< SELECTING AND ENTERING DATA >>

There are several methods of selecting or inputting data.  These methods are

explained in this section using typical examples.  Review the examples

carefully before attempting to program a transceiver.

< KEY SUMMARY >

Keys frequently used to access the screens and functions in this program are

listed below:

Arrow

Moves the cursor to different items for editing.  Also scrolls forward and

backward through the help screens opened by using [F1].

[Enter] or [Return]

With the Menu Bar active, press to activate the highlighted menu.  Also used

to input data selections.

[Alt]

Opens and closes menu windows or selects the previous screen.

[F1]  (Help)

Displays help messages.  Pressing [F1] displays the help message

corresponding to the field where the cursor is located.

- 11 -

KPG-34D Instruction  Manual

Space bar or [Shift]+space bar

In the "QT/DQT Select No" field, pressing the space bar cycles through

selections in one direction; pressing [Shift]+space bar cycles in the

opposite direction.  In the "Frequency" or "QT/DQT No." field, after pressing

[F2], press the space bar to cycle through selections in one direction; press

[Shift]+space bar to cycle in the opposite direction.  When a display similar

to the "Read from radio" window appears (Program menu\ Read from radio), you

may press the space bar to select either "[Ok]" or "[Cancel]", then press

[Enter].

[F2] [Shift]+[F2]

Cycles through available choices at the current cursor location.  Pressing

[F2] cycles through selections in one direction; pressing [Shift]+[F2] cycles

in the opposite direction.  When a numerical value must be set, these keys

step through all values.  When a display similar to the "Read from radio"

window appears (Program menu\ Read from radio), you may press [F2] to select

either "[Ok]" or "[Cancel]", then press [Enter].

[Page Up] or [Page Dn]

Moves the help message forward or backward one page at a time.

[Esc]

Moves the cursor to the "Channel Information" window.

< PROMPTED INPUT >

While using File, Model, Program, Setup, or some Edit menu functions, various

information windows open to prompt you for input.  Use one of the methods

described in the examples below for entering data in these cases.

USING ARROW KEYS

When a display similar to the "Read from radio" window appears

(Program menu\ Read from radio), use the right- or left-arrow key to select

either "[Ok]" or "[Cancel]", then press [Enter].  When a display similar to

the "Optional Feature" window appears (Edit menu\ Optional feature), use the

up- or down-arrow key to move the cursor to the item you wish to select,

then press [Enter].

USING QUICK KEYS

When a display similar to the "Read from radio" window appears, press a

Quick Key (capitalized letter) corresponding to the item that you wish to

select.  For example,

* Press "O" to select "[Ok]", or

* Press "C" to select "[Cancel]".

If you are using a color display, Quick Key characters appear in a different

color from other characters.

- 12 -

KPG-34D Instruction  Manual

USING THE KEYBOARD

When a display similar to the "Drive & Path" window appears

(File menu\ save As), enter the drive and path using the keyboard.  The

drive and path shown are the default names provided by the program.

1)   Press the left-arrow key to move the cursor to the left side of the

entry field.

2)   Press [Del] to delete the present character.

3)   Type the new drive name and path name in the correct sequence starting

from the left using the alphanumeric keys on the keyboard.

4)   Press [Enter].

< NON-PROMPTED INPUT >

While using some Edit menu functions, you may be required to input data

without the program issuing any window prompts.  Move the cursor to the item

that you want to edit, then use either [F2] or space bar (see "Key Summary").

In some cases, more than one method works.  The Key Summary along the bottom

of the screen lists the available functions.

<< EXITING THE PROGRAM >>

When you finished programming, close the program correctly rather than simply

turning off the computer.

1)   Press [Alt] to activate the Menu Bar.

2)   Press the right- or left-arrow key to open the File window.

3)   Move the cursor to "eXit" by using the up- or down-arrow key, then press

[Enter] or simply press the Quick Key; the letter which is capitalized,

in this case the "X".

Pressing [F3] also opens this window.

4)   Select [Ok] with the space bar, the right- or left-arrow key, or [F2]

then press [Enter].  Alternatively, use a Quick Key if you prefer.

The program closes and the DOS prompt appears.

5)   If running the program from a floppy disk, remove the disk from its

drive.

6)   Switch OFF the power to the computer.

7)   Switch OFF the power to the transceiver.

- 13 -

KPG-34D Instruction  Manual

6   CHANNEL INFORMATION WINDOW

==================================================================

The "Channel Information" window allows entry of all frequency and signalling

data.  This window is normally displayed when the program is running.  Using

the arrow keys, move the cursor to the items that you want to input or

change.

<< DATA FIELDS >>

The purpose of each of the data fields in the "Channel Information" window is

explained below.

< CHANNEL NUMBER >

Ch no. :

This is a non-editable field that shows the channel number for the frequency

and Signalling data on the same horizontal line.

< FREQUENCY (RECEIVE/TRANSMIT) >

Frequency Receive/Transmit :

Select the receive/transmit frequency for each channel.  Press [F2], then use

the up- and down-arrow keys to select from the choices displayed.  Press

[Enter] after selecting a frequency.

< QT/DQT SELECT NO. >

QT/DQT Select No.

Use the keyboard to enter the QT/DQT memory position number (1 to 9) you

would like the transceiver to use upon power on.

< QT/DQT SIGNALING DATA (DECODE/ENCODE) >

QT/DQT Decode/Encode :

Each channel has nine different QT/DQT (Quiet Talk/ Digital Quiet Talk)

memory positions that can be programmed with QT/DQT data.

Enter QT or DQT data by using the alphanumeric keys, or press [F2] to display

the QT/DQT table.  Select data from the table by using [Page Up], [Page Dn],

and the arrow keys.  When entering DQT data, enter "D" (for DQT) for the

first digit and "N" (for Normal) or "I" (for Inverse) for the last digit.

For example, type 71.9 [Enter] to store QT tone 71.9 Hz, type D023N [Enter]

to store DQT code 023 Normal, or type D754I [Enter] to store DQT code 754

Inverse.  The program automatically inserts "N" for Normal if neither Normal

nor Inverse is specified.  Possible selections are as follows:

QT Map

67.0       82.5       100.0       123.0       151.4       186.2       233.6

69.3       85.4       103.5       127.3       156.7       192.8       241.8

71.9       88.5       107.2       131.8       162.2       203.5       250.3

74.4       91.5       110.9       136.5       167.9       210.7

77.0       94.8       114.8       141.3       173.8       218.1

79.7       97.4       118.8       146.2       179.9       225.7

- 14 -

KPG-34D Instruction  Manual

DQT Normal Map

D023N      D073N      D156N       D261N       D365N       D503N       D654N

D025N      D074N      D162N       D263N       D371N       D506N       D662N

D026N      D114N      D165N       D265N       D411N       D516N       D664N

D031N      D115N      D172N       D271N       D412N       D532N       D703N

D032N      D116N      D174N       D306N       D413N       D546N       D712N

D043N      D125N      D205N       D311N       D423N       D565N       D723N

D047N      D131N      D223N       D315N       D431N       D606N       D731N

D051N      D132N      D226N       D331N       D432N       D612N       D732N

D054N      D134N      D243N       D343N       D445N       D624N       D734N

D065N      D143N      D244N       D346N       D464N       D627N       D743N

D071N      D152N      D245N       D351N       D465N       D631N       D754N

D072N      D155N      D251N       D364N       D466N       D632N

DQT Inverse Map

D023I      D073I      D156I       D261I       D365I       D503I       D654I

D025I      D074I      D162I       D263I       D371I       D506I       D662I

D026I      D114I      D165I       D265I       D411I       D516I       D664I

D031I      D115I      D172I       D271I       D412I       D532I       D703I

D032I      D116I      D174I       D306I       D413I       D546I       D712I

D043I      D125I      D205I       D311I       D423I       D565I       D723I

D047I      D131I      D223I       D315I       D431I       D606I       D731I

D051I      D132I      D226I       D331I       D432I       D612I       D732I

D054I      D134I      D243I       D343I       D445I       D624I       D734I

D065I      D143I      D244I       D346I       D464I       D627I       D743I

D071I      D152I      D245I       D351I       D465I       D631I       D754I

D072I      D155I      D251I       D364I       D466I       D632I

To program a QT tone or a DQT code:

1   Move the cursor to the desired encode/ decode column (memory position,

No 1 to No 9) under QT/DQT.

2   Press [F2].  The QT/DQT window opens.

Or, using the keyboard, type the desired QT tone or DQT code, then press

[Enter].  If typing in QT or DQT, skip step 3.

3   Move the cursor to the desired QT or DQT, then press [Enter].

To move horizontally, press the right- or left-arrow key.  To step up

through the numeric values, press the down-arrow key, space bar, or [F2];

to step down through the numeric values, press the up-arrow key,

[Shift]+space bar, or [Shift]+[F2].

- 15 -

KPG-34D Instruction  Manual

<< FUNCTION KEYS >>

The purpose of each of the function keys is explained below.

< [F3] >

EXIT :

Press [F3] to exit the program.  Select "Ok" using the arrow keys then press

[Enter], or use the "O" Quick Key.  The DOS prompt is restored.  Select

"Cancel" or press [Alt] or [Esc] to abort the function.

< [F4] >

SAVE :

Press [F4] to store the edited data.  Select "Ok" using the arrow keys then

press [Enter], or use the "O" Quick Key.  Select "Cancel" or press [Alt] or

[Esc] to abort the function.

< [F5] >

PRINT :

Press [F5] to print.  Select "Ok" using the arrow keys then press [Enter], or

use the "O" Quick Key.  Select "Cancel" or press [Alt] or [Esc] to abort the

function.

< [Shift]+[F3] >

CHANNEL COPY :

Place the cursor on any field of a channel, then press [Shift]+[F3] to open

the Channel Copy window.  Type the destination data then press [Enter].

Press [Alt] or [Esc] to close the window.

< [HOME] >

HOME :

Press to move the cursor to the currently highlighted field in Channel 1.

< [END] >

END :

Press to move the cursor to the currently highlighted field in Channel 4.

- 16 -

KPG-34D Instruction  Manual

7   USING THE MENU

==================================================================

This section describes the various functions in the Menu Bar.

<< FILE >>

The File menu allows you to create, open, save, delete, and print data files.

This menu also provides two methods to exit the program.

< New >

The New function clears the data file already created then allows you to

create a new file.  This function resets all data to initial values.  Note

that any changes you made to the file will be lost unless you first use the

save command [F4].

1)   Open the File menu.

2)   Select "New", then press [Enter].

A warning window opens.

3)   Select "[Ok]" or "[Cancel]", then press [Enter].  Alternatively, use a

Quick Key if you prefer.

Selecting "[Ok]" resets all data to initial values. The

"Channel Information" window opens.

Selecting "[Cancel]" aborts the New function.  The File menu opens.

< Load >

The Load function reads transceiver data files previously saved on a hard

disk or floppy disk, then writes them into the buffer memory of your

computer.  You can change, display, write, or print the loaded file.

1)   Open the File menu.

2)   Select "Load" then press [Enter].

The "Drive & Path" window opens and displays the currently loaded drive

and path name.

3)   Using the keyboard, type the drive and path name of the desired file,

then press [Enter].

The "Directory" window opens and displays the name(s) of the file(s) on

that path.

An error message window will open in the following cases:

1  If the entered drive does not contain a disk.

Press any key, insert the correct disk in the specified drive, then

press [Enter].

2  If the entered drive and/or path does not exist.

Press any key, correct the entered drive and/or path name, then

press [Enter].

3  If there are no files at the location indicated by the drive and

path name.

Press any key, correct the specified drive and path information,

then press [Enter].

- 17 -

KPG-34D Instruction  Manual

4)   Select the desired file name, then press [Enter].

The Load file name window opens.

5)   Select "[Ok]" or "[Cancel]", then press [Enter].  Alternatively, use a

Quick Key if you prefer.

Selecting "[Ok]" loads the file selected in Step 4. The

"Channel Information" window opens.

Selecting "[Cancel]" cancels the selection.  The "Directory" window

opens.

< Save(=F4) >

The Save function saves the file that you have created to the drive and path

name previously used.  If no drive and path name was previously used the Save

function saves the file in the data file created when the program was

installed (if installed on drive C,  C:KPG34D\DATA).  This method permits you

to omit specifying the drive name and path name.

1)   Open the File menu or press [F4].

If [F4] is pressed, skip Step 2.

2)   Select "Save(=F4)", then press [Enter].

The "Save file" window opens and shows the name of the currently

loaded file.  If the file already exists the "Warning over write !!"

window opens instead of the "Save file" window.

3)   Select "[Ok]" or "[Cancel]", then press [Enter].  Alternatively, use a

Quick Key if you prefer.

Selecting "[Ok]" saves the file.  The "Channel Information" window

opens.  Selecting "[Cancel]" aborts the Save function.  The File menu

opens.

< save As >

The save As function saves the file that you have created to the drive, path,

and name that you specify.  The program automatically enters the previous

information used.  If this information is correct, press [Enter]; otherwise,

use the keyboard to enter the desired information then press [Enter].

If no drive and path were previously used the Save function saves the file in

the data file created when the program was installed (if installed on drive

C,  C:KPG34D\DATA).  If no name was specified the program automatically uses

the name "KPG".

1)   Open the File menu.

2)   Select "save As" then press [Enter].

The "Drive & Path" window opens.  The drive and path name displayed were

previously entered (or are default conditions).

- 18 -

KPG-34D Instruction  Manual

3)   Type the desired drive and path name from the keyboard, then press

[Enter].

The "Save file name" window opens.

An error message window will open in the following cases:

1  If the entered drive does not contain a disk.

Press any key, insert the correct disk in the specified drive, then

press [Enter].

2  If the entered drive and/or path does not exist.

Press any key, correct the entered drive and/or path name, then

press [Enter].

4)   Type the desired file name using the keyboard, then press [Enter].

If you prefer, you can select the file name from the "Directory" window.

Press [F9] to open the "Directory" window.  Make a selection, then press

[Enter] twice.

An error message window will open, after pressing [F9], if there are no

files at the location indicated by the drive and path name.  Press any

key to clear the message and change to the previous screen.  The "Save

file name" window reopens.  If the file already exists the "Warning over

write !!" window opens instead of the "Save file name" window.

5)   Select "[Ok]" or "[Cancel]", then press [Enter].  Alternatively, use a

Quick Key if you prefer.

Selecting "[Ok]" saves the file specified in Step 4, then re-displays

the "Drive & Path" window.  Press [Esc] to return to the "Channel

Information" window.

Selecting "[Cancel]" re-displays the "Save file name" window.

< Delete >

The Delete function deletes an existing file.  It allows you to delete

unneeded files from a specified disk thereby freeing up disk space.

1)   Open the File menu.

2)   Select "Delete" then press [Enter].

The "Drive & Path" window opens.  The drive and path name displayed were

previously entered or are the program defaults.

3)   Type the desired drive and path name using the keyboard, then press

[Enter].

The "Directory" window opens.

An error message window will open in the following cases:

1  If the entered drive does not contain a disk.

Press any key, insert the correct disk in the specified drive, then

press [Enter].

2  If the entered drive and/or path does not exist.

Press any key, correct the entered drive and/or path name, then

press [Enter].

3  If there are no files at the location indicated by the drive and

path name.

Press any key, correct the specified drive and path information,

then press [Enter].

4)   Select a file name with the Up- or Down-arrow key, then press [Enter].

The "Delete file name" window opens.

- 19 -

KPG-34D Instruction  Manual

5)   Select "[Ok]" or "[Cancel]", then press [Enter].  Alternatively, use a

Quick Key if you prefer.

Selecting "[Ok]" deletes the file. The "Directory" window opens.

Note that if there are no more files in the directory you opened, then

after selecting "[Ok]" the "Channel Information" window opens.

Selecting "[Cancel]" opens the "Directory" window.

6)   Repeat Steps 4 and 5 if you want to delete additional files or press

[Esc] to open the "Channel Information" window.

< Print(=F5) >

The Print function directs data to the printer.  The data may have been

created, loaded from a file, or read from the transceiver.  The information

that can be printed includes the following:

* File name

* Model name

* Key assignment

* Optional features

1)   Open the File menu or press [F5].

If [F5] is pressed, skip Step 2.

2)   Select "Print(=F5)", then press [Enter].

The "Edit data print out" window opens.

3)   Select "[Ok]" or "[Cancel]", then press [Enter].  Alternatively, press a

Quick Key if you prefer.

Selecting "[Ok]" prints the file.  The "Channel Information" window

opens.  Selecting "[Cancel]" aborts the Print function.  The File menu

opens.

If the printer is not ready an Error message will open.

Check the printer, then select "[Ok]" or "[Cancel]".

< Go to dos >

The Go to dos function is a temporary exit used when you want to enter DOS

commands at the DOS prompt while the program is still running in the

background.

1)   Open the File menu.

2)   Select "Go to dos", then press [Enter].

The "Type Exit to return to FPU" message and the DOS prompt appear

(DOS commands can be entered).

3)   To resume using the KPG-34D program, type "exit" then press [Enter].

Note:  The KPG-34D program can support this command only if your computer has

sufficient memory.

- 20 -

KPG-34D Instruction  Manual

< eXit (=F3) >

The eXit function stops the program completely and the DOS prompt appears.

1)   Open the File menu or press [F3].

If [F3] is pressed, skip Step 2.

2)   Select "eXit", then press [Enter].

The "Return to DOS" window opens.

3)   Select "[Ok]" or "[Cancel]", then press [Enter].  Alternatively, use a

Quick Key if you prefer.

Selecting "[Ok]" closes the program and the DOS prompt appears.

Selecting "[Cancel]" aborts the eXit function.  The File menu opens.

<< EDIT >>

The Edit Menu is used to enter or edit data needed by the transceiver for

operating and test purposes.

< Key assignment >

Key assignment programs the MONI key on the transceiver side panel.  The

following options are available: Monitor Momentary, Monitor Lock, SQ Off

Momentary, or Off.

Monitor Momentary : Carrier Squelch is restored while the key is pressed.

Monitor Lock :      Each press of the key toggles the Carrier Squelch ON

or OFF.

SQ Off Momentary :  Squelch is opened, and received signals are monitored

while the key is pressed.

Off : The MONI key has no function.

1)   Open the Edit menu.

2)   Select "Key assignment", then press [Enter].

The "Key Assignment" window opens.

4)   Change the settings by pressing the space bar, [Shift]+space bar, [F2],

or [Shift]+[F2], then press [Enter].

5)   Press [Alt] to close the "Key assignment" window.  The Edit menu opens.

6)   Press [Alt] again or [Esc] to open the "Channel Information" window.

- 21 -

KPG-34D Instruction  Manual

Optional feature

Allows configuration of optional features on the transceiver.  Change the

settings by pressing the space bar or [F2].  For functions, such as the Time

Out Time, that require you to enter a numeric value, there are two methods of

inputting the desired value:

1  Direct Input :  Type the desired value with the numeric keys on the

keyboard, then press [Enter].

2  Selection :  Press the space bar or [F2] to step upwards through the

numeric values; press [Shift]+space bar or [Shift]+[F2] to

step downwards through the numeric values.

The following functions are included in the Optional feature window:

Time out timer (tot)

tot Pre-alert

tOt rekey time

tot rEset time

Squelch level

seLf-program

Time out timer (tot)

The tot stops transmission after the programmed time.

Range:  Off, 30 sec. to 300 sec. in steps of 30 sec.

Default : 60 sec.

tot Pre-alert

Time duration from the time you press the PTT switch until the Pre-alert tone

sounds.  This function does not work when the time set for this function is

longer than the time set for the Time out timer.

Range : Off, 5 sec. to 250 sec. in steps of 10 sec.

Default : 55 sec.

tOt rekey time

Time duration from the time tOt stops transmission until you can transmit

again.  The timer starts counting right after the Time out timer expires

regardless of whether the PTT is released or not.  Attempting to transmit

during the tOt rekey time will sound an alarm.  Release PTT and wait until

the tOt rekey time expires before attempting to transmit again.  This

function cannot be set if the Time out timer is set to Off.

Range : Off, 1 sec. to 60 sec. in steps of 1 sec.

Default : Off

tot rEset time

Time duration from the time the PTT switch is released until the Time out

timer is reset to 0.  If you transmit again before the tot rEset time has

expired, the tot rEset time will reset to 0 and the Time out timer will

continue counting from the last transmission time.  This function cannot be

set if the Time out timer is set to Off.

Range : Off, 1 sec. to 15 sec. in steps of 1 sec.

Default : Off

Squelch level

Squelch level setting.

Range : 0 to 9 in steps of 1

Default : 5

seLf-program

The seLf-program gives the user the option of being able to change QT/

DQT and frequency.  If set to 'Disable', turning the power ON while pressing

PTT and LOW will have no effect.  If set to 'Enable', turning the power ON

while pressing PTT and LOW allows you to change QT/DQT and frequency.

Range : Enable, Disable

Default : Enable

- 22 -

KPG-34D Instruction  Manual

<< PROGRAM >>

The Program menu allows you to read data from a transceiver, write data to a

transceiver, and perform various adjustments in Test mode.

< Read from radio >

1)   Open the Program menu.

2)   Select "Read from radio", then press [Enter].

The "Read from radio" window opens.

3)   Select "[Ok]" or "[Cancel]", then press [Enter].  Alternatively, press a

Quick Key if you prefer.

Selecting "[Ok]" reads data from the transceiver.  When completed, a

window opens to report the completed status.

Selecting "[Cancel]" aborts the Read function.  The Program menu opens.

Note:

If communication with the transceiver is not possible for any reason, the

"Check Connection Error" window opens.  Confirm that the transceiver is

connected to the computer correctly via a KPG-8 or KPG-22 cable then select

"[Retry]" to attempt transferring the data again or select "[Cancel]" to

abort the Read function.

< Write to radio >

1)   Open the Program menu.

2)   Select "Write to radio", then press [Enter].

The "Write to radio" window opens.

3)   Select "[Ok]" or "[Cancel]", then press [Enter].  Alternatively, press a

Quick Key if you prefer.

Selecting "[Ok]" writes data into the transceiver.  When completed, a

window opens to report the completed status.

Selecting "[Cancel]" aborts the Write function.  The Program menu opens.

Note:

If communication with the transceiver is not possible for any reason, the

"Check Connection Error" window opens.  Confirm that the transceiver is

connected to the computer correctly via a KPG-8 or KPG-22 cable then select

"[Retry]" to attempt transferring the data again, or select "[Cancel]" to

abort the write function.

< Test Mode >

Note:  Test mode data is only used when the radio is in Test mode.

1.)   Open the Program menu.

2.)   Select "Test mode" then press [Enter].

The "Test Mode Channel & Signalling" window should open.

3.)   Enter a channel frequency number (1 to 4).

The frequency which is set to each channel becomes the frequency which is set to the transceiver.

- 23 -

KPG-34D Instruction  Manual

4.)   Enter a signalling tone number (0 to 39) according to the table below.

No.|   Freq.   || No.|   Freq.   || No.|   Freq.   || No.|  Freq.

---|-----------||----|-----------||----|-----------||----|----------

0  |  None     || 10 |  91.5 Hz  || 20 | 127.3 Hz  || 30 | 179.9 Hz

1  |  67.0 Hz  || 11 |  94.8 Hz  || 21 | 131.8 Hz  || 31 | 186.2 Hz

2  |  69.3 Hz  || 12 |  97.4 Hz  || 22 | 136.5 Hz  || 32 | 192.8 Hz

3  |  71.9 Hz  || 13 | 100.0 Hz  || 23 | 141.3 Hz  || 33 | 203.5 Hz

4  |  74.4 Hz  || 14 | 103.5 Hz  || 24 | 146.2 Hz  || 34 | 210.7 Hz

5  |  77.0 Hz  || 15 | 107.2 Hz  || 25 | 151.4 Hz  || 35 | 218.1 Hz

6  |  79.7 Hz  || 16 | 110.9 Hz  || 26 | 156.7 Hz  || 36 | 225.7 Hz

7  |  82.5 Hz  || 17 | 114.8 Hz  || 27 | 162.2 Hz  || 37 | 233.6 Hz

8  |  85.4 Hz  || 18 | 118.8 Hz  || 28 | 167.9 Hz  || 38 | 241.8 Hz

9  |  88.5 Hz  || 19 | 123.0 Hz  || 29 | 173.8 Hz  || 39 | 250.3 Hz

5.)   Press the space bar or the [F2] key to toggle between transmit and

receive.

6.)   Press [Esc] to open the Program menu.

CAUTION:

Always connect a 50 ohm dummy load prior to transmission.

Note:

If communication with the transceiver is not possible for any reason, the

"Check Connection Error" window opens.  Confirm that the transceiver is

connected to the computer correctly via a KPG-8 or KPG-22 cable then select

"[Retry]" to attempt transferring the data again, or select "[Cancel]" to

abort the write function.

Tuning Mode

The Tuning Mode menu is located in the Test Mode menu.

1.)   Open the Program menu.

2.)   Select "Test Mode", then press [Enter].

3.)   Enter a channel frequency number (1 to 4) and a Signalling Tone

(0 to 39) number (see Test mode above).

4.)   Press [F10].

The "Tuning Mode" window opens

5.)   Select the desired item from the "Tuning Mode" window then press

[Enter].

If "Squelch Level" is selected, the "Squelch" window opens.  Vary the

squelch setting by pressing the right- or left-arrow key, then press

[Enter].

If "RF High Power" is selected, the "RF High Power" window opens.  Vary

the power setting by pressing the right- or left-arrow key.  Press

[Enter] to accept the new data or [Alt] to ignore the data. The

previous window opens.

If "Print Tuning Data(=F5)" is selected, the "print" window opens.

Select "[Ok]" with the space bar, then press [Enter], or press the "O"

Quick Key.  Select "[Cancel]" to close the window and abort the print

function.  An example printout is shown below.

- 24 -

KPG-34D Instruction  Manual

<< KENWOOD TK-261/361 Radio Programming Information >>

Page : 1

Month Date Year     Time

Model Type          : TK-361E

File Name           : KPG34D.U1T

============================================================================

==================================================================

Tuning Mode

==================================================================

RF High Power                                      | 200

RF Low Power                                       |  10

Battery                                            |  94

Squelch (Tight)                                    |  58

Squelch (Open)                                     | 120

QT Deviation                                       | 120

DQT Deviation                                      | 119

------------------------------------------------------------------

============================================================================

After printing is completed, the "Print out complete" window opens.

6.)   Press any key to continue.

<< SETUP >>

Setup allows you to set the communication port, the color scheme for the

screen, and the Help path.

< Communication port >

Select either COM1 or COM2 as the port for communicating with a transceiver.

If the computer has only one communication port, the port is automatically

set to COM1.

1)   Open the Setup menu.

2)   Select "Communication port", then press [Enter].

The "Communication Port" window opens.

Note:  If there is only a COM1 port on your computer, a window opens to

report this fact.  In this case, the communication port cannot be changed.

3)   Select either "[COM1]" or "[COM2]", then press [Enter].  Alternatively,

press a Quick Key if you prefer.

4)   Press [Esc] to open the "Channel Information" window.

- 25 -

KPG-34D Instruction  Manual

< Display color >

Display Color allows you to select the Color mode or Black & White mode.

Color mode permits changing colors of the Menu Bar, windows, messages, etc.

1)   Open the Setup menu.

2)   Select "Display color" then press [Enter].

The Display Color Mode window opens.

3)   Select either "[Color]" or "[Black & White]", then press [Enter].

Alternatively, use a Quick Key if you prefer.

Selecting "[Black & White]" configures the program to function with a

non-color display.  If you select "[Black & White]", skip steps 4 to 6.

Selecting "[Color]" opens the Setup Display Color window.

4)   Use the up- or down-arrow key to move the cursor to the item that you

wish to configure.  Alternatively, use a Quick Key if you prefer.

5)   Select the desired color by pressing the space bar, the [F2] key, or the

right-arrow key.  Pressing the left-arrow key, [Shift]+[F2], or

[Shift]+space bar cycles through available choices in the opposite

direction.

If you want to return all colors to their initial settings, press [F10].

6)   Repeat Steps 4 and 5 to select the remaining colors.

7)   Press [Esc] to open the "Channel Information" window.

< Help path >

Use Help path to select the drive and path name of the Help file.  The Help

file is called KPG34D.HLP and is loaded in the same folder as the program

file upon installation.

To change the setting:

1)   Open the Setup menu.

2)   Select "Help path" then press [Enter].

The "Drive & Path" window opens.

3)   Type the drive and path name from the keyboard then press [Enter].

The window closes and the "Channel Information" window opens.

Note:

* If the entered drive does not contain a disk, the "Error!! Disk not

inserted" message appears.  Press any key, insert the correct disk in the

specified drive, then press [Enter].

* If the entered drive or path does not exist, the "Error Drive or Path not

found" message appears.  Press any key, correct the entered drive or path

name, then press [Enter].

* If there is no Help file at the location indicated by the drive and path

name, the "Error File not found" message appears.  Press any key, correct the

specified drive and path information, then press [Enter].

- 26 -

KPG-34D Instruction  Manual

<< F1=HELP >>

< Help Hot Key [F1] >

The KPG-34D program offers field specific help that consists of brief

explanations pertaining to the data field or menu item where the cursor is

currently located.  For example, if the cursor is in the Frequency data

field, pressing [F1] will automatically retrieve an explanation for

"Frequency Help".  Pressing [F1] again calls up an index of related topics

for which you can also request help.  Each press of [F1] calls up a higher

level index, if available.  Select topics from these indexes by using the up-

and down-arrow keys and [Enter].  When using the KPG-34D program, press the

[F1] Hot Key for assistance anytime you are unsure of what to do next.

Additionally, there is a General Help function that summarizes information

related to screen organization.  Plus, there is a Keyboard Help function that

explains how to use the keyboard.

< General help >

1)   Open the F1=Help menu.

2)   Select "General help", then press [Enter].

The "General Help" window opens.

Pressing [Alt] closes the window and re-displays the F1=Help menu.

< Keyboard help >

1)   Open the F1=Help menu.

2)   Select "Keyboard help", then press [Enter].

The Keyboard Help window opens.

Pressing [Alt] closes the window and opens the F1=Help menu.

< Keys Active while Using Help >

The following keys can be used while accessing Help screens:

[Alt]

Exits Help; re-displays the previous screen.

[Esc]

Returns to the "Channel Information" window.

Up-arrow key

Scrolls the screen up.

Down-arrow key

Scrolls the screen down.

[Page Up]

Displays the previous page.

[Page Dn]

Displays the next page.

[F5]

Prints help information.

- 27 -