Field Programming the Tk-270/370

Press and hold <light> & <dial> while turning on radio

Hold until <SEL> is displayed

Pressing <SCN> allows you to program the following

1.	Monitor Key

Off = off

1= Monitor momentary

2= Monitor Lock

3= Squelch off momentary

2.	Scan Key

Off = off

Co= carrier operate

To = Time operate

3.	Dial Key

Off = off

On = enable

4.	TA key

TA= talkaround

RV= Reverse

Off

5.	Lo Key

On

Off

6.	Priority

None

1=fixed

2=selected

7.	Fixed Priority Channel

Turn selector to channel

8.	Look back time A

Turn selector to time in seconds

9.	Look back time B

Turn selector to time in seconds

10.	Revert Channel

1= Selected

2= Last Call

3= Last used

4=Selected + Talkback

5=Priority

6=Priority + Talkback

11.	Tx Dwell time

Turn selector to time in seconds

12.	Drop out delay

Turn selector to time in seconds

13.	Time out timer

Turn selector to time in seconds

14.	Tot Pre-alert

Turn selector to time in seconds

15.	Tot Rekey time

Turn selector to time in seconds

16.	Tot reset time

Turn selector to time in seconds

17.	Squelch Level

Turn selector to Squelch

18.	Beep tone

On

Off

19.	Signalling

And

Or

20.	Battery Save

On

Off

21.	Selectable QT

On

Off

22.	Clr to transpond

On

Off

23.	Del/ Add

On

Off

Pressing Dial allows programming of:

DTMF

24.	digit time

Turn selector to time in milliseconds

25.	inter digit time

Turn selector to time in milliseconds

26.	First digit time

Turn selector to time in milliseconds

27.	Rise time

Turn selector to time in milliseconds

28.	Rise time with QT

Turn selector to time in milliseconds

29.	PTT ID

Off

1=connect

2=disconnect

3=Both

30.	Dial ID

On

Off

31.	Connect ID

32.	Disconnect ID

33.	 # of DTMF keys on Keypad

12

16

34.	DTMF hold time

On

Off

35.	Store & send

On

Off

36.	D key assignment

Turn selector to D or time in seconds for pause

37.	Dtmf Signaling

Off

1= code squelch

2= Selective Call

38.	Intermediate Code

OFF

A

B

C

D

E

F

39.	Group Code

OFF

A

B

C

D

E

F

40.	Auto Reset time

Turn selector for time in Seconds

41.	Call Alert/ Transpond

Off

1=

2=

3

4

5

42.	Transprond

On

Off

Pressing <TA> will allow you to program Frequencies

Use selector to choose channel

Press PTT

Enter Rx freq. using Selector Knob to change kHz;   Selector Knob + <light> to change MHz.   If no channel is Shown press the <LO> button.

Press PTT

Enter Rx CTCSS code

Press PTT

Enter Tx Freq.

Press PTT

Enter Tx CTCSS code

Press PTT

1.	Option Signal

Off

1= DTMF

2= 2 Tone

2.	PTT ID

On

Off

3.	Add/ Del

4.	Busy Tx

5.	Clock frequency shift

6.	Power

Hi

Low

Repeat for the other 31 channels