- [Description]

The updated version of the HDC4300 is released. (V1.80)

- Upgrade it as necessary.

Upgrade the Firmware and PLD of each board in accordance with the procedure below.

- Upgrade the peripheral units that are connected at the same time to the following recommended software version.

- ■Applicable Model

- ・HDC4300/L CED　：40001～40608

- ・HDC4300/L CNB　：50001～50790

- ・HDC4300/L J4　　：35048～35472

- ・HDC4300/L SYL　：10001～11449

- ・HDC4300/T CNB　：50055～50772

- ・HDC4300/T J4　　：35001～35471

- ・HDC4300/T JN4　 ：30001～30005

- ■Recommended Software version

- ・BPU4000				：V4.40 or higher

- ・BPU4500、BPU4500A		：V3.20 or higher

- ・HDCU4300				：V2.30 or higher

- ・MSU-1000/1500, RCP-1500/1501/1530	：V3.60 or higher

- ・RCP-3500, RCP-3501			 : V1.11 or higher

- （Related technical memo）

- ・HDCM20-057（BPU4000 Software Release [V4.40]）

- ・HDCM20-058（BPU4500/4500A Software Release [V3.20]）

- ・HDCM20-056（HDCU4300 Software Release [V2.30]）

- ・HDCM20-032 (RCP-1500/1501/1530 Software Release [V3.60])

- ・HDCM20-033 (MSU-1000/1500 Software Release [V3.60])

- ・HDCM20-034R（RCP-3500/3501 Software Release [V1.11]）

- [Change Point]

- <Added Function>

- ・ARIA GAIN Limit operation from MSU / RCP

- ARIA GAIN Limit can be operated from MSU / RCP.

- ＜Improvement>

- ・Warning display and operability improvement on MSU/RCP when Remote Back Focus Active.

- When Zoom / Focus Control is Active, a caution message is displayed on MSU / RCP and VF.

・Changed to reflect not only the light reception level but also the occurrence status of transmission failure on the optical condition bar display. The following warning is displayed on MSU / RCP.

CHU : Reflection Care

- When this warning occurs, OPTICAL FIBER CABLE cleanings recommended.

- [Preparation]

- ＜Software File＞

File name  ：hdc4300_v1.80.zip

hdc4300_v1.80_allpackage.zip

※ Obtain it from Software Download site.

The following files are obtained by extracting the above zip File.

* hdc4300_v1.80.zip: The Software/PLD File to be upgraded is placed.

* hdc4300_v1.80_allpackage.zip: All Software/PLD File is placed.

| Package Name |  | Package File Name | File Type |
| --- | --- | --- | --- |
| hdc4300_v1.80_allpackage.zip | hdc4300_v1.80.zip | hdc4300_app.pkg | Software file |
|  |  | hdc4300_os.pkg | OS file |
|  |  | hdc4300_sy.pkg | PLD file |
|  |  | hdc4300_tx.pkg | PLD file |
|  |  | hdc4300_dpr1.pkg | PLD file |
|  |  | hdc4300_dpr2.pkg | PLD file |
|  |  | hdc4300_mem1.pkg | PLD file |
|  |  | hdc4300_mem2.pkg | PLD file |

<Others>

USB memory, 1 piece

- [Upgrading Procedure]

Use USB memory for upgrading.

- Refer to SERVICE MANUAL as for the work procedure.

[Confirmation after Upgrading]

- Confirm the version of each item in the camera menu DIAGNOSIS - <ROM VERSION>(D3) page. CAMERA APP V1.80

- [Software History]

- HDC4300

- V1.01：	Initial Release
CAMERA APP	V1.01	：Initial Release
SY			V3.01	：Initial Release
DPR1		V1.01	：Initial Release
DPR2		V1.01	：Initial Release
MEM1		V1.01	：Initial Release
MEM2		V1.104	：Initial Release
TX			V2.01	：Initial Release

- V1.03：	Error Correction
CAMERA APP	V1.03	：Changed
SY			V3.03	：(HDCM15-042)
DPR1		V1.01	：

- DPR2		V1.01	：
MEM1		V1.01	：
MEM2		V1.104	：
TX			V2.01	：
Technical Memo HDCM15-061

- V1.10：	Adding Function
CAMERA APP	V1.10	：Changed
SY			V3.03	：
DPR1		V1.01	：
DPR2		V1.01	：
MEM1		V1.20	：Changed
MEM2		V1.20	：Changed
TX			V3.00	：Changed
Technical Memo HDCM15-072

(Note)

When connecting with BPU4000 in V1.10 or higher, be sure to upgrade the software of  BPU4000

to V3.10 or higher

- V1.20：	Adding Function
CAMERA APP	V1.20	：Changed
SY			V3.03	：
DPR1		V1.01	：
DPR2		V1.01	：
MEM1		V1.20	：
MEM2		V1.20	：
TX			V3.00	：

- Technical Memo HDCM16-037

- V1.30：	　Adding Function
CAMERA APP	V1.30	：Changed
SY			V3.03	：
DPR1		V1.10	：Changed
DPR2		V1.01	：
MEM1		V1.20	：
MEM2		V1.20	：
TX			V3.10	：Changed

- Technical Memo HDCM16-059

- V1.41：	　Adding Function
CAMERA APP	V1.41	：Changed
OS			V1.01	：
SY			V3.03	：
DPR1		V1.10	：
DPR2		V1.01	：
MEM1		V1.20	：
MEM2		V1.20	：
TX			V3.20	：Changed

- Technical Memo HDCM17-037

- V1.42：	　Adding Function
CAMERA APP	V1.42	：Changed
OS			V1.01	：
SY			V3.04	：Changed
DPR1		V1.10	：
DPR2		V1.01	：
MEM1		V1.20	：
MEM2		V1.20	：
TX			V3.20	：

- Technical Memo HDCM17-059

- V1.50：	Adding / Changing Function
CAMERA APP	V1.50	：Changed
OS			V1.01	：
SY			V3.04	：
DPR1		V1.10	：
DPR2		V1.01	：
MEM1		V1.20	：
MEM2		V1.21	：Changed
TX			V3.20	：

- Technical Memo HDCM17-079

- V1.60：	Adding / Changing Function
CAMERA APP	V1.60	：Changed
OS			V1.01	：
SY			V3.04	：
DPR1		V1.12	：Changed
DPR2		V1.03	：Changed
MEM1		V1.21	：Changed
MEM2		V1.21	：
TX			V3.20	：

- Technical Memo HDCM18-076

- V1.70：	Adding / Changing Function
CAMERA APP	V1.70	：Changed
OS			V1.01	：
SY			V3.04	：
DPR1		V1.12	：
DPR2		V1.03	：
MEM1		V1.21	：
MEM2		V1.21	：
TX			V3.20	：

- Technical Memo HDCM20-020

- V1.80：	Adding / Changing Function
CAMERA APP	V1.80	：Changed
OS			V1.01	：
SY			V3.04	：
DPR1		V1.12	：
DPR2		V1.03	：
MEM1		V1.21	：
MEM2		V1.21	：
TX			V3.20	：

- Technical Memo HDCM20-055